SA WG2 Meeting #137E	S2-201928

# February 24 - 27, 2020 Electronic meeting    							 (revision of S2-200419)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2008 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the BDT policy re-negotiation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon], Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the SA2#136 meeting the Background Data Transfer (BDT) policy re-negotiation description based on the updated Network Performance analytics by NWDAF is agreed. However, it is unreasonable for PCF to remove the old BDT policy stored in the UDR before the PCF confirm from AF about the new BDT policy selected from the candidate list. Otherwise, the PCF can’t update the BDT policy stored in the UDR for the corresponding Background Transfer Reference ID that has been removed in the UDR. Changes in SA2#137E (CR from Huawei, HiSilicon): During the SA2#136AH meeting, it was clarified that the PCF should not remove the old BDT policy before it receives the new selected BDT policy from the AF. Such handling will lead to inconsistencies, i.e. it is unclear how the outdated BDT policy will be removed if the AF doesn’t select any new candidate policy; in addition, the BDT policy becomes invalid due to change on the conditions negotiated, such as change of the load in the area where the BDT policy happens, and is unrelated to the fact that the AF may select a new BDT policy or not. With original proposal, in step 4 of 4.16.7.3, the PCF only removes the BDT policy in the UDR but the BDT context (including the number of UEs, network area) is still kept in the PCF for handling notification and possible subsequent BDT policy re-selection. When the re-selection happens, the steps 8-13 from clause 4.16.7.2 are executed, which includes the procedure that the PCF stores the Background Data Transfer Reference ID together with the new transfer policy in the UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the PCF should not remove the old BDT policy before it receives the new selected BDT policy from the AF. Changes in rev 1 Allow reoval of BDT policy from UDR when conditions change that invalidat it. The figure 4.16.7.3-1 is updated to show UDR service operation of deletion to match the procedure description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BDT policy re-negotiation description is technically incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 389 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.16.7.3	Procedure for BDT warning notification

Figure 4.16.7.3-1: The procedure for BDT warning notification

1.	The negotiation for Background Data Transfer (BDT) described in clause 4.16.7.2 is completed.

2.	The PCF is notified when the network performance in the area of interest goes below the criteria set by the operator from the NWDAF as described in TS 23.288 [50].

3.	The H-PCF may request from the UDR the stored BDT policies using Nudr_DM_Query (Policy Data, Background Data Transfer) service operation

4.	The UDR provides all the Background Transfer Policies to the H-PCF. The H-PCF identifies the BDT policies affected by the notification received from NWDAF. For each of them, the H-PCF determines the ASP of which the background traffic will be influenced by the degradation of network performance and which requested the H-PCF to send the notification. The PCF then decides based on operator policies, whether a new list of candidate Background Data Transfer policies has to be calculated.

NOTE 1:	The BDT Policies that are applicable for future sessions are checked by the PCF in step 4.

5-6.	The PCF removes the BDT policy from UDR.

7.	The PCF sends the notification to the NEF by invoking Npcf_PolicyControl_Notify (Background Data Transfer Reference ID, optionally Network Area Information, Time window, list of candidate Background Data Transfer policies) service operation.

8.	The NEF notifies the AF by invoking Nnef_BDTPNegotiation_Notify (Background Data Transfer Reference ID, optionally Network Area Information, Time window, list of candidate Background Data Transfer policies) service operation.

9.	The AF selects one of the new Background Data Transfer policies included in the candidate list in the BDT warning notification.

NOTE 2:	If the AF doesn't select any of the new Background Data Transfer policies next steps are not executed, the previously negotiated Background Data Transfer policy becomes obsolete and not applicable any longer.

10.	The steps 8-13 from clause 4.16.7.2 are executed.

11.	If there is a new Background Data Transfer policy stored in the UDR, the PCF is notified by UDR and then the PCF identifies if the URSP rules to UE needs to be updated using the procedure defined in clause 4.16.12.2.

The AF can send Reset for notification by invoking Nnef_BDTPNegotiation_Update service, when the AF requests not to receive the notification anymore. Then, the NEF invokes Npcf_BDTPolicyControl_Update service in order to provide this information for the H-PCF.

# * * * * End of changes * * * *