3GPP SA WG2 Meeting #137E	S2-2001930

# February 24 - 27, 2020, Electronic meeting    			(rev. of S2-2001197, merged with S2-2002129)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0400 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Replace the reference to 23.203 to the clause in 23.503 in 4.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the agreement addressed in the DP “TS 23.203 Endorsement” (S2-1903636), any reference to 23.203 shall be replaced by the referenced content. However, 4.3.6 still references TS 23.203. Changes in rev.2 (revision of Nokia and Nokia Shangai-Bell CR): - Replace the list of procedures defined in 23.502 by requirements as for other features described in 23.503. - Requirements for Service Capability Exposure are split into both those capabilities related to non session management but related to UE configuration and those capabilities related to session management. - The list of capabilities is completed including Rel-16 features for TSN support, negotiation of BDT policies, provisioning of 5G VN Group Data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 4.3.6 to include content defined in clause 4.7 of TS 23.203. Changes in rev.2: - New subclause to define requirements for service capability exposure of non session management policy control functionality. - Replace the list of procedures by requirements for service capability exposure of session management policy control functionality. - Extend UE Policy requirements to include V2X Policies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description of the Service Capability Exposure of Policy control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new), 4.3.6, 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 4.2.x	Support for non-session management related network capability exposure

The non-session management  policy control related capability expotureinclude:

-	Management of PFDs as defined in clause 4.2.4 and clause 4.18 of TS 23.502 [3]:

-	Negotiations for background data transfer policy; as defined in clause 6.1.2.4 and clause 4.16.7 of TS 23.502 [3]:

-	Applying a previously negotiated background data transfer policy to a UE or group of UEs; as defined in clause 6.1.2.4 and clause 4.15.6.8 of TS 23.502 [3]:

-	Traffic steering control, AF influenced traffic diversion, as defined in clause 4.3.7:

-	Service specific parameter provisioning for V2X communication (see clause 5.20 of TS 23.501 [2] and clause 4.15.6.7 of TS 23.502 [3]);

-	5G VN group management (see clause 5.29 of TS 23.501 [2] and clause 4.15.6 of TS 23.502 [3]);

-

Next Change

### 4.3.6	Support for session management related network capability exposure

The session management policy control related capability expoture include:

- Set or change a chargeable party at AF session setup (see clause clause 4.15.6.4 and 4.15.6.5 of TS 23.502 [3]);

- Set up an AF session with required QoS (see clause 6.1.3.22 and clause 4.15.6.6 of TS 23.502 [3]);

- Transfer of traffic characteristics of Time Sensitive Communication from the TSN AF, see clause 6.1.3.23;

Next Change

### 4.2.2	UE policy control requirements

The 5GC shall be able to provide policy information from the PCF to the UE. Such policy information includes:

-	Access Network Discovery & Selection Policy (ANDSP): It is used by the UE for selecting non-3GPP accesses network.

-	UE Route Selection Policy (URSP): This policy is used by the UE to determine how to route outgoing traffic. Traffic can be routed to an established PDU Session, can be offloaded to non-3GPP access outside a PDU Session, or can trigger the establishment of a new PDU Session.

-	V2X Policy (V2XP): This policy provides configuration parameters to the UE for V2X communication over PC5 reference point or over PC5 reference point or both. V2X Policies are defined in TS 23.287 [28].

End of changes