SA WG2 Meeting #S2-137E  	S2-2001940r01

# 24 - 27 February, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2097 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MA PDU Session Release when UE is registered to different PLMNs over 3GPP access and non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. The MA PDU Session Release procedure for the scenario that the UE is registered to different PLMNs over 3GPP access and non-3GPP access is still missing. 2. Current description of the trigger for MA PDU Session release "when S-NSSAI of the MA PDU Session is not in the Allowed NSSAI" is somewhat confusing and better to be clarified. 3. Some editorial corrections are needed in current specification on MA PDU Session Release procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the MA PDU Session Release procedure when the UE is registered to different PLMNs over 3GPP access and non-3GPP access. 2. Modify the general description on MA PDU Session Release triggers. 3. Other Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of specification on the MA PDU Session Release procedure when the UE is registered to different PLMNs over 3GPP access and non-3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.10.1, 4.22.10.2, 4.22.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

4.22.10	MA PDU Session Release

4.22.10.1	General

The MA PDU Session Release procedure is used to release the MA PDU Session over both accesses or over a single access. The MA PDU Session release over a single access may be triggered by the network due to e.g. when the UE is deregistered over an access or when the S-NSSAI of the MA PDU Session is no longer in the Allowed NSSAI of either 3GPP access or non-3GPP access.

4.22.10.2	UE or network requested MA PDU Session Release (non-roaming and roaming with local breakout)

The signalling flow for a MA PDU Session Release when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is based on the signalling flow in Figure 4.3.4.2-1 with the following differences and clarifications:

-	In step 1, If the AMF need to release the MA PDU Session over a single access, the AMF invokes the Nsmf_PDUSession_UpdateSMContext service operation to request the release of the MA PDU Session over a single access. In this case, the AMF includes in which access the MA PDU Session should be released.

NOTE:	When the SMF received the release request from the AMF, the SMF decides whether the MA PDU Session is released over both accesses or over a single access based on its local policy.

-	In step 1, if the AMF need to release the MA PDU Session (e.g. locally released when the UE is CM-IDLE), the AMF invokes the Nsmf_PDUSession_ReleaseSMContext service operation to request the release of the MA PDU Session.

-	In step 1, when the SMF initiates MA PDU Session Release procedure, the SMF includes access type (e.g. 3GPP, non-3GPP or both access) it want to release in the PDU Session Release Command message.

-	In step 3, if the SMF releases the MA PDU Session over a single access, the SMF shall not include "skip indicator" in the Namf_Communication_N1N2MessageTransfer service.

In step 3, if the SMF releases the MA PDU Session over both accesses and user plane resources are established in both accesses, the SMF includes both N1 SM container (PDU Session Release Command) and N2 SM Resource Release request together in the Nsmf_PDUSession_UpdateSMContext or Namf_Communication_N1N2MessageTransfer service so that the UE does not request to activate user plane resources. The SMF releases user plane resources of the other access by including N2 SM Resource Release only in Namf_Communication_N1N2MessageTransfer service.

-	In step 3, when the SMF provides N1 SM container and/or N2 SM information, the SMF includes access type in the Namf_Communication_N1N2MessageTransfer to provide routing information to the AMF.

-	In step 11, the SMF triggers Nsmf_PDUSession_SMContextStatusNotify service only when the MA PDU Session is released in both accesses.

4.22.10.3	UE or network requested MA PDU Session Release (home-routed roaming)

The signalling flow for a MA PDU Session Release when the UE is registered to the same VPLMN over 3GPP access and non-3GPP access and the PDU Session Anchor (PSA) is located in the HPLMN, is based on the signalling flow in Figure 4.3.4.3-1 with the following differences and clarifications:

-	In step 1, If the V-AMF need to release the MA PDU Session over a single access, the V-AMF may invoke the Nsmf_PDUSession_UpdateSMContext service operation to request the release of the MA PDU Session over a single access. In this case, the AMF shall include in which access the MA PDU Session should be released. The V-SMF invokes Nsmf_PDUSession_Update service operation to request the release of the MA PDU Session over a single access. The V-SMF shall include in which access the MA PDU Session should be released.

NOTE:	When the H-SMF received the release request from the V-SMF, the H-SMF decides whether the MA PDU Session is released over both accesses or over a single access based on its local policy.

-	In step 1, if the V-AMF need to release the MA PDU Session (e.g. locally released when the UE is CM-IDLE), the AMF invokes the Nsmf_PDUSession_ReleaseSMContext service operation to request the release of the MA PDU Session. The V-SMF invokes Nsmf_PDUSession_Release service operation to request the release of the MA PDU Session.

-	In step 1, when the H-SMF initiates MA PDU Session Release procedure, the H-SMF includes access type (e.g. 3GPP, non-3GPP or both access) it want to release in the Nsmf_PDUSession_Update service operation.

-	In step 5, when the V-SMF initiates MA PDU Session Release procedure, the V-SMF includes access type (e.g. 3GPP, non-3GPP or both access) it want to release in the PDU Session Release Request or PDU Session Release Command message.

-	In step 5, if the V-SMF releases the MA PDU Session over a single access network, the V-SMF shall not include "skip indicator" in the Namf_Communication_N1N2MessageTransfer service.

-	In step 5, if the V-SMF releases the MA PDU Session over both accesses and user plane resources are established in both accesses, the V-SMF includes both N1 SM container (PDU Session Release Command) and N2 SM Resource Release request together in the Nsmf_PDUSession_UpdateSMContext or Namf_Communication_N1N2MessageTransfer service so that the UE does not request to activate user plane resources. The V-SMF releases user plane resources of the other access by including N2 SM Resource Release only in Namf_Communication_N1N2MessageTransfer service.

-	In step 5, when the V-SMF provides N1 SM container and/or N2 SM information, the V-SMF includes access type in the Namf_Communication_N1N2MessageTransfer to provide routing information to the V-AMF.

-	In step 16, the H-SMF triggers Nsmf_PDUSession_StatusNotify service only when the MA PDU Session is released in both accesses . The V-SMF triggers Nsmf_PDUSession_SMContextStatusNotify service only when the MA PDU Session is released in both accesses.

When the UE is registered to different PLMNs over 3GPP access and non-3GPP access (one of the PLMN may be HPLMN) and the PDU Session Anchor (PSA) is located in the HPLMN, the MA PDU Session is released over a single access or both accesses using the signalling flow in Figure 4.3.4.3-1 (in case of roaming on one or both access legs) or Figure 4.3.4.2-1 (in case of non-roaming on one access leg) with the following differences and clarifications:

-	In step 1, if the (H-)SMF decides to release the MA PDU Session over both accesses, the (H-)SMF invokes Nsmf_PDUSession_Update service operation (in case of roaming, Figure 4.3.4.3-1) or generates a PDU Session Release Command message (in case of non-roaming, Figure 4.3.4.2-1) to request the release of the MA PDU Session over the other access.

NOTE:	When the (H-)SMF received the release request from the V-SMF or the AMF, the (H-)SMF decides whether the MA PDU Session is released over both accesses or over a single access based on its local policy.

-	In step 5 of Figure 4.3.4.3-1 (in case of roaming) or in step 3 of Figure 4.3.4.2-1 (in case of non-roaming), the (V-)SMF shall not include "skip indicator" in the Namf_Communication_N1N2MessageTransfer service.

# * * * * End of changes * * * *