3GPP TSG-SA2 Meeting #137-E	S2-2001960r02

# Online, , 24th Feb 2020 - 27th Feb 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2101 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling MA PDU Sessions over ATSSS-incapable AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is agreed that AMF indicates ATSSS capability to UE during Registration procedure (TS 23.501, S2-2001145). Meanwhile, the contexts of already established MA PDU Sessions can be transferred to ATSSS-incapable AMF. Since the ATSSS-incapable-AMF cannot differentiate whether the PDU Session is MA PDU Session or not, it is required for UE to release the established MA PDU Sessions whose contexts are handled by the ATSSS-incapable AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the Registration Accept message, AMF provides ATSSS Capability, which indicates whether ATSSS is supported or not. - If the Registration Accept message indicates that ATSSS feature is not supported and also contains the PDU Session Status of MA PDU Sessions, UE performs the MA PDU Session Release procedure for each MA PDU Session in the PDU Session Status over the access to which the Registration Request message is sent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MA PDU Sessions can be activated via ATSSS-incapable AMF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.22.9	Connection, Registration and Mobility Management procedures

#### 4.22.9.1	Registration procedures

The signalling flow for a Registration is based on the signalling flow in Figure 4.2.2.2.2-1 with the following differences and clarifications:

-	In step 1, if the UE wants to re-activate the user plane of the MA PDU Session(s) over the access the Registration message is sent to, the UE indicates PDU Session ID(s) of the MA PDU Session(s) in the List Of PDU Sessions To Be Activated.

If the UE locally releases the MA PDU Session(s) in both accesses, the UE indicates it in the PDU Session Status. If the AMF receives the PDU Session Status and finds mismatch, regardless of roaming mode of the MA PDU Session(s) (i.e. non-roaming, local breakout roaming, home routed roaming in the same PLMN or home routed roaming in different PLMNs), the AMF invokes Nsmf_PDUSession_ReleaseSMContext service towards the SMF(s) in order to release any network resources related to the MA PDU Session(s).

-	In step 22, if the AMF indicates that the PDU Session(s) has been released in the PDU Session Status to the UE in Registration Accept message, the UE removes locally any internal resources related to the MA PDU Session(s) that are not marked as established. If the AMF provides the MA PDU Session Support indicator in the Registration Accept message to inform the UE that ATSSS is not supported and the PDU Session Status in the Registration Accept message contains the PDU Session ID(s) of MA PDU Session(s), the UE performs the MA PDU Session Release procedure for each MA PDU Session in the PDU session Status over the access to which the Registration Request message is sent.

# * * * * End of changes * * * *