3GPP TSG-SA2 Meeting #137-E	S2-2002010

24 – 27 February, 2020, Electronic meeting    	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2164 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to network selection with multiple subscribed SNPNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SNPN roaming is not supported in Rel-16. The UE can connect only to the SNPN for which it has subscription. The UE can have subscription with multiple SNPNs, in which case it needs to be configured with a (SUPI, credential) tuple for each of the SNPN subscriptions. A UE with multiple SNPN subscriptions is similar to a multi-USIM device. The user can activate one or more than one USIM at the time, which is a user’s choice. The user also selects a specific USIM when they wish to obtain a specific service (e.g. use USIM1 for voice, use USIM2 for data). The same applies to the UE with multiple SNPN subscriptions. Specifically, the user chooses which SNPN subscriptions to activate and chooses which SNPN subscription (among the activated ones) to use when it wishes to obtain a specific service. The bottomline is that the multiple SNPN subscriptions need to be independent of each other (in the same way as multiple USIMs in a multi-USIM device are), and the same observation applies to the SNPN selection procedures that need to be done independently for each activated SNPN subscription. However, clause 5.30.2.4 states the following: If multiple SNPNs are available that the UE has respective SUPI and credentials for, then the priority order for selecting and attempting to register with SNPNs is based on UE implementation. Similarly Stage 3 text in TS 23.122 clause 4.9.3.1.1 states the following: ##### 4.9.3.1.1 Automatic SNPN selection mode procedure The MS selects another SNPN, if available, allowable, and identified by an SNPN identity in an entry of the "list of subscriber data" in the ME. If more than one SNPN are available, allowable, and each of them is identified by an SNPN identity in an entry of the "list of subscriber data" in the ME, the MS shall select one of those SNPNs in MS implementation specific order. It is implied that the multiple subscriptions are equivalent in terms of services that the SNPNs can provide, and that it is just a matter of priority ordering (not standardized by 3GPP) when the UE needs to select which SNPN to connect to. Moreover, it is implied that the SNPN subscription to be used is not a user’s choice, but is determined as part of the SNPN network selection process. When compared to the multi-USIM device (where the two USIMs, as well as the services that they can provide to the user, are completely independent), it is clear that this a NOT the right way to deal with multiple SNPN subscriptions. Each SNPN subscription may provide services that are specific to that subscription, which is why the selection of an SNPN subscription: - should be a user’s choice, and - should not be done automatically as part of network selection process. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The sentence implying priority ordering among multiple SNPN subscriptions for the purpose of network selection has been changed to remove the notion of “priority ordering”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect assumption about how multiple SNPN subscriptions interact with the SNPN selection procedure (they should not). Without this change Stage 3 will be implemented incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** Start of changes ***

#### 5.30.2.4	Network selection in SNPN access mode

When a UE is set to operate in SNPN access mode the UE does not perform normal PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17].

UEs operating in SNPN access mode read the available PLMN IDs and list of available NIDs from the broadcast system information and take them into account during network selection.

For automatic network selection, the UE selects and attempts to register with the available SNPN identified by a PLMN ID and NID for which the UE has SUPI and credentials.

NOTE:	The details of SNPN selection will be defined in TS 23.122 [17].

If multiple SNPNs are available that the UE has respective SUPI and credentials for, then how the UE selects SNPN is based on UE implementation.

For manual network selection UEs operating in SNPN access mode provide to the user the list of NIDs and related human-readable names (if available) of the available SNPNs the UE has respective SUPI and credentials for.

When a UE performs Initial Registration to an SNPN, the UE shall indicate the selected NID and the corresponding PLMN ID to NG-RAN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.

# *** End of changes ***