3GPP TSG-SA2 Meeting #137-E	S2-2002013

# Online, , 24th Feb 2020 - 27th Feb 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2165 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for Support for multiple TSN working domains |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IEEE Std 802.1AS-Rev/D7.3 contains support for multiple gPTP domains. This means each TSN working domain can use different time scale, and can have their gPTP time synchronization information delivered through the 5GS independenty. gPTP supports two types of timescales; timescale PTP where the epoch is defined in IEEE Std 802.1AS-Rev/D7.3 and therefore known and can be converted to UTC, and timescale ARB where the epoch can be anything as set by the administrator. gPTP domain 0 uses the PTP timescale, other gPTP domains use the ARB timescale. 802.1Qcc and 802.1Q do not support indication of timescale in bridge configuration, therefore it is assumed that a known timescale i.e. PTP timescale is always used in bridge configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NOTE: In this Release of specification, support for multiple TSN working domains is limited related to IEEE 802.1AS [104] and does not apply to interaction involving TSN AF and CNC. The corresponding IEEE specifications (i.e. 802.1Qcc [95], 802.1Qbv [96], and 802.1Q [98]) do not support it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether and how multiple TSN domains can be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

#### 5.27.1.3	Support for multiple TSN working domains

Each TSN working clock domain sends its own gPTP messages. The related Ethernet frames carry the gPTP multicast Ethernet destination MAC address and the gPTP message carries a specific PTP "domainNumber" that indicates the time domain they are referring to. The NW-TT makes ingress timestamping (TSi) for the gPTP event messages of all domains.

A UE receives gPTP messages and forwards them all to the DS-TT. The DS-TT receives the original TSN clock timing information and the corresponding TSi via gPTP messages for one or more working domains. The DS-TT then makes egress timestamping (TSe) for the gPTP event messages for every external TSN working domain. Ingress and egress time stamping is based on the 5G system clock at NW-TT and DS-TT.

NOTE 1:	An end-station can select TSN timing information of interest based on the "domainNumber" in the gPTP message.

The process described in "Distribution of TSN clock and time-stamping" is thus repeated for each TSN working domain between a UE and the UPF it is connected to. When a UE is connected to multiple TSN working domains via the same UPF, then all gPTP messages from these domains are forwarded to the UE via the same PDU session.

NOTE 2:	If all TSN working domains can be made synchronous and the synchronization can be provided by the 5G clock, the TT output ports at the UPF side (NW-TT) towards the connected TSN networks propagate the 5G clock using the 802.1AS profile (i.e. the 5G system as an IEEE 802.1AS [104] compliant time-aware system).

NOTE 3: 		In this Release of specification, support for multiple TSN working domains is limited related to IEEE 802.1AS [104] and does not apply to interaction involving TSN AF and CNC. The corresponding IEEE specifications (i.e. 802.1Qcc [95], 802.1Qbv [96], and 802.1Q [98])  can be only supported using one specific Time Domain among them and use that Time Domain associated with  802.1Qcc [95], 802.1Qbv [96], and 802.1Q [98].

.

## ---End of the Change---