3GPP TSG SA WG2 Meeting #137E	S2-2002044

# February 24 - 27, 2020, Electronic meeting      (rev. of S2-2001606, merges S2-2002289, S2-2002133)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0394 | rev | 4 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS flow binding for TSN streams with same periodicity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) TS 23.501 clause 5.27.2 states: “The TSN AF may aggregate multiple TSN Streams if they have the same periodicity and traffic class.” How those aggregated TSN streams (that have the same periodicity) are mapped to a QoS flow is unclear. The TSN AF may also provide other TSC parameters such as “periodicity” that has no mapping to one of the QoS characteristics listed in 23.501, as such the periodicity will be not be used to generate a 5QI but included in the PCC Rule to the SMF. The SMF binds PCC Rules to QoS flows using QoS parameters, mainly 5QI and ARP, if provided the MDBV or the periodicity or both. This allows mapping of the TSN streams to the QoS flows that has the right QoS profile and periodicity. 2) The TSN paramerers from AF to PCF are currently described in several sections (e.g. in 5.3.1 and 6.1.3.23) and they are partly conflicting. 3) The current text in 6.1.3.23 implies the service data flow filter in PCC Rule contains the UE MAC address. This would not work as the Ethernet frames do not convey the UE MAC address. 4) As agreed in S2-2001504, the SMF must be able to report multiple ports and MAC addresses in NW-TT to the PCF/AF. 5) Editorial: align with 23.501; change the “TSN AF QoS container” to “TSCAI input container”. Change all “TSN QoS parameters” to “TSN QoS information”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe that the TSN AF provides some TSC parameters that are not used to generate a 5QI, and included in the PCC Rule then used by the SMF for binding PCC Rules into QoS flows. Add a definiton of TSN AF Parameters (from AF to PCF) to 6.1.3.23. This is aligned with TS 23.502 which uses the “TSN AF Parameters” in the service interface definitions. In 6.1.3.23, the AF must indicate the service data flow description (traffic class, optionally destination MAC address of the TSN stream) to the PCF that the PCF uses to generate the PCC Rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation of the TSN feature, not aligned with 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 6.1.3.2.4, 6.1.3.23, 6.2.1.2, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3.2.4	QoS Flow binding

QoS Flow binding is the association of a PCC rule to a QoS Flow within a PDU Session. The binding is performed using the following binding parameters:

-	5QI;

-	ARP;

-	QNC (if available in the PCC rule);

-	Priority Level (if available in the PCC rule);

-	Averaging Window (if available in the PCC rule);

-	Maximum Data Burst Volume (if available in the PCC rule)

When the PCF provisions a PCC Rule, the SMF shall evaluate whether a QoS Flow with QoS parameters identical to the binding parameters exists unless the PCF requests to bind the PCC rule to the QoS Flow associated with the default QoS rule. If no such QoS Flow exists, the SMF derives the QoS parameters, using the parameters in the PCC Rule, for a new QoS Flow, binds the PCC Rule to the QoS Flow and then proceeds as described TS 23.501 [2] clause 5.7. If a QoS Flow with QoS parameters identical to the binding parameters exists, the SMF updates the QoS Flow, so that the new PCC Rule is bound to this QoS Flow.

NOTE 1:	For PCC rules containing a delay critical GBR 5QI value, the SMF can bind PCC Rules with the same binding parameters to different QoS Flows to ensure that the GFBR of the QoS Flow can be achieved with the Maximum Data Burst Volume of the QoS Flow.

The SMF shall identify the QoS Flow associated with the default QoS rule based on the fact that the PCC rule(s) bound to this QoS Flow contain:

-	5QI and ARP values that are identical to the PDU Session related information Authorized default 5QI/ARP; or

-	a Bind to QoS Flow associated with the default QoS rule and apply PCC rule parameters Indication.

NOTE 2:	The Bind to QoS Flow associated with the default QoS rule and apply PCC rule parameters Indication has to be used whenever the PDU Session related information Authorized default 5QI/ARP (as described in clause 6.3.1) cannot be directly used as the QoS parameters of the QoS Flow associated with the default QoS rule, for example when a GBR 5QI is used or the 5QI priority level has to be changed.

When a QoS Flow associated with the default QoS rule exists, the PCF can request that a PCC rule is bound to this QoS Flow by including the Bind to QoS Flow associated with the default QoS rule Indication in a dynamic PCC rule. In this case, the SMF shall bind the dynamic PCC rule to the QoS Flow associated with the default QoS rule (i.e. ignoring the binding parameters) and keep the binding as long as this indication remains set. When the PCF removes the association of a PCC rule to the QoS Flow associated with the default QoS rule, a new binding may need to be created between this PCC rule and the QoS Flow as described above.

The binding created between a PCC Rule and a QoS Flow causes the downlink part of the service data flow to be directed to the associated QoS Flow at the UPF (as described in TS 23.501 [2] clause 5.7.1). In the UE, the QoS rule associated with the QoS Flow (which is generated by the SMF and explicitly signalled to the UE as described in TS 23.501 [2] clause 5.7.1) instructs the UE to direct the uplink part of the service data flow to the QoS Flow in the binding.

Whenever the authorized QoS of a PCC rule changes, the existing bindings shall be re-evaluated. The re-evaluation may, for a service data flow, require a new binding with another QoS Flow.

NOTE 2:	A QoS change of the PDU Session related information Authorized default 5QI/ARP values doesn't cause the QoS Flow rebinding for PCC rules with the Bind to QoS Flow associated with the default QoS rule Indication set.

When the PCF removes a PCC Rule, the SMF shall remove the association of the PCC Rule to the QoS Flow.

The SMF shall report to the PCF that the PCC Rules bound to a QoS Flow are removed when the corresponding QoS Flow is removed.

The QoS Flow binding shall also ensure that:

-	if a PCC rule contains the TSN TSCAI input container, the PCC rule is bound to a new QoS Flow and no other PCC rule is bound to this QoS Flow.

NOTE x:  The TSN AF provides the TSN TSCAI input container to be sent to RAN and also to be used at the SMF to bind TSN streams into QoS flows

# * * * * End of changes * * * *