3GPP TSG-SA WG2 Meeting #137E	S2-2002050

# 24 - 27 February, 2020, Electronic meeting                              (revision of S2-2000555)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2071 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional traffic pattern calculations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It addresses the FASMO item #22 It is possible to easily calculate traffic pattern parameters for DL aggregated flow of TSN Streams (all streams that belong to a traffic class) using 802.1Qbv scheduling information. Indeed, since TSN Streams of the same traffic class are already aggregated using the 802.1Qbv, the information can be useful to extract traffic pattern. This is only used for DL (DS-TT egress port) since there is a single PDU session associated to this port. The calculation method is the same as used for PSFP parameters. However, these traffic pattern parameters based on 802.1Qbv are with reference to the egress port, as opposed to the parameters obtained using PSFP which are referenced to the ingress port. Therefore the burst arrival time is adapted to reflect the time at the corresponding ingress port since bridge delay per port pair is known to the TSN AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional information on calculation of traffic pattern parameters of aggregated TSN Stream using 802.1Qbv (reference to egress port) only for DL.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing traffic pattern parameter calculation for aggregated TSN Streams (in DL). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | I.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Need to implement S2-2001510 23.501 CR2085R2 first. The new text on Annex I.1 goes after the text from CR2085. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***BEGIN CHANGES***

Annex I (normative):
TSN usage guidelines

# I.1	Determination of traffic pattern information

As described in clause 5.27.2, the calculation of the TSCAI relies upon mapping of information for the TSN stream(s) based upon certain information.

The traffic pattern parameter determination based on PSFP (IEEE P802.1Q [98]) is as follows:

-	Periodicity of a TSN stream is set equal to PSFPAdminCycleTime. For aggregated TSN streams with same periodicity, the periodicity is set equal to PSFPAdminCycleTime received from CNC for one of the TSN streams that are aggregated.

NOTE:	Given that only TSN streams that have the same periodicity can be aggregated, the PSFPAdminCycleTime for those TSN streams is assumed to be the same.

-	Burst Arrival time of a TSN stream at the ingress port is determined based on the following conditions:

-	If the PSFPgateStatesValue is Closed for the first timeIntervalValue, then the Arrival time is set to PSFPAdminBaseTime plus the first timeIntervalValue. If the PSFPgateStatesValue is Open for the first timeIntervalValue, then the Burst Arrival time is set to PSFPAdminBaseTime. For aggregated TSN streams, the arrival time is calculated similarly, but using the time interval to the first PSFPgateStatesValue that is Open from the aggregated TSN streams.

-	Flow direction of a TSN stream is determined based on the following conditions:

-	If the PSFP information is targeted for a DS-TT port, the Flow direction is UL. If the PSFP information is targeted for a NW-TT port, the Flow direction is DL. For aggregated TSN streams, the flow direction of any TSN stream in the group is used for the aggregated flow.

The traffic pattern parameter determination for DL aggregated TSN streams is based on 802.1Qbv [96] as follows:

-	Periodicity of an aggregation of all TSN streams belonging to a traffic class in DL is set equal to AdminCycleTime, if there is a single open gate in the cycle, if there is a single GateControl entry in the cycle where the gate for this traffic class is open.

-	Burst Arrival time of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port is determined based on the following conditions:

-	The Burst Arrival Time should be set to AdminBaseTime plus the sum of the timeIntervalValues for which the gateStatesValue is Closed in the AdminControlList until the first gate open time (i.e. until gateStatesValue set to Open is found) minus the corresponding bridge delay. If the gateStatesValue is Open for the first timeIntervalValue, then the Burst Arrival time is set to AdminBaseTime.

-	Flow direction of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port is determined based on the following:

-	Since determination of traffic pattern based on 802.1Qbv information is targeted for a DS-TT port (DL), the Flow direction is DL.

-	Burst size of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port (which is useful to map to MDBV) is determined as follows:

-	The Burst size is equal to the timeIntervalValue (converted from ns to s) with gateStatesValue = Open multiplied by the bitrate of the corresponding port, if there is a single burst in the cycle.

-	Maximum Flow Bitrate size of an aggregation of all TSN streams belonging to a traffic class in DL at the egress port (which is useful to map to GFBR) is determined as follows:

-	The Maximum Flow Bitrate is equal to the summation of all timeIntervalValue (converted from ns to s) with gateStatesValue = Open, multiplied by the bitrate of the corresponding port, and divided by AdminCycleTime.

***END OF CHANGE***