SA WG2 Meeting #137E	S2-2002057r02

# February 24 - 27, 2020 Electronic meeting    							 (revision of S2-20xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0413 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Determination of QoS parameters applicable to TSN AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no definition on how: - the PCF derives the SDF templates for the PCC Rules. - the GBR, MBR and ARP parameters of the PCC Rule are derived from the TSN AF QoS provided information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Indicate that the PCF determines the GBR, MBR and ARP needed for the QoS profile for the QoS flow to transfer TSN flows. - Indicate that if dynamic 4QI are to be transferred to RAN, those are sent at SM policy assocaition establoshment or modification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support for TSN requirements to build PCC Rules for TSN traffic. Misleading name of the TSN AF container. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.1.3.23	Support of integration with Time Sensitive Networking

Time Sensitive Networking (TSN) support is defined in TS 23.501 [2], where the 5GS represents virtual TSN bridge(s) based on the defined granularity model. The TSN AF and PCF interact to perform QoS mapping as described in clause 5.28.4 of TS 23.501 [2].

The PCF may provide the list of the dynamically assigned 5QI values and their explicity signalled QoS Characteristics to the SMF at Session Management Policy Association modification within the PDU Session related policy information.

The PCF provides the following parameters to the TSN AF: Port Management Container, port numbers associated with the NW-TT and DS-TT, and a UE MAC address (i.e. MAC address of the DS-TT port). The TSN AF may use this information to construct IEEE managed objects, to interwork with IEEE TSN networks.

The TSN AF decides the TSN QoS information (i.e. priority, delay and maximum TSC burst size) based on the received the configuration information of 5GS Bridge from the CNC as defined in clause 5.28.2 of TS 23.501 [2] and the bridge delay information at the TSN AF.

The TSN AF provides an Application Identifier, to allow PCF to determine the ARP and other QoS parameter if not derived from the TSN QoS information and the GBR and MBR values requested for each Flow Description.

The PCF receives a request from the TSN AF that includes UE MAC address (i.e. MAC address of the DS-TT port) for PDU session and the TSN QoS parameters, i.e. priority and delay. The PCF performs Session binding using the UE MAC address, and then the PCF derives the TSN QoS parameters into a 5QI.

The PCF generates one or several PCC Rule(s) with service data flow filter  including the Flow Description(s), the derived QoS information includingthe mapped 5QI. The SMF binds the PCC Rule to a QoS Flow as defined in clause 6.1.3.2.4.

# * * * * End of changes * * * *