3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002144

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2045 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAG-only indication and empty Allowed CAG list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Two problems are solved in this paper: Issue 1: The situation that an UE has a CAG-only indication and an empty list may happen (according to 5.30.3.4). For example, the UE removes the CAG ID from the list after receiving the NAS rejection. If that’s the only CAG ID in the list, the UE may end up with an empty list. When this situation happens, the UE may not initial the access the PLMN from any cell. As a result, if CAG information update in network side, the network may not be able to update UE’s configuration. Issue 2: Assuming CAG information in a UE is not aligned with network, e.g. the UE is a new device without any subscription, when the UE Register to the 5G network, AMF may reject the Registration request based on the not allowed CAG ID list in subscription. However, based on the current specification, the CAG information in the network will not sent to the UE. As a result, the UE have no idea about the CAG information and keep registering to the network via the cell which probably not allowed by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For issue 1, 1, Clarify that a UE may ignore the limitation of configuration if the Allowed CAG list is empty and CAG-only is configured. In other word, UE may keep attemt to register to the 5G network in this scenario. For issue 2, 2, In NAS reject message, the network shall provides the latest Allowed CAG list. The UE will update the configuration accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the UE’s configuration in AMF is with CAG-only indication and an empty list, the UE will be updated with the same configuration. UE will be permanently unable to access to the network. Meanwhile, the network may not be able to provide and update new allowed CAG list to UE’s configuration. The same problem related to new configuration update is also applicable to the case when the configuration in UE side is out of sync with that in network side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.3.3	UE configuration, subscription aspects and storage

To support CAG, the UE may be pre-configured or  (re)configured with the following CAG information, included in the subscription as part of the Mobility Restrictions:

-	an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

-	optionally, a CAG-only indication whether the UE is only allowed to access 5GS via CAG cells (see TS 38.304 [50] for how the UE identifies whether a cell is a CAG cell); If the CAG-only indication is configured and the Allowed CAG list is empty, the UE may ignores the limitation and access non-CAG cells or CAG cells.

The HPLMN may configure or re-configure a UE with the above CAG information using the UE Configuration Update procedure for access and mobility management related parameters described in TS 23.502 [3] in clause 4.2.4.2.,

The above CAG information is provided by the HPLMN on a per PLMN basis. In a PLMN the UE shall only consider the CAG information provided for this PLMN.

The UE shall store the latest available CAG information for every PLMN for which it is provided and keep it stored when the UE is de-registered.

NOTE:	CAG information has no implication on whether and how the UE accesses 5GS over non-3GPP access.

5.30.3.4	Network and cell (re-)selection, and access control

The following is assumed for network and cell selection, and access control:

-	The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell (see TS 38.300 [27], TS 38.304 [50]);

NOTE 1:	The above also implies that cells are either CAG cells or normal PLMN cells.

-	In order to prevent access to NPNs for authorized UE(s) in case of network congestion/overload, existing mechanisms defined for Control Plane load control, congestion and overload control in clause 5.19 can be used, as well as the access control and barring functionality described in clause 5.2.5, or Unified Access Control using the access categories as defined in TS 24.501 [47] can be used.

-	For aspects of automatic and manual network selection in relation to CAG, see TS 23.122 [17];

-	For aspects related to cell (re-)selection, see TS 38.304 [50];

-	The Mobility Restrictions shall be able to restrict the UE's mobility according to the Allowed CAG list (if configured in the subscription) and include an indication whether the UE is only allowed to access CAG cells (if configured in the subscription);

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell, the UE shall provide the selected CAG Identifier to NG-RAN and the NG-RAN shall provide the CAG Identifier to the AMF:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

-	If the CAG Identifier received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

-	During transition from RRC Inactive to RRC Connected state:

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a CAG cell, NG-RAN shall reject the RRC Resume request from the UE if none of the CAG Identifiers supported by the CAG cell are part of the UE's Allowed CAG list according to the Mobility Restrictions received from the AMF.

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a non-CAG cell, NG-RAN shall reject the UE's Resume request if the UE is only allowed to access CAG cells according to the Mobility Restrictions received from the AMF.

-	During connected mode mobility procedures:

-	Based on the Mobility Restrictions received from the AMF:

-	Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and none of the CAG Identifiers supported by the CAG cell are part of the UE's Allowed CAG list;

-	Source NG-RAN shall not handover the UE to a non-CAG cell if the UE is only allowed to access CAG cells;

-	Update of Mobility Restrictions:

-	When the AMF receives the Nudm_SDM_Notification from the UDM and the AMF determines that the Allowed CAG list or the indication whether the UE is only allowed to access CAG cells have changed;

-	The AMF shall update the Mobility Restrictions in the UE and NG-RAN accordingly; and

-	If the UE is currently accessing a CAG cell and the CAG Identifier(s) supported by the CAG cell have been removed from the Allowed CAG list or if the UE is currently accessing a non-CAG cell and the indication that the UE is only allowed to access CAG cells has been set in the subscription, then the AMF shall release the NAS signalling connection for the UE by triggering the AN release procedure.

NOTE 2:	When the UE is accessing the network for emergency service the conditions for AMF in clause 5.16.4.3 apply.

-	After UCU, the AMF may release the NAS signalling connection by triggering the AN release procedure to allow the UE to reselect a cell based on the updated Allowed CAG list and CAG-only indication, e.g., if the CAG Identifier of current cell is not part of the updated Allowed CAG list.

# * * * * End of changes * * * *