3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002146r02

# Elbonia, February 24 – 27, 2020	(revision of S2-2001614)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1799 | rev | 8 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of CAG ID privacy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson, Qualcomm Incorporated, NTT DOCOMO, MediaTek Inc., AT&T, Deutsche Telekom AG, Sony, Nokia, Nokia Shanghai Bell], Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no need for the UE to send a CAG ID to the network (neither in AS signaling nor in NAS signaling) given that: - AMF does not require the CAG ID from the UE to be able to validate whether the UE is allowed in a CAG cell (AMF verifies whether a UE is allowed to access a CAG cell based on the CAG IDs supported by a CAG cell and the Allowed CAG list in the subscription). Further, normally a UE cannot always be trusted, and therefore it would be more appropriate to not rely upon a UE provided CAG ID as input to such validation. - CAG ID is not used for AMF selection and network slice selection as already stated in TS 23.501 clause 5.30.3.1. In addition, SA3 requested in LS S2-1908715 to maintain privacy of the CAG ID such that it is not sent in clear by the UE. Some additional reason for change is: Based on the working assumption from the RAN3 group, the NG-RAN should transfer the supported CAG ID(s) of the CAG cell related to the selected PLMN to the AMF which can then verify whether UE access is allowed by Mobility Restrictions. Because the CAG cell can be shared by different PLMNs, and the CAG ID is only unique in the PLMN, existing descriptions may raise some misunderstandings. For example, it is supposed that CAG cell1 supports CAG-ID1 related to PLMN1, and CAG-ID2 related to PLMN2, and UE only supports PLMN1-CAG-ID2, if CAG-ID1 and CAG-ID2 are both transferred to the AMF in PLMN1, not based on the selected PLMN, wrong judgement will happen. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removing the requirement for the UE to send the CAG ID in AS/RRC, and instead the NG-RAN provides the list of CAG IDs of the CAG cell associated to the selected PLMN to the AMF to enable AMF authorization of the UE. - Clarify the CAG information transferred from RAN to AMF in N2 is based on the selected PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - CAG privacy cannot be maintained, inappropriate design - relying upon UE provided information as input to access authorization. - Existing descriptions may raise some misunderstandings on the CAG cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.3, 5.30.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.30.3.4	Network and cell (re-)selection, and access control

The following is assumed for network and cell selection, and access control:

-	The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell (see TS 38.300 [27], TS 38.304 [50]);

NOTE 1:	The above also implies that cells are either CAG cells or normal PLMN cells.

-	In order to prevent access to NPNs for authorized UE(s) in case of network congestion/overload, existing mechanisms defined for Control Plane load control, congestion and overload control in clause 5.19 can be used, as well as the access control and barring functionality described in clause 5.2.5, or Unified Access Control using the access categories as defined in TS 24.501 [47] can be used.

-	For aspects of automatic and manual network selection in relation to CAG, see TS 23.122 [17];

-	For aspects related to cell (re-)selection, see TS 38.304 [50];

-	The Mobility Restrictions shall be able to restrict the UE's mobility according to the Allowed CAG list (if configured in the subscription) and include an indication whether the UE is only allowed to access CAG cells (if configured in the subscription);

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

NOTE 2:	It is assumed that the AMF is made aware of the supported CAG Identifier(s) of the CAG cell by the NG-RAN.

-	If at least one of the supported CAG Identifier(s) related to the selected PLMN received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If none of the supported CAG Identifier(s) related to the selected PLMN received from the NG-RAN are part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, then the UE removes the CAG Identifier(s) of the CAG cell related to the selected PLMN, if any exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

-	During transition from RRC Inactive to RRC Connected state:

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a CAG cell, NG-RAN shall reject the RRC Resume request from the UE if none of the CAG Identifiers supported by the CAG cell related to the Serving PLMN are part of the UE's Allowed CAG list according to the Mobility Restrictions received from the AMF.

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a non-CAG cell, NG-RAN shall reject the UE's Resume request if the UE is only allowed to access CAG cells according to the Mobility Restrictions received from the AMF.

-	During connected mode mobility procedures:

-	Based on the Mobility Restrictions received from the AMF:

-	Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and none of the CAG Identifiers supported by the CAG cell related to the Serving PLMN are part of the UE's Allowed CAG list;

-	Source NG-RAN shall not handover the UE to a non-CAG cell if the UE is only allowed to access CAG cells;

-	Update of Mobility Restrictions:

-	When the AMF receives the Nudm_SDM_Notification from the UDM and the AMF determines that the Allowed CAG list or the indication whether the UE is only allowed to access CAG cells have changed;

-	The AMF shall update the Mobility Restrictions in the UE and NG-RAN accordingly; and

-	If the UE is currently accessing a CAG cell and the CAG Identifier(s) supported by the CAG cell related to the Serving PLMN have been removed from the Allowed CAG list or if the UE is currently accessing a non-CAG cell and the indication that the UE is only allowed to access CAG cells has been set in the subscription, then the AMF shall release the NAS signalling connection for the UE by triggering the AN release procedure.

NOTE 3:	When the UE is accessing the network for emergency service the conditions for AMF in clause 5.16.4.3 apply.

-	After UCU, the AMF may release the NAS signalling connection by triggering the AN release procedure to allow the UE to reselect a cell based on the updated Allowed CAG list and CAG-only indication, e.g., if the CAG Identifier of current cell related to the Serving PLMN is not part of the updated Allowed CAG list.

# * * * * Second change * * * *

#### 5.30.3.3	UE configuration, subscription aspects and storage

To support CAG, the UE may be pre-configured or (re)configured with the following CAG information, included in the subscription as part of the Mobility Restrictions:

-	an Allowed CAG list i.e. a list of CAG Identifiers related to a PLMN the UE is allowed to access; and

-	optionally, a CAG-only indication whether the UE is only allowed to access 5GS via CAG cells (see TS 38.304 [50] for how the UE identifies whether a cell is a CAG cell);

The HPLMN may configure or re-configure a UE with the above CAG information using the UE Configuration Update procedure for access and mobility management related parameters described in TS 23.502 [3] in clause 4.2.4.2.,

The above CAG information is provided by the HPLMN on a per PLMN basis. In a PLMN the UE shall only consider the CAG information provided for this PLMN.

The UE shall store the latest available CAG information for every PLMN for which it is provided and keep it stored when the UE is de-registered.

NOTE:	CAG information has no implication on whether and how the UE accesses 5GS over non-3GPP access.

# * * * * End of changes * * * *