3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002226

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2204 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for Bridge Delay information reporting and QoS mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The NW-TT residence time should be part of the UPF residence time. - The SMF needs to store the port number of the DS-TT Ethernet port and port number of the serving NW-TT Ethernet port(s) for a PDU Session. Then when it receives a Port Management Information Container, it can deduce the related PDU Session for the port and send it to the DS-TT port or NW-TT port via NAS and N4. - It was agreed that 5GS Bridge delay can be updated after PDU Session establishment based on UE-DS-TT Residence time reported by the SMF. But how does AF derive the corresponding information from the reporting is not clear. - The PDB pre-configured on the TSN AF is not accurate since the TSN AF doesn’t know the clock drift between 5GS and TSN clocks. It is proposed to let SMF to report the PDB in TSN clock during PDU session establishment together with UE Residence Time, so that the TSN AF can derive a more accurate 5GS Bridge Delay and update to CNC. - The delay capability of the PDU Session (i.e. The minimum PDB/delay the PDU Session can provide) is provided by the SMF to the TSN AF and it is in reference to TSN GM. - If the pre-configured bridge delay in the TSN AF is exceeded the delay capability reported by the SMF/PCF (i.e. the pre-configured bridge delay is lower than the minimum PDB), the TSN AF uses the minimum PDB value to update the bridge delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The UPF Residence time includes NW-TT Residence time. - The port number of the DS-TT Ethernet port and port number of the serving NW-TT Ethernet port(s) for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. - The TSN AF deduces the related port pair(s) from the port number of the DS-TT Ethernet port and port number(s) of the serving NW-TT Ethernet port(s) when the TSN AF receives the 5GS Bridge information for a newly established PDU Session and calculates the bridge delays per port pair. - The SMF reports the UE-DS-TT Residence Time and minimum PDB in reference to TSN time to the AF via PCF. The TSN AF updates the 5GS Bridge delay based on the received information from the SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS Bridge delay values reported to the TSN CNC are not correct and the delay requirement of the TSC traffic may not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.5, 5.28.1, 5.28.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 2150 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR 23.503 CR 0425 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.27.5	5G System Bridge delay

In order for 5G System to participate as a TSN bridge according to gate schedules specified, the 5GS bridge is required to provide 5GS Bridge Delays for each port pair and traffic class to an IEEE TSN system. In order to determine 5GS bridge delays, following components are needed:

1.	UE-DS-TT Residence Time: the time taken within the UE and DS-TT to forward a packet between the UE/DS-TT port.

2.	The PDB as defined in clause 5.7.3.4: packet delay between the UE and the UPF that terminates the N6 interface. The PDB is determined from the 5QI (as in standardized 5QI to QoS characteristics mapping as in clause 5.7.4) assigned for the TSC QoS Flow. It is assumed that the PDB includes the UPF Residence Time and the NW-TT Residence time.

Residence times are the same for uplink and downlink traffic. The TSN AF calculates the 5GS independentDelayMax/independentDelayMin value of bridge for each port pair and for each traffic class using the above components.

NOTE 1:	With supporting the hold and forward buffering mechanism in UE-DS-TT and NW-TT, the values of independentDelayMax and independentDelayMin for 5GS Bridge can be the same.

The dependentDelayMax and dependentDelayMin for 5GS Bridge are derived from frame length-dependent delays in DS-TT and in NW-TT for inputting the packet via the ingress port and outputting the packet via the egress port.

NOTE 2:	Whether the values of dependentDelayMax and dependentDelayMin are the same is dependent on implementation.

Since Residence times may vary among UEs and among UPFs, the 5GS bridge delay is determined after the PDU Session Establishment for the corresponding UPF and the UE by the TSN AF. The TSN AF deduces the related port pair(s) from the port number of the DS-TT Ethernet port and port number of the serving NW-TT Ethernet port(s) when the TSN AF receives the 5GS Bridge information for a newly established PDU Session and caculates the bridge delays per port pair.

UE-DS-TT Residence Time is provided at the time of PDU Session Establishment by the UE to the SMF.

# * * * * Second change * * * *

### 5.28.1	5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the ports on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the logical TSN bridge is per UPF. The bridge ID of the logical TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side, a port on UPF/NW-TT side, and a PDU Session during reporting of logical TSN bridge information.

There is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single virtual bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5G virtual bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 1:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based virtual bridge

NOTE 2:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple logical TSN bridges.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Report the bridge information of 5GS Bridge to TSN network.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes at least the following:

-	Information for 5GS Bridge:

-	Bridge Address (unique MAC address that identifies the bridge used to derive the bridge ID);

-	Bridge Name;

-	Number of Ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	Topology of 5GS Bridge as defined in IEEE 802.1AB [97]:

-	Chassis ID subtype and Chassis ID of the 5GS Bridge.

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q [98].

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

Bridge ID of the 5GS Bridge, port numbers of the Ethernet port in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs. Port number of Ethernet port on the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment. The port number of the DS-TT Ethernet port and port number(s) of the serving NW-TT Ethernet port(s) for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. SMF provides the port numbers and MAC addresses of the Ethernet ports in DS-TT and NW-TT of the related PDU sessionto the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released, then SMF informs TSN AF accordingly.

The AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

# * * * * Third change * * * *

### 5.28.4	QoS mapping tables

The mapping tables between the traffic class and 5GS QoS Profile is provisioned and further used to find suitable 5GS QoS profile to transfer TSN traffic over the PDU Session. QoS mapping procedures are performed in two phases: (1) QoS capability report phase as described in clause 5.28.1, and (2) QoS configuration phase as in clause 5.28.2

(1)	The TSN AF shall be pre-configured (e.g. via OAM) with a mapping table. The mapping table contains TSN traffic classes, pre-configured bridge delays, and priority levels. Once the PDU session has been setup, and after retrieving the information related to DS-TT residence times, the AF updates bridge delays per port pair and traffic class and reports the bridge delays and other relevant TSN information such as the Traffic Class Table for every port, according to the IEEE 802.1Q [98] and IEEE 802.1Qcc [95] to the CNC.

(2)	CNC distributes the TSN QoS requirements and TSN scheduling parameters to 5G virtual bridge via TSN AF.

The PCF mapping table provides a mapping from TSN QoS information (see TS 23.503 [45], clauses 6.2.1.2 and 6.1.3.23) to 5GS QoS profile. Based on trigger from TSN AF, the PCF may trigger PDU session modification procedure to establish a new 5G QoS flow or use the pre-configured 5QI for 5G QoS Flow for the requested traffic class according to the selected QoS policies and the TSN AF traffic requirements.

Figure 5.28.4-1 illustrates the functional distribution of the mapping tables.

Figure 5.28.4-1: QoS Mapping Function distribution between PCF and TSN AF

The minimum set of TSN QoS-related parameters that are relevant for mapping the TSN QoS requirements are used by the TSN AF: traffic classes and their priorities per port,  5GS bridge delays per port pair and traffic class (independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), and propagation delay per port (txPropagationDelay).

NOTE 2:	At 5QI selection in the QoS mapping table the PCF takes into consideration the maximum burst size of TSN streams in the port traffic class.

Once the CNC has received the necessary information, it proceeds to calculate scheduling and paths. The configuration information is then set in the bridge as described in clause 5.28.2 and 5.28.3. At this point, it is possible to retrieve the TSN QoS requirements by identifying the traffic class of the port. The traffic class to TSN QoS and delay requirement mapping can be performed using the QoS mapping table in the TSN AF as specified in TS 23.503 [45]. Subsequently in the PCF, the 5G QoS flow can be configured by selecting a 5QI as specified in TS 23.503 [45]. This feedback approach uses the reported information to the CNC and the feedback of the configuration information coming from the CNC to perform the mapping and configuration in the 5GS.

QoS mapping table in the PCF between TSN parameters and 5GS parameters should match the delay and priority, while preserving the priorities in the 5GS. An operator enabling TSN services via 5GS can choose up to eight traffic classes to be mapped to 5GS QoS profiles.

Once the 5QIs to be used for TSN streams are identified by the PCF as specified in TS 23.503 [45], then it is possible to enumerate as many bridge port traffic classes as the number of selected 5QIs.

# * * * * End of changes * * * *