SA WG2 Meeting #S2-137E e-meeting	S2-2002241r01

24 - 27 February, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2155 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting EPS interworking with ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2020-2-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Although 5GS has supported mobility to EPS without N26 interface, it is not clear whether and how the interworking without N26 is supported for the UE having MA PDU session(s). It is proposed to capture the simple modification on the exisiting procedure for the 5GS to EPS interworking without N26 in order to support ATSSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that during the interworking procedure without N26 interface, for the UE having MA PDU session(s), PGW-C + SMF releases the session over N3GPP access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature is incomplete and the deployment of ATSSS can be restricted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.6.1, 4.22.6.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.22.6.1	General

This clause includes procedures for interworking with EPS.

* * * Second Change * * * *

4.22.6.2.X	5GS to EPS mobility without N26 interface

Based on the signaling flow in Figure 4.11.2.2-1, the procedure is performed with the following differences and modifications:

-	In step 10 (and step 13 in clause 4.11.2.4.1), if the MA PDU Session is established in 5GS and the MA PDU Session is moved to EPS, the PGW-C + SMF triggers the MA PDU Session Release procedure in 5GS.

-	In step 13, during the additional PDN Connectivity Procedure, if the MA PDU Session is established in 5GS and the MA PDU Session is moved to EPS, the PGW-C + SMF triggers the MA PDU Session Release procedure in 5GS.

* * * End of Change * * * *