SA WG2 Meeting #138E (e-meeting)	S2-2002682r01

# April 20 - 24, 2020, Elbonia

Title:	[Draft] Reply LS on manual CAG ID selection and granularity of UAC parameters for PNI-NPNs

Response to:	LS on manual CAG ID selection and granularity of UAC parameters for PNI-NPNs

Release:	Rel-16

Work Item:	NG_RAN_PRN-Core, Vertical_LAN

Source:	SA WG2

To:	RAN WG2

CC:	CT WG1

# Contact Person:

# Name:	Xiaowan Ke

Email Address:	xiaowan.ke@vivo.com

# Attachments:

# 1. Overall Description:

SA2 thanks RAN2 for LS on Manual CAG ID selection and granularity of UAC parameters for PNI-NPNs (S2-2002656/R2-2002417). Accordingly, SA2 would like to provide the feedback to the questions as follows:

- Question 1.1; TO: SA2; CC: CT1

If a UE performs manual CAG selection and a successful registration, then whether the UE shall stay on cells supporting the manually selected CAG ID in RRC_CONNECTED state especially in the case when after registration the Allowed CAG List in the UE does not contain the manually selected CAG ID?

Answer1: If a UE performs manual CAG selection and a successful registration, the UE’s lastest allowed CAG ID list is provided to the UE in the Registration Accept Message.

-  In case the manually selected CAG ID is in the UE’s latest allowed CAG ID list, the UE shall stay the selected cell and under the control of NG RAN during RRC_CONNECTED state..

-  In case the manually selected CAG ID is not in the UE’s latest allowed CAG ID list at the network side but the selected cell supports both the manually selected CAG ID and at least one of the allowed CAG ID for the UE, the UE registration can be successful. This is described in 23.501 clause 5.30.3.4 quote:

-     If at least one of the CAG Identifier(s) received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

SA2 understands only this case can match RAN’s assumption “especially in the case when after registration the Allowed CAG List in the UE does not contain the manually selected CAG ID”. In this case, the selected cell is allowed for the UE, UE shall stay at the selected cell and under the control of NG RAN during RRC_CONNECTED state.

- In case the selected cell doesn’t allowed for the UE but registration success, no matter the manually selected CAG ID is in the UE’s latest allowed CAG ID list not, this is an error case and need not be took into account.

- Question 1.2; TO: SA2; CC: CT1

Shall a UE prioritize for cell reselection the cells supporting the manually selected CAG ID over other suitable cells that do not support the manually selected CAG ID after a successful registration?

Answer2: After a successful registration, in case the UE is still in manually selection mode during cell reselection and the manually selected CAG ID is in the UE’s latest allowed CAG ID list, the manually selected CAG ID shall be used for cell reselection; in case the manually selected CAG ID is not in the UE’s latest allowed CAG ID list, the UE shall turn to automatically selection mode. In case of automatically selection mode, the latest allowed CAG ID list is used for cell reselection and the manually selected CAG ID is not used any more.

# 2. Actions:

# To RA2:

# ACTION: 	SA2 respectfully ask RAN2 to take the above into account.

# 3. Date of Next TSG-RAN WG2 Meetings:

SA2#139E (TBD)	25-29 May 2020			(TBD)

SA2#139Adhoc (TBD)	13-17 July 2020			(TBD)