3GPP TSG-SA2 Meeting #138-E	S2-2002696r01

# Online, , 20th Apr 2020 - 23rd Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2177 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Availability after DDN Failure event with UPF buffering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The procedures do not consider an optional PCC control. At SA2#137e there was a principle agreement to follow solution B in discussion paper S2-2001809 and corresponding changes for the DDD status event were already agreed (see S2-2002347). - Step 0 to configure the UPF to buffer packets as described in clause 5.8.3 in TS 23.501 is assumed to be always executed before the AF-initiated "Availability after DDN failure" subscription procedure in step 0. This is incorrect. The SMF rather configures the UPF to buffer packets only when the UE is not reachable and removes that configuration again when the UE becomes reachable to trigger the UPF to transmit the buffeed packets. While the UPF is configured to buffer, it will never transmit packets. - Step 2 intermingles UPF configuration to report discarded packets and discarding packets. However, the discarding happens at a later point in time than the configuration and will not always occur |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Procedures for the case where PCC control is used as proposed as solution B in S2-2001809 are added. - Step 0 is removed. - Step 2 is split into two parts; Configuration and discarding packets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - An optional PCC control is not possible for the "Availability after DDN Failure" event with UPF buffering - Always applied UPF buffering configuration would prevent than any downlink packets are delivered. - The proccedures in step 2 are hard to comprehend and confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.15.3.2.9	Information flow for Availability after DDN Failure with UPF buffering

The procedure is used if the SMF requests the UPF to buffer packets. The procedure describes a mechanism for the Application Function to subscribe to notifications about availability after DDN failure.

Cancelling is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID.

Figure 4.15.3.2.9-1: Information flow for availability after DDN Failure event with UPF buffering

1.	AF interacts with NEF to subscribe availability after DDN failure subscription procedure as described in clause 4.15.3.2.7.

2.	If the SMF is informed the UE is unreachable via a Namf_Communication_N1N2MessageTransfer service operation, the SMF provides the Traffic Descriptor in the PDR and requests the UPF to report when there are corresponding discarded packets in the UPF as described in clause 5.8.3 in 23.501 [2]. If PCC is not used and there is no installed PDR with the exact same traffic descriptor, the SMF copies the installed PDR that would have previously matched the incoming traffic described by the traffic descriptor in the notification subscription, but provides that traffic descriptor, a higher priority, and a buffer notification action within.
If PCC is used. the SMF checks if the Policy Control Trigger Request are set to report the reception of a subscription to DDN failure and then the SMF requests PCC Rules to the PCF before contacting the UPF; the PCF then provides PCC rule(s) including a request for notifications about DDN failure.

3-4.	When the UPF determines to discard DL data matching the received PDR, it reports the corresponding event to the SMF and the SMF sends the event reports the DDN Failure status with NEF Correlation ID, by means of Nsmf_PDUSession_SMContextStatusNotify Nsmf_EventExposure_Notify message, to the AMF indicated as notification endpoint. If the UE is not reachable after the AMF received the notification from the SMF, the AMF shall set a Notify-on-available-after-DDN-failure flag corresponding to the Notification Correlation Id and the identifier of the UE if available.

5-6.	[Conditional] The AMF detects the UE is reachable and sends the event report(s) based on the Notify-on-available-after-DDN-failure flag, by means of Namf_EventExposure_Notify message(s), only to the NEF(s) indicated as notification endpoint(s) identified via the corresponding subscription in step 3. In this way, only the AF(s) for which DL traffic transmission failed are notified.

7.	The NEF sends Nnef_EventExposure_Notify message with the "Availability after DDN Failure" event to AF.