SA WG2 Meeting #138E (e-meeting)	S2-2002804r02

# April 20 – 23, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0154 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of data collection for any UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the moderated email discussion, data analytics for any UE needs to be supported in Rel 16, so that the NWDAF shall be able to collect data for any UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions on how NWDAF collects data for any UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data analytics for any UE can not be supported as data collection for any UE is not support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

### 6.2.2	Data Collection from NFs

#### 6.2.2.1	General

The Data Collection from NFs is used by NWDAF to subscribe/unsubscribe at any 5GC NF to be notified for data on a set of events.

The Data Collection from NFs is based on the services of AMF, SMF, UDM, PCF, NRF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF as defined in TS 23.502 [3] clause 4.15 and clause 5.2.

-	other NF services (e.g. Nnrf_NFDiscovery and Nnrf_NFManagement in NRF as defined in TS 23.502 [3] clause 4.17)

This data collection service is used directly in order to retrieve behaviour data for individual UEs or groups of UEs (e.g. UE reachability), and also to retrieve global UE information (e.g. Number of UEs present in a geographical area).

Table 6.2.2.1-1: NF Services consumed by NWDAF for data collection

| Service producer | Service | Reference in TS 23.502 [3] |
| --- | --- | --- |
| AMF | Namf_EventExposure | 5.2.2.3 |
| SMF | Nsmf_EventExposure | 5.2.8.3 |
| PCF | Npcf_EventExposure (for a group of UEs or any UE) Npcf_PolicyAuthorization_Subscribe (for a specific UE) | 5.2.5.7 |
| UDM | Nudm_EventExposure | 5.2.3.5 |
| NEF | Nnef_EventExposure | 5.2.6.2 |
| AF | Naf_EventExposure | 5.2.19.2 |
| NRF | Nnrf_NFDiscovery | 5.2.7.3 |
|  | Nnrf_NFManagement | 5.2.7.2 |

NOTE 1:	The present document specifies that NWDAF can collect some UPF input data for deriving analytics, but how NWDAF collects these UPF input data is not defined in this Release of the specification.

To retrieve data related to a specific UE, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| Type of NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

To retrieve mobility related data for any UE, the NWDAF has to discover all related AMF instances by querying NRF with the received Analytics Filter Information. Then the NWDAF subscribes UE mobility events from the AMF instances, in the subscription request, the target of event reporting is set to any UE and the event filter(s) is set according to the Analytics Filter Information. The AMFs identify the target UEs based on event filter(s) and report the required data per each specific UE.

To retrieve communication related data for any UE, if Analytics Filter Information doesn’t include Area of Interest, the NWDAF can discover all related SMF instances by querying NRF with the received Analytics Filter Information, and then subscribe UE communication related events from the SMF instances, in the subscription request, the target of event reporting is set to any UE and the event filter(s) is set according to the Analytics Filter Information. The SMFs identify the target UEs based on the event filters, and report the required data per each specific UE. Otherwise, the NWDAF can:

-	Query the NRF with the received Analytics Filter Information to discover all related AMF instances, then subscribe to all AMF instances to obtain the UE list within the Area of Interest.  Based on the obtained UE list, the NWDAF retrieves the required data from SMFs per each specific UE. or

-	Query the NRF with the received Analytics Filter Information to discover all related SMF instances, then subscribes to all SMF instances with the target of event reporting set to any UE, the event filter(s) in subscription request includes the Area of Interest. The SMFs contacted by the NWDAF shall subscribe UE list within the Area of Interest from the corresponding AMF(s), then the SMF can report the required events for each specific UE within the UE list obtained from the AMF(s).

The UDM instance should be determined using NRF as described in clause 4.17.4 of TS 23.502 [3] and factors to determine as described in clause 6.3.8 of TS 23.501 [2].

The AMF, SMF instances should be determined using a request to UDM providing the SUPI or the group identity. To determine the SMF serving a PDU session, the NWDAF should in addition provide the DNN and S-NSSAI of this PDU Session; otherwise the NWDAF will obtain a list of possibly multiple SMFs (e.g. one per PDU session).

The BSF instance should be discovered using NRF thanks to optional request parameters (e.g. DNN list, IP domain list, IPv4 address range, IPv6 prefix range) as stated in clause 4.17.4 of TS 23.502 [3], or based on local configuration at the NWDAF.

The PCF instance serving UE PDU Session(s) should be determined using a request to BSF with the allocated UE address, DNN and S-NSSAI.

When NWDAF receives a request addressed to an Internal Group ID from a consumer, NWDAF may need to initiate data collection from several 5GC NFs, such as AMF, SMF, UDM, PCF, NEF/AF, etc. NWDAF may first discover the instances of the required 5GC NFs deployed in the network, e.g. by querying NRF.

For discovering the UDM, NWDAF can query the NRF with the Internal Group ID as the target of the query. For discovering AMF, SMF, PCF, NEF, and AF, NWDAF may need to discover all the instances in the network by using the Nnrf_NFDiscovery service.

NOTE 2:	It is assumed that all members of an Internal Group ID belong to the same UDM Group ID. NWDAF can select a UDM instance supporting the UDM Group ID of the Internal Group ID.

Then, if data needs to be collected from AMF, SMF, UDM, and PCF, NWDAF may initiate the data collection with the Internal Group ID as the target, e.g. subscribing to the event exposure in all the instances of a given type of network function. This subscription to all the instances of required source of event exposure handles, e.g. mobility of UEs across AMFs, or initiation of new PDU sessions with different allocated SMFs.

For collecting data from AMF and SMF, NWDAF may collect the data directly from AMF and/or SMF, or indirectly via UDM, according to TS 23.502 [3] clause 4.15.3.2.3.

The NEF instance that is serving a specific network slices and/or applications of a UE should be determined using NRF using optional request parameters as defined in clause 6.3.14 of TS 23.501 [2]

If NWDAF needs to collect data from an AF deployed outside the operator's domain, the NWDAF shall contact NEF with a SUPI or Internal Group ID as the target of the data collection. NEF is responsible for translation of SUPI to GPSI, or internal to external group identifiers, by querying UDM, prior to contacting the AF.

To retrieve mobility related data for any UE, the NWDAF subscribes to events from the AMF and/or SMF instances it has determined, setting the target of event reporting to "any UE" and the event filter(s) according to the Analytics Filter Information.

* * * End of changes * * *