3GPP TSG-WG SA2 Meeting #138E e-meeting  	S2-2002976

# Elbonia, April 20 – 23, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2241 | rev | 03 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating NRF Status Notify service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-004-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a consumer subscribes to the UE abnormal behaviour analytics ID with analytics target set to “Any UE” and analytics filter information contains an indication of AoI, NWDAF has to determine all the AMFs and SMFs that are serving the indicated AoI. A subscription to analytics ID will lead to multiple analytics output generation over time. To be certain that NWDAF is collecting data from the updated list of AMF and SMFs, NWDAF should use the available mechanisms form NRF to be periodically updated. As defined in TS 23.288 Clause 6.2.2.4, Nnrf_NFDiscovery_Request and the Nnrf_NFManagement_NFStatusNotify can both be used for “set-up and maintain a consistent network map for data collection”. Nnrf_Discovery_Query service is a request/response mode of acquiring information that supports already a query based on the TAIs. Using this mechanism to maintain the map required by NWDAF would lead to eventually unnecessary signalling between NRF and NWDAF, if the AMF and SMF NF Profiles had no change of their supported TAIs (support for TAIs in AMF and SMF NF profile is defined TS 29.510, respectively in Clause 6.1.6.2.11 Clause 6.1.6.2.12). Using the Nnrf_NFManagement_StatusSubscribe service enables NWDAF to reduce signalling, as NWDAF would only be notified if the NF profiles are changed. But, Nnrf_NFManagement_StatusSubscribe does not support to filter the notifications of AMFs and SMFs per TAIs. Which in its turn, will lead again to unnecessary signalling between NWDAF and NRF, when the changes are not related to the TAIs of the NF Profiles from AMF and SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new input parameter in the Nnrf_NFManagement_StatusSubscribe service operation to allow the subscription to status of AMF and SMF to be filtered by TAIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Subscriptions to abnormal behaviour with “Any UE” and AoI in the analytics filter informaiton run the risk of being generated wihtout the updated information about AMFs and SMFs in the AoI. This can increase the risk of misuse of UEs to go undeteced in the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.2.7.2.5	Nnrf_NFManagement_NFStatusSubscribe service operation

Service Operation name: Nnrf_NFManagement_NFStatusSubscribe.

Description: Consumer can subscribe to be notified of the following:

-	Newly registered NF along with its NF services.

-	Updated NF profile.

-	Deregistered NF.

Inputs, Required: NF type (if NF status of a specific NF type is to be monitored), NF instance ID (if NF status of a specific NF instance is to be monitored), NF service (if NF status for NF which exposes a given NF service is to be monitored).

Inputs, Optional:

-	PLMN ID of the target NF/NF service, in the case of the subscription to the status of NF/NF service instance(s) in home PLMN from the serving PLMN.

-	For the UPF Management defined in clause 4.17.6: UPF Provisioning Information as defined in that clause.

-	For AMF, Consumer may include list of GUAMI(s).

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	For AMF and SMF, Consumer may include a list of TAIs.

-	For NWDAF, Consumer may include Analytics ID(s) and TAI(s). Details about NWDAF discovery and selection are described in clause 6.3.13, TS 23.501 [2].

-	For NEF, Consumer may include Event ID(s) provided by AF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

NOTE:	Alternatively, other means such as OA&M can also be used to subscribe for NF status.

# * * * * End of changes * * * *