SA WG2 Meeting #139E	S2-2003750

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #6, New Sol: UAV Controller and UAV association

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract: This contribution proposes a solution for Key Issue #6 – UAV Controller and UAV association in TR 23.754.

# 1. Introduction

We propose to include this solution for Key Issue 6 - UAV Controller and UAV association.

The Key issue#6 is about how are an UAV Controller and a (set of) UAV(s) associated and considered a UAS, e.g. to enable UTM flight mission authorization for the UAS, and to what extent is the 3GPP system involved    in the association?

START of CHANGE

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  |  | X |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| X |  |  |  |  |  | X |  |

NEXT CHANGE (All text is new)

## 6.X	Solution #X: UAV Controller and UAV Association

### 6.X.1	Introduction

This solution describes procedures in which UTM maintains the UAS configuration containing UAV, UAC and the location/area in which the UAS configuration may be valid. Based on certain factors (e.g. mobility, location, tracking information, etc.) the UTM may trigger the association or disassociation of the UAV from a UAC or UAS either via user plane (C2-Communication link) or via control plane (using UE Configuration Update procedure by AMF).

### 6.X.2	Functional Description

The solution considers the following assumptions:

UTM maintains a UAS configuration that contains the association of the UAV, UAC, association and the area in which the UAS is valid.

Table 6.X.2-1: UAS configuration

| UAS Configuration | UAV Controller | UAV | Locations |
| --- | --- | --- | --- |
| UAS 1 | UAV controller-A | UAV a/b/c | Area-A |
| UAS 2 | UAV controller-B | UAV d/e/f | Area-B |
| …… | ….. | …… | …… |

The location column in the table defines an area in which the UAV can be operated by a particular UAC. This is applicable to UAV/UAC registered for business enterprise purpose. The location restriction may not be applicable for UAV/UAC registered for recreational purpose.

Location/Area may be geofence boundary, or a city or group of Tracking Areas/Cell IDs. The location area definition is implementation dependent.

The solution proposes the procedure how UAV Controller and UAV are associated and disassociated to a UAS configuration. In UAS system, there are some areas where the UAV(s) are controlled by the designated UAV Controller to ensure stability, reliability, latency, etc.

The UAS configuration may be updated by UTM in case of e.g. UAV moves out of the approved location area, UAC encounters a critical problem (crash, reset, etc).

### 6.X.3	Procedures

#### 6.X.3.1 	UAV association update by UTM via AMF

The procedure in Figure 6.X.3-1 shows the procedure of a UAV association update by UTM via AMF:

Figure 6.X.3.1-1: UAV association update by UTM via AMF

Pre-Conditions:

-	UAV, UAV Controller-A and UAV Controller-B have registered on 5GC.

-	UAV and UAV Controller-A are authorised and considered as a UAS.

Procedures:

1.	UTM maintains the UAS Configuration table.

2.	UAV and UAV Controller-A are associated and registered on 5GC. UAV/UAV controller-A report Tracking information to UTM via the user plane.

3.	UTM determines to update the association comparing the tracking information with UAS Configuration table. UTM triggers to disassociate with UAV Controller-A and associate with target UAV Controller-B. Following cause codes could be optionally provided to the UTM:

-	Cause code#1: UAV moves from area-A to area-B which are covered by different UAV Controllers.

-	Cause code#2: Current UAV Controller becomes unavailable unexpectedly.

4. UTM initiates Association Update Request to AMF(s) serving the UAV(s) and UAC(s) with following information:

-	UAV ID, Current UAV Controller-A ID and Target UAV Controller-B ID.

-	Cause code.

6.	AMF sends UE Configuration Update Command to UAV Controller-B with the related information for association:

-	UAV ID, UAV Controller-B ID.

-	Association Reconfiguration for association.

-	Cause code.

7.	UAV Controller-B sends UE Configuration Update Complete to AMF.

8.	AMF sends UE Configuration Update Command to UAV with the related information for association:

-	UAV ID, UAV Controller-B ID.

-	Association Reconfiguration for association.

-	Cause code.

9.	UAV sends UE Configuration Update Complete to AMF.

10.	AMF sends UE Configuration Update Command to UAV Controller-A with the related information for disassociation:

-	UAV ID, UAV Controller-A ID.

-	Association Reconfiguration for disassociation.

- 	Cause code.

11.	UAV Controller-A sends UE Configuration Update Complete to AMF.

12.	AMF sends Association Update Response to UTM:

-	UAV ID, UAV Controller-B ID,

NOTE: 	In case the UAC is a non-3GPP device or connected to UAV via non-3GPP access, the UAV will inform the UAC via the C2-communication link prior to connect or disconnect..

13.	UAV and UAV Controller-B are associated and report the tracking information to network.

### 6.X.4	Impacts on existing entities and interfaces

The solution has impacts in the following entities:

-  AMF <--> UTM communication needs to be defined.

### 6.X.5 	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

End of CHANGES