SA WG2 Meeting #S2-139e	S2-2003567r02

June 1 ~ 12, 2020

Source:	Juniper Networks, Vivo, BBC

Title:	5MBS terminologies on delivery methods

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

# 1	Introduction

As discussed in the mailing list some time ago, it is important to clarify and synchronize on the use of some terminologies related to the addressing and delivery scheme.

# 2	Proposal

This document proposes to standardize the use of various terminologies, which are explained in clause 4.4 below.

A companion S2-2004162 covers the changes in TR 23.757 to use these terminologies.

- * * * * First change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Broadcast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

Broadcast service area: The area within which data of one or multiple Broadcast session(s) are sent.

Broadcast session: A session to deliver the broadcast communication service. A broadcast session is characterised by the content to send and the geographical area where to distribute it.

Multicast service area: The area within which data of one or multiple Multicast session(s) are sent.

Editor's note:	The definitions of broadcast service area and multicast service area may be updated further according to 5MBS study.

MBS session: A multicast session or a broadcast session.

Multicast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data).

NOTE:	The transport in the 5GC for broadcast and multicast services will be determined as part of this study. A multicast communication service could for instance use a lower layer unicast or multicast transfer in the 5GC and the access interfaces.

Multicast session: A session to deliver the multicast communication service. A multicast session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it.

Receive Only Mode: A UE configuration option that allows a UE to receive only broadcast service without the need to access and register with the PLMN offering the MBS service. Use of Receive Only Mode does not require USIM for the UE.

Shared MBS Network: A network shared by multiple PLMNs that provides multicast or broadcast services. At least the northbound data entrance point in the 5GS is shared.

5GS Individual delivery: 5GS receives a single copy of MBS packet and delivers MBS packets to UE via per UE PDU session.

NOTE:	It will be determined based on the selected solutions whether the 5GS Individual delivery method is supported.

5GC shared delivery with PTP: 5G CN receives a single copy of MBS packet and delivers a single copy of an MBS packet to a RAN node, and the RAN node delivers each individual copy over radio to UE individually.

5GC shared delivery with PTM: 5G CN receives a single copy of MBS packet and delivers a single copy of an MBS packet to a RAN node, and the RAN node delivers a single copy of each MBS packet over radio to a set of UEs.

- * * * * Next change * * * *

# X4.4	MBS Traffic delivery methods

MBS traffic needs to be delivered from a single data source (Application Service Provider) to multiple UEs. Depending on many factors, multiple delivery modes may be used to deliver MBS traffic in the 5GS. For clarity, delivery methods are not referred to as unicast/multicast/broadcast but as described below.

From the view point of 5G CN, two delivery methods are possible:

- 5GS Individual delivery method: 5G CN receives a single copy of MBS packet and delivers individual copies of MBS packets to UEs via per-UE PDU sessions.

NOTE:	It will be determined based on the selected solutions whether the Individual delivery method is supported.

- 5GC Shared delivery method: 5G CN receives a single copy of MBS packet and delivers a single copy of an MBS packet to a RAN node, which then delivers it to one or multiple UEs. From the view point of RAN, (in case of the shared delivery) two delivery methods are available for the transmission of MBS packet flows over radio:

    - Point-to-Point (PTP) delivery method: a RAN node delivers each individual copy of an MBS packet over radio to a UE individually.

    - Point-to-Multipoint (PTM) delivery method: a RAN node delivers a single copy of each MBS packet over radio to a set of UEs.

A RAN node may use a combination of PTP/PTM to deliver an MBS packet to UEs.

NOTE x: The PTP/PTM delivery method is defined in RAN WGs and they’re listed here for reference only.

As depicted in the following figure, Shared PTP or PTM delivery method and Individual delivery method can be used at the same time for a 5G MBS session .

Figure X4.4-1: Schematic showing delivery methods

- * * * * End of first change * * * *