SA WG2 Meeting #139e	S2-2003662r12

- June 1th-12th, 2020 ; Elbonia        		       	(revision of S2-2003662)

Source: 	Nokia, Nokia Shanghai Bell, Apple, Broadcom, ZTE, Deutsche Telekom, Ericsson, Tencent

Title: 	New Key Issue for R17 ATSSS: Additional Steering Methods

Document for: 	Agreement (P-CR)

Agenda Item: 	8.6

Work Item / Release:	FS_ATSSS_Ph2 / Rel-17

Abstract of the contribution: New Key Issue for R17 ATSSS: Additional Steering Methods

# 1 Discussion

This text in

security aspects, e.g. related with QUIC currently mandating usage of TLS 3.0, will be studied in conjunction with SA3;

is not entirely correct. QUICv1 mandates TLS 3.0 for key exchange, but only during an initial handshake. It does not use TLS records, as described in https://tools.ietf.org/html/draft-ietf-quic-tls-28#section-3.

## 3.  Protocol Overview

QUIC [QUIC-TRANSPORT] assumes responsibility for the confidentiality

and integrity protection of packets.  For this it uses keys derived

from a TLS handshake [TLS13], but instead of carrying TLS records

over QUIC (as with TCP), TLS Handshake and Alert messages are carried

directly over the QUIC transport, which takes over the

responsibilities of the TLS record layer, as shown in Figure 3.

+--------------+--------------+ +-------------+

|     TLS      |     TLS      | |    QUIC     |

|  Handshake   |    Alerts    | | Applications|

|              |              | |  (h3, etc.) |

+--------------+--------------+-+-------------+

|                                             |

|                QUIC Transport               |

|   (streams, reliability, congestion, etc.)  |

|                                             |

+---------------------------------------------+

|                                             |

|            QUIC Packet Protection           |

|                                             |

+---------------------------------------------+

Figure 3: QUIC Layers

QUIC also relies on TLS for authentication and negotiation of

parameters that are critical to security and performance.

This text would be more correct as

security aspects, e.g. related with QUIC currently mandating usage of TLS 3.0 for key exchange, authentication, and negotiation of security and performance parameters, will be studied in conjunction with SA3;

# 2 Proposal

It is proposed to update TR 23.700-93 as follows

FIRST CHANGE (all text is NEW)

## 5.y	Key issue #y: Additional Steering Functionalities

### 5.y.1	Description

This key issue aims to study whether and how to support additional steering functionality(ies).

Traffic splitting for Ethernet and UDP based traffic is not fully supported in Rel-16: for example, traffic within an UDP/IP flow may not be split across multiple accesses without introducing out of order packet delivery.

NOTE:	IETF is actively defining QUIC with the target to have the core protocol sent to IESG in July 2020 and planning for a Multipath extension document sent to IESG by end of 2021.

The key issue will study:

-	whether additional steering functionality(ies) can be defined for ATSSS_Ph2, and if defined study:

-	use -cases of traffic splitting for Ethernet and UDP;

-	the impact on user plane performance of additional steering functionality(ies);

-	how to negotiate the support of additional steering functionality(ies) between the UE and the network and potentially between NF(s) (e.g. between SMF and UPF);

-	how it(they) co-exist with MPTCP and ATSSS-LL from Rel-16;

-	whether and how to enhance PCC rules, ATSSS rules and N4 rules to support these additional steering functionality(ies);

-	what type of traffic these new steering functionality(ies) address, e.g.

-	PDU Session type,

-	Ethernet / UDP,

-	whether and how to support latency sensitive and real time traffic,

-	etc.

-	UE impacts in order to support additional steering functionality(ies).

If steering functionality(ies) defined require(s) new protocol(s) between the UE and the 5GC, the work will (as per study item scope) focus only on the usage of QUIC protocol and its extensions from IETF, considering the following:

-	security aspects, e.g. related with QUIC currently mandating usage of TLS 3.0 for key exchange, authentication, and negotiation of security and performance parameters, will be studied in conjunction with SA3;

-	work on this key issue may trigger additional liaison exchanges with IETF.

End of CHANGES