SA WG2 Meeting #139E	S2-2003762r02

01 - 12 June, 2020, Electronic, Elbonia

Source:	NEC

Title:	KI#15 Sol#3: Update to User consent for UE data colection

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes some update and Editor’s Note resolution in Solution #3 User consent for UE data collection and Editor’s Note resolution.

# 1	Background

This contribution proposes the following updates to Solution #3 User consent for UE data collection of Key Issue #15 User consent for UE data collection/analysis in TR 23.700-91 0.3.0.

* * * * Start of Change * * * *

## 6.3	Solution #3: User consent for UE data collection

### 6.3.1	Description

#### 6.3.1.1	General

To satisfy the requirement in Key Issue #15: User consent for UE data collection/analytics for privacy of some of the UE data, a User consent may be required for some or all the UE information. It is network operator's responsibility to collect and manage the User consent, if required, before initiating UE data collection and reporting by the UE. The user consent information can be considered as part of the subscription data.

Editor's note:	Coordination with SA3 is required.

#### 6.3.1.2	Procedure

The provision of the User consent for the UE data collection is shown in Figure 6.3.1.2-1 below.

Figure 6.3.1.2-1: User consent provision for UE data collection

1). 	Nnef_ParameterProvision_Update Request(UE_ext_Id, user consent for UE data collection) - The Service Provider (AF) may collect the user consent for UE data collection (if applicable) from its users and provide the information via the NEF in the user consent for UE data collection parameter along with the UE_ext_Id for which the user consent applies. The Service Provider may also update or revocate the user consent.

2). AF authorisation – The NEF checks whether the AF is authorized for user consent provision or update.

3). The NEF may inquire with the UDM to translate UE ext_Id to a 3GPP internal identity UE_Id. Then the NEF forwards the request from the AF to the UDM via Nudm_ParameterProvision_Update Request (UE_Id, user consent for UE data collection).

4).The user consent for UE data collection parameter is stored in the UDM as a subscription information. The user consent for UE data collection parameter may define the user consent for UE level data collection from UE or application or network side. The user consent for UE data collection parameter may also be provisioned by the network operator.

Alternatively, the user consent for UE data collection parameter may be configured in the UDM/UDR as a user subscription information.

Editor's note:	How the user consent is validated is FFS.

5). Nudm_SDM_Get (UE_Id, user consent for UE data collection) - The NWDAF may retrieve the user consent for UE data collection before triggering some UE data specific analytics and data collection.

6). Nudm_SDM_Get (UE_Id, user consent for UE data collection) - The AMF may retrieve the user consent for UE data collection before triggering some UE data specific data collection

Editor’s note: The retrieval of user consent by the AMF as well as collection of data based on such consent is FFS and subject to KI#8 conclusion on methods for data collection from the UE.

Editor’s note:	Whether step 6 is needed or can be covered via regular AMF - UDM communication during UE registration is FFS.

### 6.3.2	Impacts on services, entities and interfaces

UDM:

-	Holds the user consent for UE data collection as a subscription data.

NEF:

-	AF authorisation and rejection if needed.

NWDAF/AMF:

-	Retrieves and checks the user consent for UE data collection before triggering some UE data specific analytics and data collection.

* * * * End of Change * * * *