SA WG2 Meeting #139 E e-meeting	S2-2003879

Elbonia, June 1 - 12, 2020	(revision of S2-2xxxxxx)

Source:	Huawei, HiSilicon

Title:	KI #2, New Sol: Data collection in hierarchical Upper-to-Lower Levels NWDAF interactions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This document presents a solution to support inter-NWDAF instance cooperation from top level NWDAF to lower level NWDAF in hierarchical architecture for data collection coordination.

# 1. Introduction/Discussion

In TR 23.700-91, ‘KI#2: Multiple NWDAF instances’ addresses the followings.

-	‘Study whether there is a need for architectural and functional changes for the support of multiple NWDAF Instances in one PLMN, including hierarchies, roles and inter-NWDAF instance cooperation’.

-	‘Study whether data collection and/or analytics generation can be enhanced in multiple NWDAFs case, e.g. whether NWDAFs can cooperate with each other. If yes, study how to achieve a specific data collection and/or analytics generation target (e.g. for specific target UEs or specific area of interest) with multiple NWDAFs’.

This contribution discuss the highlighted points in KI#2, in particular,

- How to identify the roles and inter-NWDAF instance cooperation in a hierarchical NWDAF architecture.

- How data collection coordination can be enhanced in hierarchical NWDAF architecture.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91.

- * * * * First change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 6.X | KI#2 |  |  |  |

- * * * * Next change * * * *

## 6.X	Solution X: Roles and inter-NWDAF instance cooperation from upper level NWDAF to lower level NWDAF in hierarchical architecture

### 6.X.1	Description

#### 6.X.1.1	General

Multiple NWDAF instances can be cooperated with each other in a hierarchical architecture to support different roles and functionalities. Example of NWDAF role and hierarchy can be identified based on the Serving area such as NWDAF instances are configured with the information of the NWDAF Serving Area. A requirement of the hierarchical NWDAF architecture is inter-NWDAF instance cooperation, in particular, mechanisms for efficient data collection to coordinate between NWDAF instances in different hierarchies.

A Data Collection Coordination (DCC) functionality can be implemented in an NWDAF instance. For example, in the hierarchical architecture, the NWDAF may have DCC functionality. The lower-tier NWDAF instances may send data collection requests or subscriptions to the higher-tier NWDAF that has DCC functionality. The NWDAF having DCC functionality can request or subscribe data from different source NFs on behalf of multiple NWDAF instances and distribute data to multiple NWDAF instances.

Editor's note: The data collection in DCCF refers to the data to be collected from source NFs. Whether DCCF supports aggregated data, which refers to analytics data collected from one-tier NWDAF to be passed to another tier NWDAF is FFS.

#### 6.X.1.2	Procedure

The NWDAF instances in hierarchical architecture are categorized as top level, middle level and lower level. In each level one or more NWDAF instance is co-located with DCCF to coordinate the triggering of data collection in its associated lower level. The procedure of different cases of inter-NWDAF instance cooperation in hierarchical architecture to coordinate data collection is presented from Figure 6.X.1.2-1 to Figure 6.X.1.2-3 below.

##### 6.X.1.2.1	Case 1: Inter-NWDAF instance cooperation in hierarchical architecture

Figure 6.X.1.2.1-1: Case 1: Procedure for inter-NWDAF instance cooperation in hierarchical architecture

1.	NWDAF instances are configured with the role, e.g., the information of the NWDAF Serving Area of its own tier, as well as the NWDAF serving areas in its associated lower tiers.

NOTE 1: How an NWDAF in a given level discovers the NWDAF instances in the upper and lower levels is FFS.

2.  An Analytics consumer requests to the NWDAF in a tier n (e.g, top level) for an Analytics subscription request.

3a. The top level NWDAF is configured with the role of Data Collection Coordination (DCC) functionality. To coordinate the data collection, the top level NWDAF generates the mapping of an identification of a subscription to an Analytics IDs and the data to be collected from data source NFs. The mapping can be generated based on the identification of subscription to an Analytics IDs from step 2 and the list of identification of triggered data to be collected for such Analytics ID generation at other NWDAFs, NFs, AFs, and OAM. Then the top level NWDAF determines, based on the association information, if there an existing subscription to the same analytics ID had already triggered the subscription or request of the same data.

3b. If there an existing subscription to the same analytics ID had already triggered for the data collection, the top level NWDAF sends Analytics output to the Analytics consumer, based on the collected data.

##### 6.X.1.2.2	Case 2: Inter-NWDAF instance cooperation in hierarchical architecture

Figure 6.X.1.2.2-1: Case 2: Procedure for inter-NWDAF instance cooperation in hierarchical architecture

1-3a. Same as steps (1-3a) in Figure 6.X.1.2.1-1.

3b. If the triggered of data collection for the subscription request is not available, the top level NWDAF sends the request for data collection to the NWDAF in the middle level, that has DCC functionality.

4a. The middle level NWDAF creates and check the mapping of an identification of a subscription request and the list of identification of triggered of data to be collected according to step 3a in Figure 6.X.1.2.1-1.

4b. If  an existing subscription to the same request ID had already triggered the the data collection, the middle level NWDAF sends the request data to the top level NWDAF.

4c. The top level NWDAF sends Analytics output  to the Analytics consumer, based on the collected data.

##### 6.X.1.2.3	Case 3: Inter-NWDAF instance cooperation in hierarchical architecture

Figure 6.X.1.2-3: Case 3: Procedure for inter-NWDAF instance cooperation in hierarchical architecture

1-4a. Same as steps (1-4a) in Figure 6.X.1.2.1-2.

4b. If the triggered of data collection for the subscription request is not available, the middle level NWDAF sends the request for data collection to the NWDAF in the lower level, that has DCC functionality.

4c. If the triggered of data collection for the subscription request requires a new data subscription from the data source, e.g., NF, the middle level NWDAF sends the subscription request for collected data from NF.

5a. After receiving the request from step 4b, the lower level NWDAF creates and check the mapping of an identification of a subscription request and the list of identification of triggered of data to be collected according to step 3a in Figure 6.X.1.2.1-1.

5b. If an existing subscription to the same request ID had already triggered the data collection, the lower level NWDAF sends the request data to the middle level NWDAF as per step 4b. In a similar way, the requested data as per step 4c is sent to the middle level NWDAF by the data source, NF.

5c. The collected data is sent to the requested NWDAF, the top level NWDAF.

5d. The top level NWDAF sends Analytics output to the Analytics consumer, based on the collected data from middle level NWDAF.6.X.2

### 6.X.2	Impacts on services, entities and interfaces

NWDAF:

-	Inter-coordination of NWDAF instances in different hierarchies and mechanisms to support DCC functionalities in hierarchical architecture.

- * * * * End of changes * * * *