3GPP TSG-SA # 139E (e-meeting)	S2-2003617r07

# June 1-12, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2312 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Parameters Configuration by Parameter Provisioning and Event Exposure Subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | #1 Triggered by CT4 LS in S2-2003548/ C4-202355, inconsistency/incorrectness in clause 4.15.6.3a are observed: 4.15.6.3a Network Configuration parameters The parameters Maximum Response Time and Maximum Latency are classified in the UDM as AMF-Associated parameters and sent to the AMF. If the UDM is able to associate these parameters with subscribed DNN or S-NSSAI, then the UDM provides the associated DNN or S-NSSAI to the AMF. The AMF may take these parameters into account when a corresponding PDU Session to the indicated DNN or S-NSSAI is established. … If the UDM received multiple Network Configuration request, the UDM shall accept the request as long as the Maximum Latency (if received) and/or the Maximum Response Time (if received) are within the range defined by operator policies. The UDM shall keep the minimum value of Maximum Latency(s) and/or the maximum value of Maximum Response Time(s) as the AMF-Associated parameters. If the configured value is changed comparing to the one last time sent to the AMF, the UDM notify the AMF of the updated value via Nudm_SDM_Notification message. The Suggested Number of Downlink Packets is classified as SMF-Associated parameter. The SMF may use the Suggested Number of Downlink Packets parameter to configure the number of packets to buffer in the SMF/UPF (in the case of UPF anchored PDU sessions) or in the NEF (in the case of NEF anchored PDU session) when the UE is not reachable and extended buffering of downlink data is activated [Observation-1] The 1st para above says that UDM may be able to be associate the AMF-associated parameters with DNN or S-NSSAI and provide them to AMF, which the 2nd para above states the UDM shall consolidate the parameters and provides the min value of Maximum Latency (ML) and Maximum Response Time (MRT) to the AMF. [Observation-2] Per the 2nd para above, ML and MRT are UE level parameters, and there is no need for the AMF to associate with the DNN or S-NSSAI. [Observation-3] In the 2nd para above, “The UDM shall keep the minimum value of Maximum Latency(s)…” is not really correct, because there may be multiple requests to provision the ML or MRT, which implies that the UDM need to maintain/keep those requested values, and when there is a deletion of a request, the UDM needs to re-calculate the min value of ML or max value of MRT. This is also how the network configuration parameters are managed in HSS in EPS. [Observation-4] In the 3th para above, “Suggested Number of Downlink Packets” could be associated with DNN, and in this case, the consolidated value in UDM will be provided to the PDU Session associated with the DNN and S-NSSAI. In order to be able to manage the multiple requests of network configuration parameters in the UDM, Nudm_ParameterProvision_Create and Nudm_ParameterProvision_Delete should be involved. #2 Inconsistency is observed between clause 4.15.3.2.3b and 4.15.6.3a regarding Suggested Number of Downlink Packets, clause 4.15.3.2.3b has “If the AMF receives a suggested number of downlink packets, the AMF provides it to the SMF”, implying that the parameter is sent from UDM to AMF; while in clause 4.15.6.3a has “The Suggested Number of Downlink Packets is classified as SMF-Associated parameter”, implying that the parameter is sent from UDM to the SMF. #3 As the same network configuration parameters (e.g. ML) may be provided by Nudm_EventExposure_Subscribe (as specified in 4.15.3.2.3b) and/or by Nudm_ParameterProvision (clause 4.15.6.3a), clarification is needed how the UDM determines the value that is to be notified to the AMF/SMF including the handling at Nudm_EE_Unsubscribe and Nudm_PP_Delete. #4 Ideally Network Configuration Parameters should be managed ONLY via Nudm_ParameterProvisioning service in 5GC. ., however, there may be legacy AF that supports network parameter configuration only via Event Exposure procedure over T8 and N33 interfaces, and monitoring event supports handling of the parameters of the same UE by different AFs. It’s proposed to add notes for clarification. #5 Step 2 of clause 4.15.3.2.3b states that UDM set the suggested number of downlink packets to the newly received value if higher than the currently applied (see below), however, TS 23.682 specifies that the newly received value is added to the currently applied value (see below). It’s proposed to align the system behaviour between 5GC and EPC. ====Text from 4.15.3.2.3b=== …If the newly received suggested number of downlink packets is higher than the provided suggested number of downlink packet, the UDM shall set the suggested number of downlink packets using the newly received suggested number of downlink packets. == Text from 23.682=== - If Suggested number of downlink packets is newly received, the HSS shall add the newly received value to the currently used value of Suggested number of downlink packets if the aggregated value is within the operator defined range |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.15.3.2.3b, Correct the description related to Suggested Number of Downlink Packets to be consistent with clause 4.15.6.3a (i.e. Suggested Number of Downlink Packets are SMF-associated parameters). Clarify how UDM derives the subscribed periodic registration timer and subscribed Active Time. Correct the behavior how UDM derives the Suggested Number of Downlink Packets. Add a warning saying that Event Exposure to configure network parameters is assumed to be used only when the AF does not support Parameter Provision service operation. Clause 4.15.6.3a: Remove the text indicating that parameters Maximum Latency and Maximum Response Time are associated with DNN and S-NSSAI; Clarify that the SMF-associated parameters Suggested Number of Downlink Packets are provided by UDM to SMF; Clarify how the UDM handles multiple requests of Network Configuration parameters for AMF- and SMF-associated parameters. Clauses 5.2.3.6.3, 5.2.3.6.4: Allow Nudm_ParameterProvision_Create and Nudm_ParameterProvision_Delete to handle network configuration parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification, resulting in not implementable requirement; Potentially inconsistent user experience when UE camping in EPS and in 5GS; Parameter Provisioning for Network Configuration parameters not working as expected. Stage 3 work cannot be completed as they are waiting for SA2 input. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.1, 4.15.3.2.3b, 4.15.6.3a, 5.2.3.6.3, 5.2.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

****************** START CHANGE ***************

#### 4.15.3.1	Monitoring Events

The Monitoring Events feature is intended for monitoring of specific events in 3GPP system and making such monitoring events information reported via the NEF. It is comprised of means that allow NFs in 5GS for configuring the specific events, the event detection, and the event reporting to the requested party.

To support monitoring features in roaming scenarios, a roaming agreement needs to be made between the HPLMN and the VPLMN, I-NEF may be deployed as defined in clause 6.2.5a of TS 23.501 [2]. If I-NEF is deployed, the AMF and SMF in the VPLMN provide the configuration for a given Monitor Event at I-NEF and make monitoring event reported via the I-NEF, the I-NEF is aware of the monitoring event and make it reported via the NEF.

The set of capabilities required for monitoring shall be accessible via NEF to NFs in 5GS. Monitoring Events via the UDM, the AMF, the SMF and the GMLC enables NEF to configure a given Monitor Event at UDM, AMF, SMF or GMLC, and reporting of the event via UDM and/or AMF, SMF or GMLC. Depending on the specific monitoring event or information, it is the AMF, GMLC or the UDM that is aware of the monitoring event or information and makes it reported via the NEF.

The following table enumerates the monitoring events and their detection criteria:

Table 4.15.3.1-1: List of events for monitoring capability

| Event | Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Loss of Connectivity | Network detects that the UE is no longer reachable for either signalling or user plane communication (see NOTE 4). The AF may provide a Maximum Detection Time, which indicates the maximum period of time without any communication with the UE after which the AF is to be informed that the UE is considered to be unreachable (see NOTE 7). | AMF |
| UE reachability | Detected when the UE transitions to CM-CONNECTED state or when the UE will become reachable for paging, e.g., Periodic Registration Update timer. It indicates when the UE becomes reachable for sending downlink data to the UE. The AF may provide the following parameters: 1) Maximum Latency; 2) Maximum Response Time; 3) Suggested number of downlink packets. (see NOTE 5 and NOTE 7). | AMF, UDM |
| Location Reporting | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (one-time reporting, maximum number of reports, maximum duration of reporting, periodicity, etc., as specified in clause 4.15.1). It indicates either the Current Location or the Last Known Location of a UE. When AMF is the detecting NF: One-time and Continuous Location Reporting are supported for the Current Location. For Continuous Location Reporting the serving node(s) sends a notification every time it becomes aware of a location change, with the granularity depending on the accepted accuracy of location. (see NOTE 1) For Last Known Location only One-time Reporting is supported When GMLC is the detecting NF: Immediate and Deferred Location Reporting is supported. For Deferred Location Reporting the event types UE availability, Area, Periodic Location and Motion are supported. | AMF, GMLC |
| Change of SUPI-PEI association | This event is detected when the association between PEI and subscription (SUPI) changes (USIM change). | UDM |
| Roaming status | This event is detected when the UE's current roaming status (the serving PLMN and/or whether the UE is in its HPLMN) and notification when that status changes. (see NOTE 2). | UDM |
| Communication failure | This event is detected when RAN or NAS level failure is detected based on connection release and it identifies RAN/NAS release code. | AMF |
| Availability after Downlink Data Notification failure | This event is detected when the UE becomes reachable again after downlink data delivery failure. | AMF |
| PDU Session Status | This event is detected when PDU session is established or released. (see NOTE 6) | SMF |
| Number of UEs present in a geographical area | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (Level of aggregation, Sampling ratio, see clause 4.15.1). It indicates the number of UEs that are in the geographic area described by the AF. The AF may ask for the UEs that the system knows by its normal operation to be within the area (Last Known Location) or the AF may request the system to also actively look for the UEs within the area (Current Location). | AMF |
| CN Type change | The event is detected when the UE moves between EPC and 5GC. It indicates the current CN type for a UE or a group of UEs when detecting that the UE switches between being served by a MME and an AMF or when accepting the event subscription. (see NOTE 3) | UDM |
| Downlink data delivery status | It indicates the downlink data delivery status in the core network. Events are reported at the first occurrence of packets being buffered, transmitted or discarded, including: - Downlink data in extended buffering, including: - First data packet buffered event - Estimated buffering time, as per clause 4.2.3.3 - First downlink data transmitted event - First downlink data discarded event | SMF |
| UE reachability for SMS delivery | This event is detected when an SMSF is registered for a UE. This enables the UE to receive an SMS. HSS can subscribe to notifications about SMSF registration events in UDM for a given UE as defined in TS 23.632 [68]. | UDM: reachability for SMS |
| User State Information in 5GS | Provides user state information in 5GS. | AMF |
| NOTE 1: Location granularity for event request, or event report, or both could be at cell level (Cell ID) or TA level. The granularity can also be expressed by other formats such as geodetic uncertainty shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.) which can be mapped by NEF to AMF specific granularity levels. NOTE 2: Roaming status means whether the UE is in HPLMN or VPLMN. NOTE 3: CN type of CN Type change event is defined in clause 5.17.5.1 of TS 23.501 [2]. NOTE 4: In the case of UDM service operation information flow, the UDM should set the subscribed periodic registration timer to a smaller value than the value of Maximum Detection Time, since the value of the mobile reachable timer is larger than the value of the periodic registration timer. NOTE 5: Maximum Latency, Maximum Response Time and Suggested number of downlink packets are defined in clause 4.15.6.3a. NOTE 6: The NEF makes a mapping between the 5GS internal event "PDU Session Status" and the T8 API event "PDN Connectivity Status". NOTE 7: The preferred method for provisioning Network Configuration Parameters is External Parameter Provisioning specified in clause 4.15.6.3a and it supersedes the provisioning of event specific parameters as part of Monitoring Request which should be used only by AF that does not support Parameter Provisioning procedure specified in clause 4.15.6.3a. |  |  |

****************** NEXT CHANGES ***************

##### 4.15.3.2.3b	Specific NEF service operations information flow for loss of connectivity and UE reachability

The procedure is used by the AF to subscribe to notifications and to explicitly cancel a previous subscription for loss of connectivity and UE reachability.

Figure 4.15.3.2.3b-1: Nnef_EventExposure_Subscribe, Unsubscribe and Notify operations or loss of connectivity and UE reachability

1.	Step 1 to step 3b of Figure 4.15.3.2.3-1 are performed with the following differences:

-	For Loss of Connectivity, the subscription request may include Maximum Detection Time (see Table 4.15.3.1-1).

-	For UE reachability, the subscription request may include Maximum Latency, Maximum Response Time and/or Suggested number of downlink packets (see Table 4.15.3.1-1).

NOTE 1: It is expected that Maximum Latency, Maximum Response Time and/or Suggested number of downlink packets included in the subscription request is only used by the AF that does not support Parameter Provisioning procedure specified in clause 4.15.6.3a.

2.	[Conditional] If the subscribed periodic registration timer has not been set according to any subscription request, or a Network Configuration as defined in clause 4.15.6.3a the UDM shall set the subscribed periodic registration timer using the Maximum Detection Time or Maximum Latency; otherwise if the subscribed periodic registration timer was previously set by a different subscription identified by a different Notification Target Address (+ Notification Correlation ID), or set by a different Network Configuration identified by a different NEF reference ID for the same UE, and if the newly received Maximum Detection Time or Maximum Latency is lower than the provided subscribed periodic registration timer, the UDM shall set the subscribed periodic registration timer using the newly received Maximum Detection Time or Maximum Latency.

If Nudm_EventExposure_Unsubscribe request is performed in step 1, the UDM shall recalculate the subscribed periodic registration timer based on the remaining event subscriptions and/or Network Configurations.

In addition, for UE reachability subscription, if the newly received Maximum Response Time is longer than the provided subscribed Active Time (i.e. previously provided Maximum Response Time), the UDM shall set the subscribed Active Time using the newly received Maximum Response Time. If the suggested number of downlink packets is newly received, the UDM shall add the newly received suggested number of downlink packets to the currently used value of suggested number of downlink packets if the aggregated value is within the operator defined range.

If Nudm_EventExposure_Unsubscribe request is performed in step 1, the UDM shall recalculate the subscribed Active Time and/or Suggested Number of Downlink Packets based on the remaining event subscriptions and/or Network Configurations.

If the subscribed periodic registration timer or the subscribed Active Time are set or modified, the UDM sends the Nudm_SDM_Notification request to related serving AMF(s). If the AMF receives a subscribed periodic registration timer value from the UDM, it allocates the received value to the UE as the periodic registration timer at subsequent Registration procedure. The AMF starts monitoring of the expiration of the mobile reachable timer for Loss of Connectivity (if required) and starts monitoring of the UE entering connected mode for UE reachability (if required).

If the suggested number of downlink packets are set or modified, the UDM sends the Nudm_SDM_Notification request to related serving SMF(s). The SMF configures the data buffer at the SMF/UPF according the suggested number of downlink packets.

If the provided value is updated by the UDM, the UDM may notify the NEF (which then notifies the AF) of the actual value that is being applied in the 3GPP network.

3.	Step 4 to step 5 of Figure 4.15.3.2.3-1 are performed.

4.	Step 6c to step 6d of Figure 4.15.3.2.3-1 are performed with the following differences:

-	For Loss of Connectivity, the event is detected when the mobile reachability timer expires.

-	For UE reachability, the event is detected when the UE changes to connected mode or when the UE will become reachable for paging.

5.	Step 8 of Figure 4.15.3.2.3-1 is performed.

****************** NEXT CHANGES ***************

#### 4.15.6.3a	Network Configuration parameters

The Network Configuration parameters are the parameters sent from an AF by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2.

The Network Configuration parameters are described in Table 4.15.6.3a-1.

Table 4.15.6.3a-1: Description of Network Configuration parameters

| Network Configuration parameter | Description |
| --- | --- |
| Maximum Response Time | Identifies the time for which the UE stays reachable to allow the AF to reliably deliver the required downlink data. [optional] |
| Maximum Latency | Identifies maximum delay acceptable for downlink data transfers. Example: in order of 1 minute to multiple hours. [optional] |
| Suggested Number of Downlink Packets | Identifies the number of packets that the core network is suggested to buffer if the UE is not reachable. Example: 5 packets. [optional] |

The parameters Maximum Response Time and Maximum Latency are classified in the UDM as AMF-Associated parameters and sent to the AMF.

The UDM may use the Maximum Latency parameter to configure the time between UE reachability events (e.g. MICO mode duration, Periodic Registration Timer value).

The AMF may use the Maximum Response Time parameter as guide to configure:

-	Active Time or Extended Connected time for MICO mode;

-	when to send reachability notifications to AF relative to expected reachability events (e.g. paging occasions).

If the UDM received multiple Network Configuration requests, the UDM shall accept the request as long as the Maximum Latency (if received) and/or the Maximum Response Time (if received) are within the range defined by operator policies. The UDM shall use the minimum value of Maximum Latency(s) and/or the maximum value of Maximum Response Time(s) as the AMF-Associated parameters. If the configured value is changed comparing to the one last time sent to the AMF, the UDM notify the AMF of the updated value via Nudm_SDM_Notification message. If there is a deletion of Network Configuration request, the UDM re-calculates the values (see step 2 in clause 4.15.3.2.3b) and notify the AMF if needed.

The Suggested Number of Downlink Packets is classified as SMF-Associated parameter. If the NEF is providing DNN and S-NSSAI as specified in clause 4.15.3.2.3, then the UDM is able to associate the parameters with subscribed DNN and S-NSSAI, and provides the SMF-associated parameters to the SMF for the PDU Session associated with the specific DNN and S-NSSAI as specified in clause 4.15.6.2. The SMF may use the Suggested Number of Downlink Packets parameter to configure the number of packets to buffer in the SMF/UPF (in the case of UPF anchored PDU sessions) or in the NEF (in the case of NEF anchored PDU session) when the UE is not reachable and extended buffering of downlink data is activated.

A Validity Time may be associated with any of the Network Configuration parameters. When the validity time expires, the related NFs delete their local copy of the associated Network Configuration parameter(s).

****************** NEXT CHANGES ***************

##### 5.2.3.6.3	Nudm_ParameterProvision_Create service operation

Service operation name: Nudm_ParameterProvision_Create

Description: The consumer creates a Network Configuration with one or more parameters, or a 5G VN group related information (e.g. 5G VN group data, 5G VN membership management).

Inputs (required): AF ID, Transaction Reference ID(s).

Inputs (optional): GPSI, External Group ID, one or multiple Network Configuration parameters, or for 5G VN group creation: External Group ID and 5G VN group related information.

Outputs (required): Transaction Reference ID(s), Operation execution result indication.

Outputs (optional): Transaction specific parameters, if available; Internal Group ID if the inputs include a new 5G VN configuration.

****************** NEXT CHANGES ***************

##### 5.2.3.6.4	Nudm_ParameterProvision_Delete service operation

Service operation name: Nudm_ParameterProvision_Delete

Description: The consumer deletes one or more previously created Network Configuration parameters, or a 5G VN group.

Inputs (required): AF ID, Transaction Reference ID(s).

Inputs (optional): GPSI, External Group ID, for 5G VN group deletion or Network Configuration of Parameters.

Outputs (required): Transaction Reference ID(s), Operation execution result indication.

Outputs (optional): None.

****************** END CHANGES ***************