3GPP TSG-WG SA2 Meeting #139E e-meeting  	S2-2003673r01

# Elbonia, June 1 - 12, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.737 | CR |  | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI #1 conclusions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales, Nokia, Nokia Shanghai Bell, TNO, Hughes Network Systems Ltd., ESA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-06-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | KI #1 conclusions are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Conclusion for KI #1 is proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If no solution can be selected, then the normative work cannot progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st Change ***

## 8.1	Conclusion on solutions for Key Issue #1

Solution #1 does not identify any modification of the existing 3GPP specifications for the 5G-CN. Depending on its implementation it could require modifications for NG-RAN for UE location.

NOTE 1:	5GCN impacts may be identified later for Solution #1 during the WI phase due to incomplete specification of all aspects (e.g. concerning fixed cell relationship to radio cells) and the possibility that NG-RAN cannot absorb all impacts to support mobility management.

Solution #12 needs further evaluation and may need further work in SA2 but has so far been found to fulfil KI#1 and KI#10, but with a different set of properties and impacts to the 5GS, UE, RAN and 5G-CN. So far, no show stoppers have been found from SA2 related aspects. As RAN dependencies have been identified in both solution #1 and solution #12,  RAN working groups have been consulted to evaluate these solutions.

RAN working groups indicate in their LS that they have not considered the principles of Sol #12 in their study TR 38.821 and that no further NTN study is expected right now. This means that there is no guarantees of RAN support for Sol #12. The same applies to Sol #1 since RAN support of Sol #1 assumes that “beam size is larger than the maximum cell size supported by NR” and RAN3 notes that “RAN3 did not discuss some aspects of Solution 1”.

Furthermore RAN WGs indicated that a scheme to locate the UE will be defined during the NR-NTN normative work

Solution#1 is recommended as a general way forward for normative work but can be complimented by portions of Solution #12 if these can address limitations of Solution #1 and/or enable fewer impacts to 5GCN, NG-RAN and/or UEs.

NOTE:	The same conclusion is agreed for Key Issue #6.