SA WG2 Meeting #139E	S2-2003732r05

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple, NEC

Title:	KI #7, New Sol: Including supported operating frequency bands in Allowed NSSAI

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes a solution for 5GC assisted cell selection to access network slice. The solution is based on the assumption that UE may simultaneously register on slices that are not all accessible on the same operating frequency band and the UE may simultaneously establish PDU sessions on slices that are not all supported on the same operating frequency band.

# 1. Introduction

As highlighted in this key issue, an operator may only support certain network slices on certain operating frequency bands. One registration area may consist of multiple NG-RAN cells belonging to different operating bands and thus it is possible that support for a particular S-NSSAI in a registration area is non-homogenous.

In this paper, we propose to provide the UE assistance in Configuration Update/Registration procedure so the UE is aware about which slice is supported on which operating bands or frequency ranges. UE may use this information to reselect to a new operating band which supports the S-NSSAI user is interested in accessing and initiate radio connection with NG-RAN.

Alternatively, NG-RAN may steer the UE to appropriate frequency band which supports the S-NSSAI the UE includes in the RRC Connection Setup procedure.

* * * * Start of Changes * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| x | Including supported operating frequency bands in Allowed NSSAI | 7 |

* * * * Next Change (all new text) * * * *

## 6.X	Solution #x: Including supported operating frequency bands in Allowed NSSAI

### 6.X.1	Introduction

This solution is for Key Issue #7, "Support of 5GC assisted cell selection to access network slice". The solution is based on the following architectural assumptions:

-	The UE may simultaneously register to slices that are not all accessible on the same operating band(s). In other words, the UE may have S-NSSAIs in the Allowed NSSAI that are not all available in a common operating band.

Editor's note:	it is FFS whether it is acceptable to change the current design principle by which the Allowed NSSAI contains all the S-NSSAI the UE can use concurrently. It is also FFS to see if the scenario by which an operator does not support services on common frequency bands, that are supposed to be run concurrently.

-	The UE may simultaneously establish and access PDU sessions on slices that are not all supported on the same operating band.

Editor's note:	it is FFS how the UE can concurrently transmit and receive data over PDU Sessions not all available on the same frequency bands.

### 6.X.2	High Level Description

As part of the Registration procedure, UE shall be provided with the list of supported operating frequency bands for each allowed NSSAI (NG-RAN is aware of the operating bands for each S-NSSAI based on the slice creation). In the case where the operator slice deployment is governed by frequency ranges (e.g. FR1 and FR2), the AMF may indicate that an S-NSSAI is supported on [FR1 only], [FR2 only] or [FR1+FR2] instead of explicitly listing out all the operating frequency bands where each S-NSSAI is supported.

The UE may receive S-NSSAIs in the Allowed NSSAI that are not all supported on the same operating frequency band but that are all supported in the TAIs in the Registration Area.

Based on the slice the user is interested in accessing, UE may either continue to operate on the current operating band or be steered to operate on a new operating band by NG-RAN.

Editor’s Note: How the NG-RAN steers the UE (e.g. handover, configuring dual connectivity to simultaneously access two operating frequency bands, etc.) will be defined in RAN WGs.

### 6.X.3	Procedures

1. 	As part of the Registration Request, UE shall include all the NSSAIs it is interested in as part of "Requested NSSAI".

2.	In addition to the procedures in TS 23.501 [2], clause 5.15.5.2.1, for each allowed S-NSSAI, NSSF shall also determine the supported operating frequency bands, and pass this along with the Allowed NSSAI to the AMF.

3.	The Registration Accept message and UE Configuration Update Command from the AMF to the UE in TS 23.502 [6] are updated to include permissible operating frequency band(s) of each S-NSSAI in the Allowed NSSAI. Alternatively, AMF may indicate whether each S-NSSAI in the Allowed NSSAI is supported on: [FR1 only], [FR2 only] or [FR1+FR2].

4.	As part of the Allowed NSSAI list in Registration Accept message, AMF may include NSSAIs which are not all supported on the same operating band.

5.	UE may require to initiate PDU session establishment procedure on the network slice user is interested in accessing. If the S-NSSAI associated with the PDU session establishment procedure is not supported on the current operating frequency band, then:

5a. A UE in IDLE mode may choose to reselect to a new operating frequency band which supports the S-NSSAI the user is interested in accessing, and initiate radio connection with NG-RAN; or

Editor’s Note: It is FFS how to handle uplink and downlink traffic for the ongoing PDU sessions if UE reselects a different frequency band.

Editor’s note: This step is dependent on RAN WGs' decision.

Editor’s note: how the UE can be steered to a new operating band in connected mode is FFS.

5b. A UE in IDLE mode may initiate radio connection on the current operating frequency band and include the S-NSSAI in the RRC Connection establishment procedure with NG-RAN. Based on the S-NSSAI included in the RRC connection procedure, NG-RAN may steer the UE to access the new operating frequency band which supports the S-NSSAI user is interested in accessing.

Editor’s Note: How the NG-RAN steers the UE (e.g. handover, configuring connectivity to simultaneously access two operating frequency bands, etc.) should be defined in RAN WGs.

Editor’s Note: it is FFS whether the UE actually reselects to a new operating frequency band autonomously or is steered to the appropriate band by the NG-RAN.

6.	After the release of the PDU session on the new operating frequency band and when the UE is in IDLE mode, the UE may re-select back to a cell from the initial operating frequency band (e.g. based on UE configuration or preference).

Editor’s Note: it is FFS whether this is applicable only to idle mode, or also connected mode and how.

Editor’s Note: The handling of any ongoing PDU sessions for other S-NSSAIs is FFS.

### 6.X.4	Impacts on existing entities and interfaces

-	NSSF shall be able to provide the supported operating frequency band(s) for each allowed S-NSSAI(s) to the AMF in the current TA and the surrounding TAs candidate for being in the RA.

- 	AMF shall be able to provide supported operating frequency band(s) in the RA for each allowed S-NSSAI(s) to the UE as part of Configuration Update/Registration procedure.

- 	AMF shall be able to provide supported operating frequency band(s) for each allowed S-NSSAI(s) to NG-RAN as part of N2 message.

- 	NG-RAN shall be able to steer the UE to appropriate operating frequency band which supports the S-NSSAI included in the RRC connection setup procedure.

-	UE shall be able to include the S-NSSAI in the RRC Connection establishment procedure with NG-RAN.

-	UE shall be able to re-select to appropriate operating frequency band which supports the S-NSSAI the UE is interested to access.

Editor’s Note: it is FFS whether this is applicable only to idle mode, or it is requested also when the UE is in connected mode.

Editor's note:	it is FFS how the UE can continue to concurrently transmit and receive data over PDU Sessions not all available on the same frequency bands.

Editor's note: it is FFS how pre-Rel-17 UEs would be able to handle the changes.

### 6.X.5 	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *