SA WG2 Meeting #139E (e-meeting)	S2-2003835r01

June 01 - 12, 2020, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics, LG Uplus

Title:	KI #2, New Sol: Simultaneous communication using N3IWF

Document for:	Approval

Agenda Item:	8.2 Study on enhanced support of Non-Public Networks

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes a new solution to Key Issue #2.

Background

During the eNPN conference call, there were some discussion on the scope of Key Issue #2 and after that there were e-mail discussion to clarify the Key Issue #2 scope. During the discussion following example scenario had been clarified to show the intention of the Key Issue #2.

Figure 1 illustrates the use case that motivated the requirement of “Multi-network connectivity and service delivery” in TS 22.263, which has been translated in KI#2 as “Study means to enable a UE to receive data services from one network (e.g. NPN), and paging as well as data services from another network (e.g. PLMN) simultaneously.” Figure 1: Use case in TR 22.827 - Live Production with integrated audience services The original requirement asks for simultaneous connectivity to a PLMN and an NPN to allow UEs to simultaneously receive data services from both networks. For clarification of the scope in KI#2, following assumptions in the original VIAPA requirement are listed: - Audience can have access to dedicated AV services during an event using their UEs. The UEs are consumer devices (attendants bring their own devices) with subscription to a PLMN. - The UEs access the audience services through an NPN located at the event location and operated by a 3rd party. - No assumptions or restrictions on UE implementation regarding single radio/dual radio. - The two networks (PLMN and NPN) can be operated by different operators. - The NPN may be a SNPN including its own NG-RAN and 5GC. The NPN aims at providing local production and DL audience data services. - The DL audience services require high 5QI (e.g. audio/video data streaming with e2e required latency of 7ms and 30 multicast streams with 200kbit/s. There can be a very large number of served UEs (up to 50000) concentrated in the service area of the NPN (Note: see TS22.263, section 6.3.1 KPI requirement for low latency deterministic periodic traffic with multicast service) - A UE shall be reachable by its subscribed PLMN at any time, independently of the fact that the UE is simultaneous connected to the NPN. - UE can be simultaneously attached to both the SNPN and a PLMN while keeping an active data session with both networks at the same time. - Depending on the capabilities and configurations of the UE, limitations of data-rate and latency may be expected.

Based on above scenario, we assume following:

-	A UE has a single RX and a single TX, i.e. activate communication is only possible with one network.

-	The VIAPA services require high level QoS.

-	The services via SNPN is only provided when a UE is in the SNPN coverage.

Proposal

It is proposed to capture the following solution to TR 23.700-07.

* * * * Start of Change (All New Text) * * * *

6.X	Solution #<X>: Support of simultaneous active communication with SNPN and PLMN by using N3IWF

6.X.1	Introduction

The solution assumes a dual-subscription device, which contains an SNPN UE and a PLMN UE. From the network point view, the device is a normal UE.

This solution addresses key issue #2: NPN support for VIAPA based on following assumptions.

-	A device has a single RX and a single TX, i.e. activate communication is only possible with one network.

-	The VIAPA services require high level QoS.

-	The services via SNPN is only provided when a device is in the SNPN coverage.

6.X.2	Functional Description

The solution uses architecture shown in Figure 6.x.2-1.

Figure 6.X.2-1: SNPN access via N3IWF using a PLMN PDU Session

Because it is assumed that a device has a single RX and a single TX, the device can get a service either over a PLMN or SNPN at a given time. In order to get a service simultaneously from PLMN and SNPN, the device registers SNPN via N3IWF using a PDU Session of PLMN and establishes a PDU Session over non-3GPP access of the SNPN to get a service of SNPN. Then the device can get a service not only from a PLMN but also from an SNPN.

The issue is how to ensure QoS of the SNPN (e.g. VIAPA) service. In order to guarantee the QoS requirement of the SNPN service, there should be an SLA between PLMN and SNPN. However, the device does not know whether there is an SLA or not. So the SNPN need to provide list of PLMNs that the SNPN has an SLA during the Registration procedure. The device should access SNPN via N3IWF only when the HPLMN of the device is included in the SNPN provided list.

Another issue is how to limit access to SNPN service when a device is not in the coverage of SNPN service. For example, an SNPN provide a service in a specific area e.g. in a concert hall, the service should be provided only when a device is in the service area of the SNPN. However, when a device registers SNPN via N3IWF, the SNPN does not know the device location. So the SNPN need to get a device location from the PLMN. In order to get a location of the device, the SNPN need to know device identity (e.g. GPSI). This can be provided by the device during the Registration procedure to SNPN.

6.X.3	Procedures

Figure 6.X.3-1: Procedure for simultaneous communication with SNPN and PLMN by using N3IWF

1.	A device registers to a PLMN via 3GPP access and get a service from the PLMN.

2.	The device sends Registration Request via 3GPP access. In the Registration Request message, the device indicates that it wants to use PLMN service simultaneously.

3.	The SNPN AMF provides Supported PLMN List which includes the list of PLMNs that the SNPN has an SLA so that QoS can be supported when the device access SNPN via N3IWF using a PDU Session of the PLMN.

4.	If the device is registered to the PLMN included in the Supported PLMN List, the device registers to SNPN via SNPN N3IWF over the PDU Session of the PLMN. The device provides its GPSI in the Registration Request message.

If the device is registered to the PLMN not included in the Supported PLMN List, the device may register to PLMN via PLMN N3IWF over the PDU Session of the SNPN. The device handovers 3GPP access PDU Sessions to non-3GPP access and receives service.

NOTE:	If the handover is not allowed due to policies or if the PLMN does not N3IWF, the device cannot get a service from SNPN and PLMN simultaneous. In this case, the device may need to prioritize service and select either SNPN or PLMN service and how to do it is up to device implementation.

5.	The AMF sends Registration Accept message to the device.

6.	The device sends PDU Session Establishment Request to the SMF.

7.	The SMF sends PDU Session Establishment Accept to the device.

8.	The SMF subscribes mobility event to monitor whether the device is in the service area, i.e. subscribes Area of Interest reporting with Area of Interest, which follows existing procedure.

9.	The AMF sends acknowledgement to the SMF.

10-17.	The AMF subscribes mobility event via SNPN NEF. The AMF provides GPSI provided by the device in step 4. The NEF maps the Area of Interest to target location information (e.g. civic addresses, TA list) and provides it together with GPSI to subscribe mobility event. The AF finds PLMN NEF by using GPSI and subscribes mobility event. The PLMN NEF maps the location information to AMF specific granularity levels (e.g. TA list, Cell list) and subscribes mobility event to the PLMN AMF.

18.	If the device moves out of target location area, the PLMN AMF notifies it to the PLMN NEF and eventually the notification is sent to the SNPN SMF.

19-20.	Based on event notification in step 18, the SMF may release the PDU Session.

Editor's note: Whether mobility event subscription for device tracking to ensure the SNPN service is only allowed when the device is in the SNPN coverage is in the scope of the study is FFS.

6.X.4	Impacts on services, entities and interfaces

UE:

-	provides indication that it wants to use PLMN service simultaneously when it registers SNPN over 3GPP access.

-	decides whether to access SNPN service via SNPN N3IWF using PLMN PDU Session based on Supported PLMN List during the SNPN registration.

-	provides GPSI when it registers SNPN via N3IWF.

SNPN AMF:

-	provides Supported PLMN List to the UE during the Registration.

-	subscribes Area of Interest mobility event to the NEF.

SNPN AF/NEF:

-	supports Area of Interest mobility event subscription and notification using GPSI.

NOTE:	There is no impact to the PLMN AMF and PLMN NEF.

* * * * End of Change * * * *