SA WG2 Meeting #139E	S2-2004088r03

1 June – 12 June 2020

Source:	Samsung

Title:	KI #3A, New Sol: Supporting deterministic communication

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: This contribution proposes a new solution for supporting deterministic communication.

# Discussion

The objective of The Key Issue #3A: Exposure of deterministic QoS is to support any AFs which have deterministic application requirements regardless of TSN bridge support.

Any AF that has knowledge of deterministic application requirements should be able to request TSC services from the 5GS and as authorized, be notified of pertinent network events. This key issue is intended to support in the 5GS, requirements from TS 22.104 [4] where a TSN bridged network may not be needed and requirements from TS 22.263 [5] for Video, Imaging and Audio for Professional Applications (VIAPA). Applications provide those requirements to 5GS for any type of PDU Session.

An AF only represents an application, but a UE could operate multiple applications simultaneously. Note that the application deployment is very complex scenario. To take benefits of TSC supporting features, it is needed a mechanism to support 1) some QoS parameters are not provided by an AF, 2) multiple applications are operating simultaneously for a specific service.

This contribution proposes a solution to support deterministic communication for applications described above. The main idea is to utilize the use of network analytics to obtain/correct the QoS characteristics for an application.

Proposal 1. A new solution to support deterministic communication using network analytics by PCF. NWDAF provides Observed Service Experience analytics to PCF to help deciding QoS parameters and needed information for deterministic communication..

# 2.	Proposal

It is proposed to include the added text into the TR 23.700-20 as a solution for key issue #3A.

* * * * First Change  * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | #1 Uplink Time Synchronization | #2 UE-UE TSC communication | #3A Exposure of TSC services: Exposure of deterministic QoS | #3B Exposure of TSC services Exposure of Time Synchronization | #4 supporting the fully distributed configuration model for TSN |
| #1 | X |  |  |  |  |
| #2 |  | X |  |  |  |
| #3 |  | X |  |  |  |
| #4 |  | X |  |  |  |
| #5 |  |  | X |  |  |
| #6 |  |  | X |  |  |
| #7 |  |  |  | X |  |
| #8 |  |  |  | X |  |
| #X |  |  | X |  |  |

* * * * Second Change (all new text) * * * *

## 6.X	Solution X: Supporting Deterministic Communication

### 6.X.1	Introduction

This solution is for key issue #3A: Exposure of deterministic QoS.

The scope of this solution is to support deterministic communication to the UEs operating applications. The main source of QoS related information for applications is AF, however, expecting that every application provider maintains AFs may not be realistic assumption. Moreover, an AF only represents an application, and a UE could operate multiple applications simultaneously.. To take benefits of TSC supporting features, Rel-16 allows the AF to provide a QoS reference that is mapped by the PCF into QoS parameters

The fundamental mechanism of this solution is to utilize network analytics from NWDAF. NWDAF can collect network data, and able to provide the analytics about Observed Service Experience as defined in 23.288 collecting and analysing the network traffic (or packets), 5G network function (e.g. PCF) PCF can decide if QoS parameters fulfil the QoS requirements

This solution proposes that a PCF, that responsible for deciding a PCC rule of deterministic communication, consumes network analytics on Observed Service Experience the characteristics of network from NWDAF.

### 6.X.2	Functional Description

An AF provides QoS requirements, TSCAI related information, and survival time to the PCF, and PCF decide QoS parameters for serving QoS flow of the application. The below is the principles of the solution.

- An AF provides QoS related parameters to PCF for the deterministic communication such as GPSI, application ID, maximum/minimum delay, maximum burst data volume, periodic indicator, periodic time, and survival time with traffic description. The all parameters are not mandatory to be provided. For untrusted AF case, NEF can be uses to deliver the parameters to PCF.

- The PCF checks the received parameters maps then to a QoS value, TSCAI, and survival time used in 5GS. If, the PCF may requests to NWDAF to monitor the Observed Service Experience

Editor’s Note: It is FFS whether this is need and how for PCF to distinguish the AF request that needs PCF to derive the TSC related parameters and AF request targeting to a non TSC QoS flow. The details of the derivation of TSC Assistance Container, TSN QoS information are FFS. It is FFS whether there is a need and how to apply the TT functionalities at DS-TT or NW-TT in case the PCF derives the TSC related parameters.

- The NWDAF need to collect network data for analyse the characteristics of target UE or application from SMF or other information sources. After collecting the network data, NWDAF can generate the analytics for the traffic characteristics as an output analytics.

### 6.X.3	Procedures

For the procedure to utilize network analytics for the PCF to check if the Service Experience is fulfilledw, the procedure is depicted in figure 6.X.3-1.

Step 1. An AF provides QoS related information for a target UE or application. The deterministic communication can be targeted using the combination of GPSI, application ID, S-NSSAI, and DNN, and traffic descriptor. The QoS related information could contain burst arrival time, burst data volume, periodicity indicator, period time, and survival time.

Step 2. An UE request PDU session for a specific S-NSSAI and DNN.

Step 3. An SMF select a PCF could serve the requested S-NSSAI and DNN, and make a SM policy association.

Step 4. The PCF check that the AF provided parameters map them to  QoS parameter, TSCAI Burst Arrival Time, Periodicity, and survival time., the PCF request to monitor the Observed Service Experience to NWDAF

NOTE: 	It is FFS how NWDAF collect and generate output analytics for QoS related parameters traffic characteristics. for deterministic communication.

Step 5. The PCF derives QoS parameters, TSCAI BAT, Periodicity, and survival time if needed.

Step 6. The PCF deliver the derived information to SMF using PCC rule.

Step 7. The SMF derives TSCAI and establish/modify N4 session according to the PCC rule

Step 8. The SMF provides N2 messages to the (R)AN with TSCAI and survival time. If needed, PDU session establishment accept message can be delivered to the UE.

### 6.3.2	Impacts on Existing Nodes and Functionality

Editor’s Note: This section is TBD.

NWDAF:

- Need to collect network data to analyse traffic characteristic and QoS related parameters for a specific UE or application.

- Need to provide the generated analytics to PCF

* * * * End of change * * * *