SA WG2 Meeting #139E	S2-2004185r11

Electronic meeting, 1-12 June 2020	(Revision of S2-200xxxx)

Source:	Nokia, Nokia Shanghai Bell, Huawei, HiSilicon

Title:	KI #1, Sol #3: Update to address editor's notes

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution:

This paper discusses signalling between the SMF and the RAN, N3 tunnelling, which are some of the open issues of Solution #3 captured in editor’s notes for this solution. Proposals to update Solution #3 are made based on the conclusions including PCR to 23.757 to capture these proposals.

1	Discussion

The transfer of session related message between an SMF and a RAN and eventually also between a SMF and UE has been left for further study in solution #3. This paper elaborates on the signalling issue and provides updates to Solution #3.

The purpose of the signalling between the SMF to the RAN during the PDU session modification procedure for multicast is to a) notify the RAN about the relation between a multicast context and UE’s PDU session on which the UE joins a multicast group corresponding to the multicast context; and b) to establish user plane resources for the transport of multicast PDUs. It should be noted that the relation is indeed between UE’s PDU session and a multicast context and not between a UE and a PDU context because, in case of multiple PDU sessions, it is necessary to distinguish the PDU session on which the UE joins a multicast group, e.g. sends an IGMP Join.

The issue of point-to-point and point-to-multipoint is also discussed in this document as it is closely related to the user plane resource establishment procedures.

1.1	Signalling of the relation between a multicast context and UE

The signalling of the relation between a multicast context and UE’s PDU session on which the UE joins a multicast group corresponding to the multicast context should be performed by the SMF using a UE specific message transfer and a UE context related procedure on the N2 reference point. Using a non-UE specific signalling for the provisioning of the relations between UE’s PDU sessions and multicast contexts is seen as not optimal and should not be further considered based on the following reasons. The SMF would be required to know UE’s serving RAN node to send the signalling via the AMF. As the SMF may not have UE’s location available, this could result in frequent location request from the SMF to the AMF (e.g. Namf_ProvideLocationInfo). Moreover, the non-UE specific messages should anyway include some UE identity and possibly PDU session identities.

Proposal: UE specific signalling should be used by the SMF to signal the relation between a multicast context and UE’s PDU session to the RAN.

Enhanced PDU session modification procedure

In this option, the PDU session update procedure is enhanced to include optionally the information about the relation of UE’s PDU session to a multicast context. The SMF invokes the Namf_N1N2MessageTransfer. On N2 reference point, PDU Session Resource Modify procedure is used with the corresponding messages (PDU SESSION RESOURCE MODIFY REQUEST and PDU SESSION RESOURCE MODIFY RESPONSE).

New procedure dedicated for a multicast context and UE’s PDU session relation signalling

This option is in principle the same as option 1 above with the difference that a new procedure is introduced on N2 reference point dedicated for the signalling regarding multicast contexts through which the relations between UE’s PDU sessions and multicast context are added/modified/removed.

1.2	Signalling for user plane establishment

Point-to-point and point-to-multipoint tunnelling on N3/N9 have both their merit. Point-to-point tunnelling provides more freedom in regard to the deployment of UPF(s) and RAN (e.g. CU-UP) because the infrastructure interconnecting these elements of 5G system does not need to support multicast routing. On the other hand, point-to-multipoint tunnelling on N3/N9 can be more resource efficient if the same content is delivered to a larger number of RAN nodes.  Noting that the user plane configuration is slightly different depending on whether N3/N9 uses point-to-point (unicast) or point-to-multipoint (multicast) tunnelling, it seems anyway beneficial at the cost of additional standardization and implementation efforts to support both point-to-point and point-to-multipoint tunnelling on N3/N9 by the specification which allows to meet requirements of different deployments. For information, 3G MBMS allows both unicast and multicast tunnelling in the core network.

Proposal: The 5G system should support both point-to-point (unicast) and point-to-multipoint (multicast) tunnelling for delivering multicast PDUs through the core network.

User plane establishment between the RAN node and the ingress node is not required each time a user of the RAN node is added to a multicast group. The user plane between the ingress node and the RAN node only needs to be added when the first users served by the RAN node joins the multicast group (of a user within the multiplex group is handed over to the RAN node). The user plane between the ingress node and the RAN node only needs to be terminated when the last users served by the RAN node leaves the multicast group, terminates his session or is handed over to another RAN node.

A further complication is that the ingress node towards the PLMN for the multicast user plane can already be allocated before a user joins the multicast session and may thus be different from the SMF controlling the user context. This can happen if other users served by a different SMFs joined the multicast session before or if an AF requested to reserve resources for the multicast group.

As mobility between RAN nodes should be opaque to the SMF, it seems preferable that the RAN nodes control the establishment of the multicast distribution towards them.

Proposal: The RAN node signals towards the ingress node to request the establishment of the multicast group distribution user plane.

The following information needs to be exchanged for the multicast distribution establishment:

For unicast transport: IP address and GTP-U ID of the RAN node needs to be conveyed to the ingress node.

For multicast transport: IP multicast transport address needs to be conveyed towards the RAN node.

1.3	Signalling PDU session modification to UE

The solution #3 does not include a signalling towards UE, which needs to be corrected at least for Alt#2. In Alt#2, the UE initiates the PDU session modification for multicast by transmitting PDU Session Modification Request. When the UE transmits the PDU Session Modification Request, the UE enters the PDU session modification pending states and it expects the reception of PDU Session Modification Command in the response. Thus, the PDU Session Modification Command needs to be transmitted to the UE.

It may be also purposeful to transmit PDU Session Modification Command to the UE in case the procedure is trigged via UPF, i.e. Alt#1, to provide the UE with downlink QoS rules for identification of a Multicast flow (QoS flow) which the network is to use for a particular downlink traffic as specified in TS 24.501.

TS 24.501 #### 9.11.4.13 QoS rules The purpose of the QoS rules information element is to indicate a set of QoS rules to be used by the UE, where each QoS rule is a set of parameters as described in subclause 6.2.5.1.1.2: a) for classification and marking of uplink user traffic; and b) for identification of a QoS flow which the network is to use for a particular downlink user traffic. NOTE: The UE needs to be aware of a QoS flow which the network is to use for a particular downlink user traffic e.g. to determine whether a resource is available for downlink media of a media stream of an SDP media description provided by the UE in an IMS session. The QoS rules may contain a set of packet filters consisting of zero or more packet filters for UL direction, zero or more packet filters for DL direction, zero or more packet filters for both UL and DL directions or any combinations of these. The set of packet filters determine the traffic mapping to QoS flows.

The PDU Session Modification Command should include Multicast flows and associated QoS information. The presence of QoS rules for downlink traffic matching the multicast address sent in the join message indicates implicitly to the UE that the multicast context has been successfully created or modified. The modification of PDU Session Modification Command with explicit indication of the multicast address could be also considered.

Proposal: Solution #3 should be updated to allow for PDU Session Modification Command/Ack signalling to the UE.

2	Conclusions

It is proposed to update to solution #3 in TR 23.757 as per the proposals made in the discussion section. The proposed changes are shown below.

Proposed changes to TR 23.757

6.3	Solution #3: Integrated Multicast and Unicast Transport

6.3.1	Functional Description

6.3.1.1	System Architecture

This solution addresses Key Issue 1 and proposes a system architecture that reuses as much as possible the system architecture and procedures of current 5GS unicast system architecture. The architecture functional entities are described in Annex A.1 "5G MBS system architecture based on unicast 5GC".

6.3.1.2	Multicast Session Context and Multicast flow characteristics

The proposed multicast communication service session management is an extension of the existing solution for IPTV in TS 23.316 [7].

The Multicast Session context is identified by a Multicast Session context ID and is used to represent information about the group of UEs receiving Multicast flows with the same Multicast Session context ID. Multicast flows represents QoS flows within a Multicast Session context.

In case of IP PDU session type, the Multicast session context ID represents one IP multicast group address (i.e. any source multicast or source specific multicast). The packet filters for all Multicast flows within the Multicast context shall have the same destination and, in case of source specific multicast, also source IP addresses. The default Multicast flow shall allow for any source and destination ports and any protocols. An AF may request the PCF/NEF to create policies for Multicast flows within the Multicast Session context to meet the needs of application service flows that use different ports and protocols.

Note:	Whether or not a dedicated QoS flow(s) will be used to transmit multicast data will be determined in the normative phase.

Figure 6.3.1.2-1: Multicast Session Context, UE group and Multicast flow model

On N3 or MB-N3, the Multicast flows within one Multicast Session context use the same shared tunnel.

At the NG-RAN, the Multicast flow identifier maps to radio bearer. If the QoS characteristics of Multicast flows allow, NG-RAN can also map several Multicast flows within a Multicast Session context to one radio bearer. The characteristics of the radio bearer depends on RAN decision on whether to deliver the Multicast flow content via unicast or point-to-multipoint (PTM) transmission.

The 5G QoS model is extended to support Multicast flow. At Session Management level for a particular UE the Multicast session context may exist in association with at least one PDU session, and can be set-up during the PDU session establishment or modification procedure. It may be modified at any point via PDU session modification procedure. Note that the Multicast session context is common to all UEs configured with the Multicast session context, but the associated PDU session context is specific to each UE.

The Multicast context ID and Multicast flow ID are assigned by the SMF. The SMF provides the Multicast flow information (packet filters, etc.) to the UPF.

If point-to-point (PTP) tunnelling is used in N3, the SMF provides RAN with Multicast context ID, Multicast flows and associated QoS information. The RAN responds with downlink tunnel information for the Multicast context. The SMF configures UPF with Multicast flows, associated QoS information and the downlink tunnel information.

If point-to-multipoint tunnelling is used (MB-N3), the SMF provides the UPF with MB-N3 tunnelling information. The SMF provides Multicast context ID, Multicast flows and associated QoS information and MB-N3 tunnel information to the RAN.

Point-to-point and point-to-multipoint tunnelling on N3/N9 have both their merit. Point-to-point tunnelling provides more freedom in regard to the deployment of UPF(s) and RAN (e.g. CU-UP) because the infrastructure interconnecting these elements of 5G system does not need to support multicast routing. On the other hand, point-to-multipoint tunnelling on N3/N9 can me more resource efficient if the same content is delivered to a larger number of RAN nodes.  Noting that the user plane configuration is slightly different depending on whether N3/N9 uses point-to-point (unicast) or point-to-multipoint (multicast) tunnelling, it seems anyway beneficial at the cost of additional standardization and implementation efforts to support both point-to-point and point-to-multipoint tunnelling on N3/N9 by the specification which allows to meet requirements of different deployments.

Figure 6.3.1.2-1 depicts the user plane path for a Multicast flow.

Figure 6.3.1.2-1: Multicast context / multicast flow user plane model

When multicast transport is established, there is no need for unicast transport from the AF. In AS, when RAN decides to switch to PTM transmission, the DRB previously allocated to the QoS Flow is not used anymore.

An UPF, based on configuration received from SMF, identifies a packet as belonging to a Multicast flow, in which case it delivers it to one or multiple RAN nodes via a shared tunnel identified by a shared TEID associated with the Multicast session context to which the Multicast flow belongs to.

The AN delivers download data for the Multicast flow via PTM or PTP over the air.

Each Multicast session context is managed by one SMF that may be different from the SMF controlling the PDU session of a user within the multicast group. The identification of SMF handling Multicast session is stored in the UDR as part of multicast group context information.

User plane establishment for a multicast group between the RAN node and the ingress node is not required each time a user of the RAN node is added to a multicast group. The user plane between the ingress node and the RAN node only needs to be added when the first users served by the RAN node joins the multicast group (of a user within the multiplex group is handed over to the RAN node). The user plane between the ingress node and the RAN node only needs to be terminated when the last users served by the RAN node leaves the multicast group, terminates the PDU session, or is handed over to another RAN node.

For each specific UE, the SMF configures the associated unicast PDU session information, which can be used to transport the multicast data to the UE. This will be used e.g. in case of mobility to NG RAN node not supporting 5G MBS, or when a UE does not support receiving multicast data over the radio according to 5MBS. The SMF generates unicast QoS flows matching the multicast flows within the unicast PDU session. The unicast QoS flows is not assumed to be used when NG RAN supports 5G MBS.

6.3.2	Procedures

6.3.2.1	Multicast context and Multicast flow setup/modification via PDU Session Modification procedure

The Multicast context and Multicast flow setup/modification uses an enhanced PDU session modification procedure for unicast traffic defined in TS 23.502 [8].

Figure 6.3.2-1: PDU Session modification for multicast

1.	The content provider announces the availability of multicast using higher layers (e.g., application layer). The announcement includes at least the multicast address of a multicast group that UE can join.

The content provider may send a request to register and reserve resources for a multicast group to the NEF and communicate the related multicast address as detailed in subclause 6.3.2.2.2.

The content provider may invoke the services provided by the NEF to provision the multicast information. The multicast information is used to identify (e.g., IP Address of multicast data) and reserve resources for the multicast. The NEF selects SMF2 controlling an UPF2 serving as ingress point for the multicast data and creates a multicast context and stores related information including the SMF ID in the UDR. The SMF2 or UPF2 may allocate an IP address and Port for ingress multicast traffic, which is then provided to the content provider via NEF

NOTE 1:	The request to reserve resources for the corresponding multicast session is optional and can be replaced by configured data based on commercial agreements. If IP multicast is used in the external network, the content provider does not require information where to send the multicast data.

NOTE 2:	SMF1 and SMF2 can be identical.

2.	The UE registers in the PLMN (see clause 4.2.2.2 of TS 23.502 [8]) and request the establishment of a PDU session (see clause 4.3.2.2 of TS 23.502 [8]). The UE also indicates its capability to receive multicast data over the radio.  The AMF obtains information from the UDM whether the UE can join multicast sessions as part of the SMF Selection Subscription data. If so, for direct discovery, the AMF selects an SMF1 capable of handling multicast sessions based on locally configured data or a corresponding SMF capability stored in the NRF and also indicates the UE's capability to receive multicast data over the radio to the SMF.

3.	The content provider announces the availability of multicast using higher layers (e.g., application layer). The announcement includes at least the multicast address of a multicast group that UE can join.

4.	Alternative 1: user plane signalling (can also be used a legacy UE not supporting the capability to receive multicast data over the radio):

4a.	The UE joins the multicast group.

4b.	The UPF is a multicast capable router. The reception of the join message triggers the UPF to notify the SMF1. The UPF can be optimized to send the notification only when the UE's status in regard to number of multicast groups UE has joined changes, i.e., when the UE joins or leaves a group. The SMF initiates PDU session modification procedure upon the reception of the notification from the UPF.

5.	Alternative 2: control plane signalling:

5a.	The UE sends the PDU Session Establishment/Modification Request either upon a request from higher layers or upon a detection by lower layers of UE joining a multicast group (i.e., detection of IGMP or MLR and detection of the change of content of these messages). The PDU Session Modification Request shall include information about multicast group, which UE wants to join, such as multicast addresses listed in the IGMP and MLR messages. This information is needed for configuration of the UPF with appropriate packet filters.

5b.	The AMF invokes Nsmf_PDUSession_UpdateSMContext (SM Context ID, N1 SM container (PDU Session Modification Request with the multicast information)).

6.	If the SMF1 has no information about the multicast context for the indicated multicast group, the SMF1 checks at the UDR whether a multicast context for the multicast group (address) exists in the system. If the multicast context for the multicast group does not exist, then the SMF1 creates it when the first UE joins the multicast group, stores the multicast context including itself as multicast controlling SMF in the UDR, and configures an UPF to handle the multicast data distribution (SMF1 and SMF2, and UPF1 and UPF2 in this flow are then identical). If it is the first UE joining the multicast group, the UPF may also have to join the multicast tree towards the content provider; the SMF should request the UPF to join the multicast tree when configuring the UPF, see e.g. step 13 and 23. If a multicast context already exists in the UDR, the SMF1 retrieves the related information, including information related to SMF2 controlling the multicast ingress point.

NOTE X1:	If option A is used, SMF1 further interacts with SMF2 for e.g., determining the QoS rules for the multicast group.

7.	The SMF1 requests the AMF to transfer a message to the RAN node using the Namf_N1N2MessageTransfer service (N2 SM information (PDU Session ID, Multicast Context ID, ), N1 SM container (PDU Session Modification Command (PDU Session ID , multicast information ([Multicast Context ID], multicast address,)) to

-	create a multicast context in the RAN, if it does not exist already; and

-	inform about the relation between the multicast context and the UE’s PDU session.

NOTE X2:	In addition to step 7: If option A is used, the SMF1 further 1) maps the received QoS information of the multicast QoS flow into Qos flow of the PDU Session, 2) includes the unicast QoS flow information associating to multicast QoS flow, QoS flow information of multicast, association between the unicast QoS flow and multicast QoS flow in the N2 SM information, and 3) includes Unicast information (i.e., QoS rules for Unicast flows) in the N1 SM container; if option B is used, the SMF2 ID will be included in N2 SM information.

If the UE does not support the reception of multicast data, the PDU Session Modification Command does not include multicast information, and the N2 SM information does not include Multicast Context ID and related multicast QoS flow information.

8.	The N2 session modification request is sent to the RAN. The request is sent in the UE context using the PDU Session Resource Modify Request message enhanced with multicast related information, which includes a multicast group identity (e.g., multicast address itself, Multicast Session context ID, and multicast flow information such as multicast QoS Flow ID and associating QoS information). The RAN is using the multicast group identity to determine that the session modification procedures of two or more UEs correspond to one multicast group. In other words, the RAN learns what UEs are receiving the same multicast from the multicast group identity. When the RAN receives a session modification request for previously unknown multicast group identity, the RAN configures resources to serve this multicast group.

9. The N1 SM container (PDU Session Modification Command) is provided to the UE.

Editor's Note: For option B, how could the QoS information sent to UE in step 9 is FFS.

10.	The RAN performs the necessary access network resource modification such as configuration of PTP or PTM bearers. RAN node checks whether the user plane for the multicast group/context distribution is already established towards the RAN node. If RAN supports MBS, RAN configures the UE for receiving the multicast data via multicast session.

NOTE 4:	The details of access network resource modification should be studied in the RAN WGs.

If RAN supports MBS, and if no user plane for multicast group distribution is established towards the RAN node, steps 11 to 15 are executed

Following steps describe option A and B respectively:

OPTION A (from step 11a to step 15a):

11a.	RAN node signals the session modification response towards AMF including the multicast distribution request for the user plane establishment [Multicast context/group ID]. If the RAN node is configured to use a unicast transport for multicast distribution sessions, it allocates a downlink tunnel ID (an IP address and a GTP-U TEID) for the reception of the multicast distribution session and indicates the downlink tunnel information in the request.

12a. AMF forwards the request towards the SMF1 via the Nsmf_PDUSession_UpdateSMContext service. If RAN supports MBS, the N2 SM message sent from RAN to SMF1 indicates successful configuration of multicast session for the UE, and steps 18 to 21 apply, otherwise, steps 22 to 34 apply.

Step 13—15 only applies for the 1st time RAN need establish the shared tunnel with UPF2.

13a. SMF1 further forwards the multicast distribution request to SMF2

14a. For unicast transport of the multicast distribution session, SMF2 configures UPF2 to transmit the multicast distribution session towards the received IP address and a GTP-U TEID.

15a. SMF2 sends a multicast distribution session response to SMF1.

OPTION B (from step 11b to step 17b):

11b.	RAN nodes selects the AMF to reach SMF2 and signals a request towards AMF [SMF2 ID, Multicast context/group ID]. If the RAN node is configured to use a unicast transport for multicast distribution sessions, it allocates a downlink tunnel ID (an IP address and a GTP-U TEID) for the reception of the multicast distribution session and indicates the downlink tunnel information in the request.

12b. AMF forwards the request towards the SMF2

13b: For unicast transport of the multicast distribution session, SMF2 configures UPF2 to transmit the multicast distribution session towards the received IP address and a GTP-U TEID..

14b SMF1 sends a multicast distribution session response to AMF. It indicates QoS rule(s) for Multicast flows. For multicast transport of the multicast distribution, it also indicates in the downlink tunnel information the transport multicast address for the multicast session.

15b AMF forwards multicast distribution session response to RAN node.

16b.	The RAN sends the session modification response.

17b.	The AMF transfers the session modification response received in step 16 to the SMF1 via the Nsmf_PDUSession_UpdateSMContext service..

18.	The UPF2 receives multicast PDUs.

19.	The UPF2 sends multicast PDUs in the N3/N9 tunnel associated to the multicast distribution session to the RAN. There is only one tunnel per multicast distribution session and RAN node, i.e., all associated PDU sessions share this tunnel.

20.	The RAN selects PTP or PTM radio bearers to deliver the multicast PDUs to UEs that joined the multicast group.

21.	The RAN performs the transmission using the selected bearer.

If the UE does not support the reception of multicast data and/or if RAN does not support MBS, steps 22 to 34 apply

22.	SMF1 configures UPF1 to forward the data within unicast transport. SMF1 signals a request for the user plane establishment towards SMF2 [Multicast context/group ID]. If the SMF1 is configured to use a unicast transport for multicast distribution sessions,  SMF1 or UPF1 allocate a downlink tunnel (an IP address and a GTP-U TEID) for the reception of the multicast distribution session and SMF1 indicates the downlink tunnel information in the request.

23: For unicast transport of the multicast distribution session, SMF2 configures UPF2 to transmit the multicast distribution session towards the received IP address and a GTP-U TEID..

24: SMF2 sends a multicast distribution session response to SMF1. It indicates QoS rule(s) for Multicast flows. For multicast transport of the multicast distribution, it also indicates in the downlink tunnel information the transport multicast address for the multicast session.

NOTE X3: If option B is adopted, the SMF2 will further indicates QoS rule(s) for Multicast flows to SMF1

Following step 25 to 30 are used for Option B only.

25: SMF1 configures UPF1 to receive the multicast distribution session and forward the data within unicast transport.

If the SMF decides to establish dedicated QoS flow for the unicast transfer of the multicast data, steps 22 to 33 apply

26.	The SMF derives the QoS Rules for the unicast transfer of multicast data based on QoS parameters for multicast transmission received in step 24. SMF maps the multicast QFI to a unicast QFI of the PDU Session; other QoS parameters of unicast QoS Profile are same as the one for multicast transmission. The SMF requests the AMF to transfer a message to the RAN node using the Namf_N1N2MessageTransfer service (N1 SM container (PDU Session Modification Command (PDU Session ID, QoS rule(s)).

27	The N2 session modification request is sent to the RAN..

28. The N1 SM container (PDU Session Modification Command) is provided to the UE.

29.	The RAN sends the session modification response.

30.	The AMF transfers the session modification response received in step 29 to the SMF1 via the Nsmf_PDUSession_UpdateSMContext service.

31.	The UPF2 receives multicast PDUs.

32.	The UPF2 sends multicast PDUs in the N3/N9 tunnel associated to the multicast distribution session to UPF1. There is only one tunnel per multicast distribution session and destination UPF, i.e., all associated PDU sessions share this tunnel.

33.	The UPF1 forwards the multicast data via unicast.

34.	The RAN forwards the multicast data via unicast.

6.3.2.2	Multicast group configuration

6.3.2.2.1	General

Configuration of a multicast group in the 5GC can occur:

-	when the first UE joins the multicast group

-	based on static configuration.

-	Triggered by an AF request via the NEF.

At service layer, the MSF can manage an MBS service, and apply related NEF procedures to configure a multicast group.

6.3.2.2.2	Multicast group configuration via NEF

6.3.2.2.2-1: Multicast group configuration via NEF

1:	AF of content provider may register at NEF that it provides contents for a multicast group (identified by multicast group ID which may be IP multicast address). Multicast information may further include media type information (e.g. audio,video …), QoS requirements, UE authorization information, service area identifying the service scope, and start and end time of MBS. The AF may also request the allocation of an ingress transport address where to send tunnelled multicast data.

2.	NEF checks authorization of content provider. NEF selects SMF2 as ingress control node, possibly based on location area.

3-4:	NEF requests storage of multicast session context at UDR and provides multicast group ID, selected SMF2.

5:	The NEF may interact with the PCF by triggering a Npcf_PolicyAuthorization_Create request and provides Multicast group ID, AF Identifier, and the QoS requirements.

6: The PCF determines whether the request is authorized and notifies the NEF if the request is not authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration for this AF), and notifies the result to the NEF.

Editor’s note: It is FFS if the authorization check and check of allowed NEF input is performed by NEF or PCF.

Editor's note: It is FFS if step 5 and 6 are needed considering step 8 and 9 can be used for the same purpose.

If the request is not authorized, the required QoS is not allowed, NEF responds to the AF in step 9 with a Result value indicating the failure cause.

7: NEF request SMF2 to reserve ingress resources for a multicast distribution session and provides Multicast group ID. It also indicates if the allocation of an ingress transport address is requested.

8:	The SMF sends SM MBS Policy Association Request to PCF with the Multicast group ID.

9: The PCF responds with SM MBS Policy Association Response with policies for the Multicast group ID.

10:	SMF2 selects the UPF2 and requests it to reserve user plane ingress resources

11: If requested, UPF2 selects an ingress address (IP address and port) and provides it to SMF2

12: SMF2 indicates the possibly allocated ingress address to the NEF. It also indicates the success or failure of reserving transmission resources.

13: The NEF indicates the possibly allocated ingress address to the AF.

.

6.3.2.2.3	Example of AF use of Multicast group configuration via NEF in combination with SIP signalling.

6.3.2.X.2-1: Use of MBS Transport activation via NEF and UE configuration via SIP

1.	A SIP Invite (e.g. to join a conference) by a UE may trigger the AF to perform a Multicast group configuration via NEF. An AF may also receive similar SIP Invites by other UEs after it has configured the multicast group in step 2.

2.	If a related multicast group is not yet configured, the AF initiates MBS Multicast group configuration as described in 6.3.2.2.2.

3.	The AF includes in the SIP Invite response the multicast IP address information allocated to the multicast group.

4. The UE joins the Multicast group by initiating PDU session establishment/modification procedure including the multicast IP address information as described in 6.3.2.1.

6.3.2.x.3	Example of MBS service activation via MSF and MSF interaction with MBS transport.

Figure 6.3.2.x.3-1 shows an example of MSF (service layer) and MBS transport interaction, and AF/MSF interaction when an AF establishes an MBS service session via MSF, and configures UE via SIP signaling. Note that AF and UE may interact using different application layer protocols.

6.3.2.x.3-1: Use of MBS service activation via MSF and MSF interaction with MBS transport

1.	A SIP Invite (e.g. to join a conference) by a UE may trigger the AF to perform a Multicast group configuration via NEF. An AF may also receive similar SIP Invites by other UEs after it has configured the multicast group in step 2.

2.	If a related multicast group is not yet configured, the AF sends Activate MBS Transport Request (optional: media information, service area, start time) to MSF-C.

3.	The MSF-C initiates MBS Multicast group configuration as described in 6.3.2.2.2.

4.	The MSF-C configures the MSF-U to handle the MBS session and provides the address information received in step 3 as destination address

5.	The MSF-U allocates a transport address to receive the multicast data.

6.	The MSF-C responds to AF with Activate MBS Transport Response (MBS context ID/IP multicast address information, and MSF-U transport address information)

7.	The AF includes in the SIP Invite response the multicast IP address information allocated to the multicast group.

8. The UE joins the Multicast group by initiating PDU session establishment/modification procedure including the multicast IP address information as described in 6.3.2.1.

Editor’s note: It is FFS of the role of MSF-C and MSF-U.

6.3.3	Impacts on services, entities and interfaces

SMF: The SMF must handle a multicast context and the enhanced PDU session procedures.

UPF: If the UE joins multicast group via user plane, the UPF must support a new capability to trigger a user plane event in a response to the reception of a join message. The UPF should also act as multicast capable router but this functionality was already introduced in TS 23.316 [7].

RAN: The RAN must support the PDU session procedures and store UEs' association with multicast group in a context as received from the SMF. The RAN should be able to select PTP or PTM bearers that are used for multicast data transmission to UEs.

N3: A tunnel on this interface, which is configured when the first UE joins a multicast group and PDU session modification is performed, should be used to deliver multicast data from the UPF to the RAN.

NEF: The NEF may interacts with Content Provider to reserve reception resources for a multicast group and receive QoS requirements, UE authorization information, service area, and start and end time of MBS sessions. It may also select the SMF handling multicast transmission. It stores information related to the multicast session in the UDR

UE: It needs to indicate the MBS service information as part of the user plan join message (e.g., IGMP join), or of the control plan message (e.g., PDU Session modification request).

UDR: Stores a multicast group context