SA WG2 Meeting #139e	S2-2004201r02

Elbonia, June 1 – 12, 2020	(revision of S2-XXXXXX)

Source:	Philips International B.V.

Title:	UE-to-Network Relay discovery and handling of PDU session parameters with CN based relay selection.

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This document addresses some issues related to KI#1 (ProSe direct discovery) and KI#3 (UE-to-Network relay) of TR 23.752, in particular related to the discovery of UE-to-Network relays and how this is linked to the usage of PDU session parameters for both layer-2 and layer-3 UE-to-Network relays..

# 1. Discussion

As mentioned in KI#3 a solution is needed on how to handle PDU Session related attributes for UE-to-Network Relays. The PDU session attributes may have quite some impact on the UE-to-Network Relay. For example, if the Remote UE wishes to use a URLLC slice, but the UE-to-Network Relay has limited resources at that moment in time, it may not be able to address the low-latency requirements of the URLLC slice. Also, if a Remote UE wants to access a certain NPN, then also the UE-to-Network Relay may need to have permission to send data to the NPN.

Simply provisioning the UE-to-Network Relay with all kinds of information about supported slices, NPNs and other related attributes, and exposing this information during discovery of a UE-to-Network Relay is not desirable. A lot of this information is privacy and operator sensitive information, and a UE-to-Network Relay is likely to be an end user device that cannot be trusted with this information. It would also not be scalable, since the number of slices, NPNs is potentially quite large, and it is not known in advance which Remote UEs the UE-to-Network Relay will encounter. The Remote UE could also for example be an inbound roaming device, requesting access to a slice, NPN, or DNN of a roaming partner. So a UE-to-Network Relay would also need the necessary information on how to deal with that. Furthermore, this information would need to be kept up-to-date in the UE-to-Network Relay.

Therefore, we propose to keep as much of the information and decision power on whether a UE-to-Network Relay is capable to serve as relay for the Remote UE inside the core network, e.g. the AMF. The added benefit is that AMF can use the most up-to-date information combined from various network functions and use the  combined information from various network functions to decide which UE-to-Network Relay amongst multiple relays is best suited to serve as relay for the Remote UE. Without involving the core network in the decision, the Remote UE is unlikely to have sufficient information about each Remote UE’s capabilities and current status to make a good decision, which may result in trial-and-error if the selected UE-to-Network relay does not meet the Remote UE’s requirements.

To deal with some of the privacy concerns, we propose to extend the concept of Relay Service Codes as defined in TS 23.303. In order to avoid the overhead of the existing ProSe framework in TS 23.303, we propose to apply these service codes to a UE-to-Network Relay discovery mechanism that leverages the V2X framework as defined in TS 23.287.

The main difference with S2-2004202 “KI#3, New Sol. for UE-to-Network Relay discovery and handling of PDU session parameters with Remote UE based relay selection” is that in this solution the AMF (and not the Remote UE) is responsible to select which UE-to-Network relay (amongst multiple discovered UE-to-Network relays in vicinity of the Remote UE) is best suited to serve as relay for the Remote UE’s requested PDU session parameters.

# 2. Text Proposal

It is proposed to capture the following solution in TR 23.752.

# * * * * First change * * * *

## 6.0 Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  | X |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  | X |
| 8 |  |  |  | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  |  | X |  |  |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 |  |  |  |  |  |  | X |  |
| 15 |  |  |  |  |  |  | X |  |
| 16 |  |  |  |  |  |  |  | X |
| 17 |  |  |  |  |  |  |  | X |
| 18 | X |  |  |  |  |  |  |  |
| 19 | X |  | X |  |  |  |  |  |
| 20 |  | X |  |  |  |  |  |  |
| 21 |  | X |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |
| … | … | … | … | … | … | … | … | … |
| X | X |  | X |  |  |  |  |  |

# * * * * Second change * * * *

## 6.x	Solution #x: UE-to-Network Relay discovery and handling of PDU session parameters with CN based relay selection.

### 6.x.1	Description

This solution addresses the discovery of UE-to-Network Relays and usage of PDU session related attributes for key issues #1 and #3. The underlying principle of this solution is to make use of Relay Service Codes as defined in TS 23.303 in the context of the V2X architecture as defined in TS 23.287.

The idea is that each Relay Service Code is mapped to a combination of PDU session parameter values, such as PLMN ID (+ NID/CAG ID), S-NSSAI, DNN, PDU session type, etc., and possibly some additional parameters (such as group IDs, QoS requirements or frequency bands) that Remote UEs may wish to use for connecting to the core network via a UE-to-Network Relay. In addition, each of these Relay Service Codes may be associated to an initial security context by which the Remote UE and UE-to-Network Relay may prove that they are authorized to issue a discovery/(in)direct communication request for that particular Relay Service Code. Several of the above mentioned PDU session parameters may impose restrictions on whether a UE-to-Network Relay is capable of acting as a relay for the Remote UE (at a given moment in time). For example, if a Remote UE wants to access a certain NPN, then also the UE-to-Network Relay may need to have permission to send data to the NPN, or for example if a UE-to-Network Relay currently has limited resources, it may not be able to meet the desired low-latency requirements for a URLLC slice that the Remote UE wishes to use (e.g. because it is already serving several other Remote UEs or may be involved in a heavy online gaming session).

Since a lot of the above mentioned information is privacy sensitive and could lead to tracking of Remote UEs and expose deployment information of the operator (e.g. which slices and NPNs are supported by the core network), the information about the mapping should be stored and used as much as possible in the core network, and should not be provisioned as such to UE-to-Network Relays, which are untrusted end-user devices. Furthermore, since UE-to-Network relays could potentially act as a relay for a diverse set of Remote UEs (which may even include inbound roaming Remote UEs), the number of possible combinations of values for the above mentioned parameters may potentially be quite high and may require quite a large number of Relay Service Codes, and may require frequent messaging between the UE-to-Network relay and the network to keep the information up to date. Therefore, in this proposal the privacy sensitive and operator sensitive information, such as S-NSSAI, is kept in the network, and only provided, if necessary, to the single UE-to-Network relay that is selected by the AMF to serve as UE-to-Network relay for the Remote UE.

Furthermore, as mentioned above, various dynamic aspects may play a role in determining if a UE-to-Network relay is currently capable to meet the requirements for relaying a Remote UE’s desired PDU session. For example, the UE-to-Network Relay may not know the SLA requirements for a certain slice. Also, the UE-to-Network Relay may not know if some of its PDU sessions can be adjusted in order to accommodate the Remote UE’s PDU session. Furthermore, one UE-to-Network Relay may already serve as relay for various other Remote UEs, whilst a neighbor UE-to-Network Relay may not serve any Remote UE yet, so using the neighbor UE-to-Network Relay may be a better choice. Therefore, it is best to consult the core network rather than the UE-to-Network relay deciding on its own. Also, the UE-to-Network relay may need to be reconfigured in order to properly serve as relay for the requested set of PDU session parameters (e.g. connect to a certain CAG cell in case of a PNI-NPN). As mentioned above, for privacy reasons, it is best if the information on how to do so is not stored beforehand inside the UE-to-Network Relay. Therefore, in this solution, the AMF is responsible for doing these things. The additional benefit of doing so, is that the AMF can make use of the most up-to-date information (possibly from various other network functions) to make the assessment of whether a UE-to-Network relay is capable (at that moment in time) to act as a relay for a requested set of PDU session parameters, and properly reconfigure the UE-to-Network relay accordingly. Preferably the reconfiguration is done before the indirect communication with the core network is set up.

In this particular solution, the AMF is also responsible for selecting which UE-to-Network Relay amongst multiple UE-to-Network Relays being in discovery range of the Remote UE, is best suited to serve as relay for a given set of PDU session parameters. To this end, each UE-to-Network relay sends the Relay Service Code it receives from a Remote UE during discovery to the AMF using a Discovery Report message. In case multiple UE-to-Network relays receive (within a short timeframe) the same discovery message from the Remote UE, either the NG-RAN or AMF will collect/aggregate these Discovery Report messages. After receiving the aggregated Discovery Report message(s), the AMF will select the UE-to-Network Relay that is best suited to serve as relay for the Remote UE’s requested PDU session parameters. Without involving the core network in the decision, the Remote UE is unlikely to have sufficient information about each Remote UE’s capabilities and current status to make a good decision, which may result in trial-and-error if the selected UE-to-Network relay does not meet the Remote UE’s requirements.

In this solution the message that is used for discovery, is a (V2X) Direct Communication Request that is sent to the default “broadcast” L2 identifier which allow it to be received by multiple UE-to-Network relays. The benefit of using the (V2X) Direct Communication Request is that it allows subsequent reuse of the resulting PC5 connection between the Remote UE and the selected UE-to-Network Relay for the indirect communication with the core network.

Editor’s Note: it is FFS how to deal with the case that multiple UE-to-Network Relays in vicinity of the Remote UE use different AMFs.

In order to have to provision as little number of Relay Service Codes to the UE-to-Network Relay as possible, and also to make sure a Remote UE which may use potentially unknown or outdated Relay Service Codes can still discover a UE-to-Network Relay and request access to the network via the UE-to-Network Relay, this solution uses the concept of Generic Relay Service Codes. A Generic Relay Service Code is not bound to a single specific combination of PDU session parameters values, but rather can be seen as a “wildcard”, and should match a large number of UE-to-Network relays. A Generic Relay Service Code may however impose some restrictions of when it will match (e.g. impose a restriction to a certain PLMN ID and may only be provisioned to UEs belonging to that PLMN, so when it matches it is clear that a discovered UE-to-Network Relay belongs to the same PLMN), and/or may include some encoded substring/value matching filter information. Similarly to the other Relay Service Codes, it may also be associated to an initial security context by which the Remote UE and UE-to-Network Relay may mutually prove that they are authorized to perform discovery and (indirect) connection setup over PC5. If a Generic Relay Service Code is used by the Remote UE in a Discovery Request, then the Remote UE can include a second Relay Service Code, i.e. a specific Relay Service Code that is bound to a specific set of PDU session parameters that a Remote UE wishes to use. This may include specific Relay Service Codes that may not be known to the UE-to-Network relay.

The procedures which are described in detail in Section 6.x.2 require the following information to be provisioned in the Remote UE and UE-to-Network Relay beforehand, e.g. by using a Service Authorization and Provisioning procedure of Solution#16 or other solution for KI#8:

- For the Remote UE:

- One or more Relay Service Codes which the Remote UE is authorized to use, including a flag indicating for each Relay Service Code if it is a Generic Relay Service Code or a specific Relay Service Code.

- The mapping between each Relay Service Code that the Remote UE is authorized to use and the default “broadcast” Destination Layer-2 ID(s) for initial signalling.

- The mapping between each Relay Service Code and a set of PDU session parameter values, which may include amongst others one or more of the following:

  - PLMN ID

  - NID/CAG ID

  - S-NSSAI

  - DNN

  - PDU Session Type

  - …

NOTE: for a Generic Relay Service Code, the set may be empty or e.g. only contain a PLMN ID.

Editor’s note: it is FFS whether or not Generic Relay Service Codes will include some encoded substring/value matching filter information by which the UE-to-Network relay can perform some discovery message filtering.

- (Optional) The mapping between each Relay Service Code and an initial security context (e.g. set of credentials).

- Policy to restrict the Remote UE’s PDU sessions to the PDU session parameter values corresponding to a provisioned Relay Service Code.

- For the UE-to-Network Relay:

- One or more Relay Service Codes which the UE-to-Network Relay is authorized to expose and react upon during discovery, including a flag indicating for each Relay Service Code if it is a generic Relay Service Code or a specific Relay Service Code.

NOTE: as explained above, this is preferably a subset of all the Relay Service Codes, and may consist of only a single Generic Relay Service Code.

- The mapping between each of the Relay Service Code for which the UE-to-Network Relay is authorized to expose and react upon during discovery and the default “broadcast” Destination Layer-2 ID(s) for initial signalling.

- (Optional) The mapping between each Relay Service Code and an initial security context (e.g. set of credentials).

In addition, the AMF of the UE-to-Network Relay needs to be provided with the following information:

- An extensive list of Relay Service Codes which the UE-to-Network Relay may be able to handle and get authorized for, including a flag indicating for each Relay Service Code if it is a generic Relay Service Code or a specific Relay Service Code.

- The mapping between each Relay Service Code and a set of PDU session parameter values, which may include amongst others one or more of the following:

  - PLMN ID

  - NID/CAG ID

  - S-NSSAI

  - DNN

  - PDU Session Type

  - …

NOTE: Since the list of Relay Service Codes for the AMF may not always be updated at the same time as for the Remote UEs (and UE-to-Network Relays), which may be out-of-coverage for a while, the AMF should also keep a history of old Relay Service Codes.

The PCF in the HPLMN of the UE that needs to be provisioned to be a Remote UE or UE-to-Network Relay may interact with PCF of other PLMNs (e.g. VPLMN) to perform Relay Service Code allocation and management.

### 6.x.2 Procedures

PCF

5: Direct Communication Accept

(with requested RSC)

Remote UE

UE-to-NW Relay

NG-RAN

AMF

UDM/NSSF

SMF

UE-to-NW Relay

UE-to-NW Relay

0: Authorization and provisioning of Remote UE and UE-to-Network Relays

1: Direct Communication Request (with requested RSC)

2a: Option 1 - Discovery Report (directly to AMF)

3: UE-to-NW Relay’s AMF assesses which UE-to-NW relay best capable to serve as relay for the Remote UE’s requested PDU session parameters.

6: Configure NG-RAN and other NFs to enable the selected UE-to-NW relay to access the requested NPN, slice and/or DNN.

4: Send Relay Accepted message to selected UE-to-NW relay

8: Setup PDU session (layer-2) or send IP traffic (layer-3) via UE-to-NW relay

2b: Option 2 - Discovery Report (aggregated by NG-RAN)

7: Update UE-to-NW relay PDU session(s) and/or reconfigure UE-to-NW relay

7: UE Configuration Update of selected UE-to-NW relay

Figure 6.x.2.x: Illustration of the procedure

(NOTE: in this figure it is assumed that Remote UE and UE-to-NW relays are served by the same AMF)

- The Remote UE and UE-to-Network Relay are authorized for PC5 communication and relay discovery, and are provisioned with the respective information as described in Section 6.x.1, using one of the solutions for KI#8 “Support of PC5 Service Authorization and Policy/Parameter Provisioning”. In addition, the respective information as described in Section 6.x.1 for the AMF, is provided to the AMF of the UE-to-Network Relay.

- The Remote UE can initiate discovery of UE-to-Network Relays by sending a Direct Communication Request over PC5 as specified in TS 23.287 (or a newly defined Indirect Communication Request as suggested in step 4 of Solution#7) with the requested Relay Service Code (RSC) being used as (V2X) service/application identifier. In case the requested Relay Service Code is a Generic Relay Service Code, the Direct Communication Request includes a second Relay Service Code as payload in the message, which is indicative of a set of specific PDU parameters that the Remote UE wishes to use.

NOTE: The Direct Communication Request uses the default “broadcast” Destination Layer-2 ID configured for the requested Relay Service Code (or if known the specific Layer-2 ID of the target UE-to-Network Relay if the Remote UE wants to send the request to a specific UE-to-Network Relay).

- One or more UE-to-Network Relays receive the Direct Communication Request over PC5. If the Relay Service Code in the Direct Communication Request matches a Relay Service Code that is known to the UE-to-Network Relay, the UE-to-Network Relay sends a Discovery Report (i.e. a newly defined message) to the UE-to-Network Relay’s serving AMF via NG-RAN. The Discovery Report includes the requested Relay Service Code, or alternatively if the requested Relay Service Code was a Generic Relay Service code, then the UE-to-Network Relay includes the second specific Relay Service Code (as included in the payload of the Direct Communication Request) instead. In order to send these messages to the AMF,

- two alternatives may be considered:

  - The Discovery Report messages are NAS messages that are sent directly to the AMF. The AMF may collect a number of these messages before continuing with step 3.

  - NOTE: the UE-to-Network Relay may (re-)use the PDU session that it has established in step 2 of solution#6 for layer-3 UE-to-Network relays, or step 0 of solution#7 for layer-2 UE-to-Network relays to send the Discovery Report messages to the UE-to-Network Relay’s serving AMF.

  - The Discovery Report messages are RRC messages (possibly combined with measurement reports related to the PC5 communication request) that are sent to NG-RAN, which first collects one or more Discovery Reports before sending an aggregated discovery report message via NG-AP to the AMF, in order to reduce the number of Discovery Report messages being sent to the AMF.

NOTE: details of aggregating discovery report messages need to be coordinated with RAN2.

The Discovery Report includes the requested Relay Service Code, or alternatively if the requested Relay Service Code was a Generic Relay Service code, then the UE-to-Network Relay includes the second specific Relay Service Code (as included in the payload of the Direct Communication Request) instead.

NOTE: the UE-to-Network Relay may (re-)use the PDU session that it has established in step 2 of solution#6 for layer-3 UE-to-Network relays, or step 0 of solution#7 for layer-2 UE-to-Network relays to send the Relay Service Code to the UE-to-Network Relay’s serving AMF.

- The UE-to-Network Relay’s serving AMF will verify for each UE-to-Network Relay for which a Discovery Report was received, whether or not the UE-to-Network Relay is currently capable to meet the requirement associated with the Relay Service Code’s PDU session parameters, such as PLMN-ID, S-NSSAI, CAG ID/NID, DNN. To this end, the AMF may contact some other network functions to verify if the UE-to-Network can meet those requirements, for example the AMF may contact:

  - NSSF, e.g. to verify if the UE-to-Network Relay is allowed access to a requested network slice, and if not whether or not the UE-to-Network relay can be given (temporary) access to the requested network slice for the purpose of relaying.

  - SMF, e.g. to check about other ongoing PDU sessions of the UE-to-Network Relay and their QoS, and assess whether one or more PDU sessions can be added or adjusted within the UE-to-Network Relay in order to meet the requirements of the requested network slice requested by the Remote UE.

  - UDM, e.g. to verify if the UE-to-Network Relay is allowed access to a requested NPN, and if not whether or not the UE-to-Network relay can be given (temporary) access to the requested NPN for the purpose of relaying.

  - PCF, e.g. to fetch the policies, such as the UE Route Selection Policy (URSP), that are currently active for a UE-to-Network relay.

  - NWDAF, e.g. to fetch some analytics data that may impact the performance of selecting a particular UE-to-Network relay over another one (e.g. mobility analytics, user data congestion analytics, communication patterns).

NOTE: Potentially a new feature could be added to NWDAF to assess the reputation of UE-to-Network relays by analyzing how these UEs behave whilst relaying traffic from a Remote UE, e.g. whether they throttle bandwidth for the relayed traffic or drop connections used for relaying (possibly even signalled by the Remote UE if this leads to performance or other issues). Based on this analysis, a UE-to-Network relay could be flagged as UEs with an increased risk of not being trustworthy, which may influence whether or not they are selected to act as a UE-to-Network relay.

After assessing for each UE-to-Network Relay for which a Discovery Report was received, the AMF will select the UE-to-Network Relay that is best suited to serve as relay for the requested PDU session parameters from the Remote UE.

Editor’s Note: it is FFS how to deal with the case that multiple UE-to-Network Relays in vicinity of the Remote UE use different AMFs.

Optionally, the AMF may trigger an update of some of the other network functions to enable the selected UE-to-Network Relay to serve as a relay for the requested PDU session parameters, e.g. it may extend the Allowed NSSAI list for the UE-to-Network Relay or extend the Allowed CAG List for the UE-to-Network Relay with the CAG ID of the NPN that the Remote UE wishes to use.

- The AMF sends a Relay Accepted message (i.e. a newly defined message) to the selected UE-to-Network Relay. may send an RRC Connection Reconfiguration message and/or send a UE Configuration Update message to the selected UE-to-Network Relay, including the requested Relay Service Code. Upon receiving the RRC Connection Reconfiguration and/or UE Configuration Update, the UE-to-Network Relay may update or re-establish or start a new PDU session that will be used to relay the traffic from the Remote UE to the core network. It may also update the RAN connection, e.g. to connect a particular CAG cell in case of NPN.

NOTE: optionally, the AMF may send a “Relay rejected” message for the requested Relay Service Code to the UE-to-Network relays that have not been selected.

- The selected UE-to-Network Relay performs the PC5 unicast link security procedure and sends a Direct Communication Accept message to the Remote UE that includes the given Relay Service Code as (V2X) service/application identifier. This message may include some additional information about the relay, such as IP configuration information (e.g. for layer-3 relaying).

- Optionally, the AMF may trigger an update of some of the other network functions to enable the selected UE-to-Network Relay to serve as a relay for the requested PDU session parameters, e.g. it may ask the SMF to update an ongoing PDU session of the UE-to-Network relay, or ask the PCF to update certain policies for the UE-to-Network Relay. The AMF may also trigger an update to NG-RAN, e.g. send a PDU Session Resource Modify Request for one or more PDU sessions of the UE-to-Network Relay.

NOTE: For any policy updates, the solutions for KI#8 “Support of PC5 Service Authorization and Policy/Parameter Provisioning” are expected to be used.

- Depending on step 6, the AMF may send a UE Configuration Update message to the selected UE-to-Network Relay,  and/or the SMF may issue a PDU session modification command, and/or the NG-RAN may issue a RRC Connection Reconfiguration. Upon receiving one or more of these commands, the UE-to-Network Relay may update or re-establish or start a new PDU session that will be used to relay the traffic from the Remote UE to the core network. It may also update the RAN connection, e.g. to connect a particular CAG cell in case of NPN.

- In case of layer-2 relay, the Remote UE continues with step 4 of Solution#7 over the PC5 connection established in step 5, whilst restricting the PDU parameters (e.g. in the Initial Registration) to the configured PDU parameters related to the Relay Service Code as received in the Direct Communication Accept message.

In case of layer-3 relay, the Remote UE continues with step 4 of Solution#6, whereby the Remote UE can assume the IP traffic is forwarded to the correct destination based on the configured PDU parameters related to the Relay Service Code.

If the Remote UE wants/needs to establish a PDU session with different PDU parameters (e.g. different S-NSSAI or different CAG ID/NID), the Remote UE shall repeat steps 1-8. The AMF should monitor whether the Remote UE (in case of layer-2 relay) or the UE-to-Network Relay (in case of layer-3 relay) starts using PDU session parameters that do not correspond to the Relay Service Code that was initially requested by the Remote UE, and may reject such PDU session.

6.7.4	Impacts on Existing Nodes and Functionality

This solution may impact the following entities:

Remote UE:

- Support for using (V2X) Direct Communication Request with Relay Service Codes.

- Support provisioning of Relay Service Codes and related PDU session parameter information.

UE-to-Network Relay:

- Support for using (V2X) Direct Communication Request with Relay Service Codes.

- Support provisioning of Relay Service Codes.

- Support discovery reporting .

- Support Relay Accepted/Rejected messages.

AMF:

- Support configuring the mapping of Relay Service Codes and related PDU session parameter information

- Support assessment, selection of the best UE-to-Network Relay and reconfiguration of the selected UE-to-Network Relay based on requested Relay Service Code.

- Support discovery reporting

- Support Relay Accepted/Rejected messages.

# * * * * End of changes * * * *