SA WG2 Meeting #S2-140E	S2-2005151

19 August - 02 September 2020, Electronic, Elbonia	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2410 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the Announe message handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, it proposes the static method based on external port configuration as per IEEE 802.1AS 10.3.1 method b. And in the 802.1AS 10.3.1.2 external port configuration overview, it said: The method used by the external entity to determine the synchronization spanning tree is outside the scope of this standard. However, as with the BMCA, Announce messages are used to transport information on the time-synchronization spanning tree and Grandmaster PTP Instance time properties information from one PTP Instance to the next in the tree. But how to handle the received Announce message is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The DS-TT and NW-TT port state are set as per clause 5.27.1.1 The operations, Per PTP instance global variables and Per-Port global variables, need to comply with IEEE 802.1AS [104] clause 10.3 external port configuration Part |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to handle the Announce message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

H.X	Signalling for Announce message

The DS-TT and NW-TT port state are set as per clause 5.27.1.1.

With the external port configuration in 802.1AS [104] 10.3.1.2, when the NW-TT port which state is SlavePort receives the Announce message, the operations, Per PTP instance global variables and Per-Port global variables, it need to comply with IEEE 802.1AS [104] clause 10.3.7.2 as follows:

-	In the PortAnnounceInformationExt, the NW-TT receives and stores new Announce information received in Announce messages.

-	In the PortStateSettingEx, it may update the PTP instance variables and Per-Port variables.

-	In the PortAnnounceTranmit, it uses the PTP instance variables and Per-Port variables to construct the Announce message.

END OF CHANGES