3GPP TSG-SA WG2 Meeting #140-e	S2-2005328

# Electronic meeting, 19 August-1 September 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2415 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of HO triggered by MRL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In the incoming LS (S2-2004808/R3-204324), RAN3 has raised concern that the HO procedure triggered by the mobility restriction list is a new behaviour. RAN3 also commented that the MRL is only used to control subsequent mobility in CM-CONNECTED. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The HO triggered by the MRL is removed. If the UE is not allowed to access this cell, then the AMF releases the NAS signalling connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A new RAN behaviour is introduced. Since the MRL is only used to control subsequent mobility in CM-CONNECTED, this would mean that the UE will still stay in the cell which is not allowed to access before the next handover procedure is triggered by the existing criteria. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

5.30.3.4	Network and cell (re-)selection, and access control

The following is assumed for network and cell selection, and access control:

-	The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell (see TS 38.300 [27], TS 38.304 [50]);

NOTE 1:	The above also implies that cells are either CAG cells or normal PLMN cells.

-	In order to prevent access to NPNs for authorized UE(s) in the case of network congestion/overload, existing mechanisms defined for Control Plane load control, congestion and overload control in clause 5.19 can be used, as well as the access control and barring functionality described in clause 5.2.5, or Unified Access Control using the access categories as defined in TS 24.501 [47] can be used.

-	For aspects of automatic and manual network selection in relation to CAG, see TS 23.122 [17];

-	For aspects related to cell (re-)selection, see TS 38.304 [50];

-	The Mobility Restrictions shall be able to restrict the UE's mobility according to the Allowed CAG list (if configured in the subscription) and include an indication whether the UE is only allowed to access CAG cells (if configured in the subscription);

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

NOTE 2:	It is assumed that the AMF is made aware of the supported CAG Identifier(s) of the CAG cell by the NG-RAN.

-	If at least one of the CAG Identifier(s) received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If none of the CAG Identifier(s) received from the NG-RAN are part of the UE's Allowed CAG list, then the AMF rejects the NAS request and the AMF should include CAG information in the NAS reject message. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request and the AMF should include CAG information in the NAS reject message. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

-	During transition from RRC Inactive to RRC Connected state:

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a CAG cell, NG-RAN shall reject the RRC Resume request from the UE if none of the CAG Identifiers supported by the CAG cell are part of the UE's Allowed CAG list according to the Mobility Restrictions received from the AMF.

-	When the UE initiates the RRC Resume procedure for RRC Inactive to RRC Connected state transition in a non-CAG cell, NG-RAN shall reject the UE's Resume request if the UE is only allowed to access CAG cells according to the Mobility Restrictions received from the AMF.

-	During connected mode mobility procedures:

-	Based on the Mobility Restrictions received from the AMF:

-	Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and none of the CAG Identifiers supported by the CAG cell are part of the UE's Allowed CAG list;

-	Source NG-RAN shall not handover the UE to a non-CAG cell if the UE is only allowed to access CAG cells;

-	If the target cell is a CAG cell, target NG-RAN shall reject the N2 based handover procedure if none of the CAG Identifiers supported by the CAG cell are part of the UE's Allowed CAG list in the Mobility Restriction List;

-	If the target cell is a non-CAG cell, target NG-RAN shall reject the N2 based handover procedure if the UE is only allowed to access CAG cells based on the Mobility Restriction List.

-	Update of Mobility Restrictions:

-	When the AMF receives the Nudm_SDM_Notification from the UDM and the AMF determines that the Allowed CAG list or the indication whether the UE is only allowed to access CAG cells have changed;

-	The AMF shall update the Mobility Restrictions in the UE and NG-RAN accordingly; and

-	If the UE is currently accessing a CAG cell and the CAG Identifier(s) supported by the CAG cell have been removed from the Allowed CAG list or if the UE is currently accessing a non-CAG cell and the indication that the UE is only allowed to access CAG cells has been set in the subscription, then the AMF shall release the NAS signalling connection by triggering the AN release procedure.

NOTE 3:	When the UE is accessing the network for emergency service the conditions for AMF in clause 5.16.4.3 apply.

* * * * End of Changes * * * *