SA WG2 Meeting #S2-140E	S2-2005440

Aug 19 – Sep 01, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0182 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on data collection for statistics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specification it specifies that "Analytics information are either statistical information of the past events, or predictive information." To provide statistical information of the past events to the consumer NF, the NWDAF needs data in the past for analytics. However, so far the Exposure framework used by the NWDAF for data collection as specified in TS 23.502 [3] clause 4.15 does not allow the collection of data of the past. The Event Reporting Information and Expiry time only indicates time for data collection from the present to the future. In order to collect data of the past from the NF(s) for statistics, the NWDAF should be allowed to indicate a time period of the past for the data to be collected, using the Exposure framework. The NF(s) provides the stored data in the requested time period, if available, to the NWDAF for analytics. |  |  |  |  |  |  |  |  |
| FO |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification that the NWDAF may indicate a time period of the past for the data to be collected, during data collection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data of the past cannot be collected from the NF(s) and the NWDAF may fail to provide statistics information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.5 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

6.2.2.5	Usage of Exposure framework by the NWDAF for Data Collection

The NWDAF shall subscribe (and unsubscribe) to the Event exposure service from NF(s) reusing the framework defined in TS 23.502 [3] clause 4.15. This framework supports the possibility for the NWDAF to indicate / request:

-	Events-ID: one or multiple Event ID(s) defined in TS 23.502 [3] clause 4.15.1

-	Target of Event Reporting defined in TS 23.502 [3] clause 4.15.1: the objects targeted by the Events. Within a subscription, all Event ID(s) are associated with the same target of event reporting. In the case of NWDAF, the objects can be UE(s), UE group(s), any UE.

-	Event Filter Information defined in TS 23.502 [3] clause 4.15.1. This provides Event Parameter Types and Event Parameter Value(s) to be matched against.

-	A Notification Target Address and a Notification Correlation ID as defined in TS 23.502 [3] clause 4.15.1, allowing the NWDAF to correlate notifications received from the NF with this subscription.

-	Event Reporting Information described in TS 23.502 [3] Table 4.15.1-1. To collect data of the past for statistics, the NWDAF may indicate a time period of the past in the Event Reporting Information. The NF(s) provides the stored data in the requested time period, if available, to the NWDAF.

-	Expiry time as defined in TS 23.502 [3] clause 4.15.1.

The notifications from NFs/AFs contain on top of the Event being reported (and of dedicated information being reported for this event):

-	the Notification Correlation Information provided by the NWDAF in its request,

-	(when applicable to the event) the Target Id e.g. UE ID (SUPI and if available GPSI), and

-	a time stamp.

******************** End of Changes ********************