SA WG2 Meeting #140E	S2-2005654

19th Aug – 2nd Sep, 2020

Source:	Samsung

Title:	KI# 6, Sol #20: Update to remove ENs

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS/ Rel-17

Abstract of the contribution: This contribution proposes resolutions for the remaining Editor’s Notes in Solution #20.

- Discussion

This contribution proposes updates to resolve remaining ENs in Solution #20.

1) Editor’s Note: it is FFS whether the solution works for non-networked UAVs

The solution assumes that the USS/UTM should support an interface to 3GPP network to deliver the association information of a UAV UE to manage the PDU session for UAV and UAV-C communication. This requires that UTM should be informed whether the UAV is attached in 3GPP network to deliver the information to 3GPP network. For non-networked UAVs, how to manage the association and communication between them should be left out of scope of 3GPP standardisation. It is proposed to further clarify in the text that the solution applies when the UAVs are attached in 3GPP network.

2) Editor’s Note: it is FFS why such a re-registration is required and what is the relationship between the type of connectivity the UAV has with USS/UTM and the need to re-register.

The re-registration was proposed to update UAV’s status information at the USS/UTM. However, the exact message and procedure can be left to the UAS application. It is proposed to change the text to say that the UAV UE informs the USS/UTM of change of network connectivity when the UE completes registration in the 3GPP network. It is also proposed to add a new EN that this procedure be further affected by the conclusion on UUAA procedure.

3) Editor’s Note: it is FFS whether there is a requirement for why such a re-registration is required and what is the relationship between the type of connectivity the UAV has with USS/UTM and the need to re-register.

Similar change as above is proposed to clarify that just a notification of network connectivity change may be provided to the USS/UTM by a UAV UE if the UAV UE is already registered in USS/UTM.

4) Editor’s Note: it is FFS how the PDU Session Establishment is authorized and by what entity.

It is proposed to keep the EN until the conclusion on UUAA is achieved, and update the text based on the conclusion.

5) Editor’s Note: it is FFS which entity is the recipient of the counter party UE ID for use in connectivity control, and how it is used.

It is proposed to keep the EN until the conclusion on architectural decision on the interface between USS/UTM to 3GPP network is achieved, and update the text based on the conclusion. It is also proposed to add clarification that the counter party UE ID can be used by the SMF to control the PDU session establishment requested by a UAV UE.

6) Editor’s Note: how the CN addresses the USS/UTM, i.e. derives the information to contact the correct USS/UTM function, is FFS.

This EN is related to another key issue on UUAA. The solution proposes to reuse the DN authorization framework and should be compliant with the conclusion on UUAA. It is proposed the EN and updates the text based on conclusion achieved for the key issue.

7) Editor’s Note: it is FFS how the USS/UTM addresses the counter party UE towards the 3GPP system, i.e. what type of identity is used.

The solution proposed the use of application signalling to deliver the UAV UE ID to the counter party UE and not rely on the 3GPP network. This is already clarified in the current text and the EN is not needed. It is proposed to remove the EN.

8) Editor’s Note: whether step 15 and 16 apply depends on application layer design and are outside the scope of 3GPP.

It is proposed to remove the steps and add an additional text in the step 14 to clarify that the UAV UE and the UAV UE can start communication.

9) Editor’s Note: whether and how the solution works if the UAV and UAV controller are served by different PLMNs is FFS.

The solution does not rely on the same PLMN for the UAV and UAV controller UE. How to find the PLMN (3GPP network) is to be resolved by another EN. It is proposed to remove the EN when the conclusion on how to address the 3GPP network for a UAV UE is made.

10) Editor’s Note: it is FFS whether the procedure work when the UAV and the UAV controller are served by the same PLMN but by different AMFs and SMFs, or at least by different AMFs or different SMFs

Same as above. It is proposed to remove the EN when the conclusion on how to address the 3GPP network for a UAV UE is made.

11) Editor's note:	This clause describes impacts on existing entities and interfaces.

This is a general note for the clause. It is proposed to delete the EN.

12) Editor’s Note: It is FFS which 3GPP NF(s) shall manage and maintain the UAS association.

A similar EN is already captured in the procedure part and should be better discussed there rather than in the clause for impact analysis. It is proposed to remove the EN from the clause.

13) Editor’s Note It is FFS which 3GPP NF shall provide the interface to the USS/UTM.

The same EN is already captured in the procedure part and should be better discussed there rather than in the clause for impact analysis. It is proposed to remove the EN from the clause.

2.	Proposal

It is proposed to adopt the following changes in the TR 23.754.

*** Start of the change ***

6.20	Solution 20: UAV and UAVC Association and Connectivity Control Using Secondary Authorization

6.X.1	Introduction

This solution is for Key Issue #6: UAV controller and UAV association.

One of the UAS requirements is controlling the connectivity between UAV and UAV controller based on their association status. While the basic paring procedures between UAV and UAV controller may be performed using, e. g., on-site direct communication, their mutual association for UAS service needs to be authorized by the USS/UTM to enforce the permitted usage and related regulatory requirements.

A UAV UE or a UAV controller UE may communicate with the USS/UTM to request authorization of its association with the corresponding party, using 3GPP or non3GPP communication. Further, it can be assumed that the authorization of association may be performed during the UAS registration of the UAV UE or a UAV controller UE, or using a separate procedure defined for the purpose. The details of the authorization of association and the UAS registration protocol is defined between a UAV UE and the USS/UTM in the application layer and is out of scope of 3GPP.

When a UAV UE is attached in the 3GPP network, the registration and association status of the peer UAV or UAV controller UE is used in determining initiation of PDU session establishment procedure at the UAV (controller) UE as well as authorizing the requested PDU session at the network. That is, if the UAV UE starts PDU session establishment procedure for communication with a paired UAV controller that is not yet registered or associated, the PDU session should not be allowed by the network. Otherwise, the resources for the network and the UE will be wasted. When the UAV or UAV controller is registered and authorized for the association by the UTM, the counter party and the 3GPP network needs to be informed appropriately.

6.X.1	Functional Description

In the solution, a UAV and a UAV controller UE performs UAS registration in the USS/UTM to get authorized for UAS service. The UAS registration procedure may be performed before UE’s registration with the 3GPP network using a non3GPP networks connectivity. In the case, the UAV and the UAV controller are required to inform the USS/UTM of the change of network connectivity when they complete initial registration in the 3GPP network.

Editor’s Note: The procedure may need to be changed to align with the conclusion from discussion on UUAA procedure.

The UAV and the UAV controller UE provide information on its desired counter party UEs to the USS/UTM during the UAS registration.

A UAV or a UAV controller initiates a PDU session establishment with DN authorization towards the USS/UTM to request authorization of association with a counter party UE selected from the received list.

The USS/UTM evaluates and sends the allowed list of counter party UEs (among the available UEs) to the UAV and the UAV controller UE.

The USS/UTM informs the counter party UE of the association request and triggers the PDU session establishment by the counter party UE.

The USS/UTM delivers the IP addresses of the UAV and the UAV controller UE to the 3GPP network to request update of the PDU sessions.

The USS/UTM provides an activation signalling for UAS communication to the UAV and the UAV controller UE when it receives completion of resource setup from the network or the two UEs.

6.x.3	Procedures

Figure 6.x.3-1: UAV-UAV controller association and C2 communication setup

1.	A UAV Controller UE performs initial registration in the 3GPP network and establish a PDU session for communication between the UAV Controller and the USS/UTM.

2.	The UAV Controller UE performs UAS Registration with USS/UTM using application layer signalling to get authentication and authorization for the UAS service, if it has not registered in the USS/UTM. It is assumed that the UAV may also include information on its counter party (UAV controller) UEs in the Registration signalling message. The UAS registration is replaced by a notification of network connectivity change towards the USS/UTM if the UAV Controller UE is already registered in the USS/UTM.

NOTE: the details of such information exchange at application layer are out of scope of SA2.

3-4. The UAV UE performs the same procedures as described in step 1-2 for the UAV Controller UE.

5.	The  UAV UE initiates a PDU Session Establishment with DN authorization procedure to request USS/UTM authorization on association with the UAV Controller UE.

Editor’s Note: it is FFS how the PDU Session Establishment is authorized and by what entity. The text will be updated based on conclusion from discussion on UUAA.

6.	The UAV UE provides the UAV Controller UE ID to the USS/UTM during the authorization procedure. The authentication/authorization procedure may be simplified just to handle association authorization. Upon successful authorization, the USS/UTM provides the counter party UE ID to the network for use in connectivity control. The counter party UE ID can be used by the SMF to control the PDU session establishment requested by a UAV UE.

Editor’s Note: it is FFS which entity is the recipient of the counter party UE ID for use in connectivity control, and how it is used. The text will be updated based on architectural decision on the interface between USS/UTM to 3GPP network.

Editor’s Note: how the CN addresses the USS/UTM, i.e. derives the information to contact the correct USS/UTM function, is FFS. It is assumed to reuse the DN authorization framework and be compliant with the conclusion on UUAA. The text should be updated based on conclusion achieved for the key issue on UUAA.

7.	The UAV UE receives a PDU Session Establishment Response message from the 3GPP network for communication  with the selected counter party UE.

8	The 3GPP network provides the IP address of the UAV UE to the USS/UTM.

9.	The USS/UTM sends an application signalling to the counter party UE to trigger the PDU Session Establishment. The message includes the initiating UAV UE ID.

10.	Same DN authorization procedure as for step 5-8 is performed for the UAV Controller UE. The USS/UTM receives the IP address of the UAV Controller UE from the 3GPP network.

11-12. The USS/UTM sends a UAS Association Update message to the 3GPP network to deliver the IP
addresses of the UAV UE and the UAV controller UE.

13-14. The 3GPP network performs the PDU session modification procedure to update packet filter information for 
UAS communication. The UAC and UAV controller start UAS communication.

Editor’s Note: whether and how the solution works if the UAV and UAV controller are served by different PLMNs is FFS.

Editor’s Note: it is FFS whether the procedure work when the UAV and the UAV controller are served by the same PLMN but by different AMFs and SMFs, or at least by different AMFs or different SMFs.

6.x.4	Impacts on existing entities and interfaces

The following changes on UAV (Controller) UE and 3GPP Network are assumed to be required in the solution:

UAV UE and UAV Controller UE are required to:

- Support registration with USS/UTM when they are connected to 3GPP networks.

- Supports secondary authorization procedure for authorization of association with corresponding UAV (Controler) UE, during PDU Session Establishment for C2 connectivity.

3GPP Network is required to:

- Support secondary authorization procedure with USS/UTM

- Support an interface and protocols to perform UAS association update signalling

6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

*** End of the change ***