3GPP TSG-SA/WG2 Meeting #140e	S2-2004833r02

# Elbonia, Aug 19-Sep 1, 2020

Source:	ZTE

Title:	KI#1, Way forward on KI#1

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes discussion and evaluation for KI#1

1	Introduction

In TR 23.748 three 5GC Connectivity Models for Edge Computing are introduced.

There are total 24 solutions proposed for KI#1. Some solutions are targeting for single connectivity mode, some solutions are targeting for multiple connectivity modes. The evaluation should focus on the issues of each connectivity mode and check if those issues have been resolved, what is the alternative solutions for those issues and whether the proposed solutions can be complementary with each other. This paper provides analysis per each connectivity models and propose way forward.

1.1 Distributed Anchor Point model and Multiple Sessions model

For Distributed Anchor Point model and Multiple Sessions model, there are two issues to be resolved.

Issue 1: How can the UE establish a dedicated PDU Session towards the Local DN? There are two types of solution captured in the TR 23.748.

- URSP rule solution as described in sol#1/#17/21. The AF provides the URSP rule with the Location Criteria to the UE. The URSP rule is to associate the application and the DNN/S-NSSAI for the PDU Session towards the Local DN.

In this type of solution it is assume the UE shall perform URSP rule re-evaluation per each DNS query.

This solution also assumes different DNN or S-NSSAI combinations are deployed for different Local DNs and for central DN. This may not be the case all the time.

- DNAI solution as described in solution 7. The requested DNAI of the associated DNN and S-NSSAI combination are further provided to AMF. The AMF select the SMF based on the DNN and S-NSSAI combination, and the requested DNAI. This solution can work when separated DNN and S-NSSAI combination for each Local DN is not possible, and is complementary with the URSP rule solution in solution 1. This solution has no UE impact.

Proposal 1: Update solution 1 to add UE impacts and clarify that solution 1 is not applicable when common DNN/S-NSSAI are used for central DN and Local DN. This refers to S2-2004830.

Proposal 2: Update solution 7 to clarify this solution is complementary to URSP solution. This refers to S2-204831.

Considering the URSP is optional feature in the UE side and has not been widely deployed, we need a solution for UEs not supporting URSP feature. Therefore it is proposed to adopt both solution 1 and solution 7 for further work.

Proposal 3: Both solution 1 and solution 7 are taken as baseline for further work.

Issue 2: How can the DNS query be routed towards the DNS server? As this issue is common for all three connectivity models, it will be further discussed in clause 1.3.

1.2 Session Breakout model

For Session Breakout model, there are two issues to be resolved.

Issue 1: How can the ULCL be inserted for the PDU Session? There are two type of solutions captured in the TR 23.748.

- The ULCL/BP and Local PSA are inserted during the PDU Session establishment (pre-establishment). Per AF request the SMF insert the ULCL/BP and Local PSA for the PDU Session. This is Rel-16 feature and no open issue has been identified.

- The ULCL/BP and Local PSA are inserted after the PDU Session establishment (dynamic establishment). An LDNSR is introduced to trigger the insertion of ULCL/BP and Local PSA when it receives DNS query request(sol#3) or response(sol#22, sol#15). One open issue is the location of the LDNSR. The LDNSR can be within the SMF(sol#6, sol#22), or UPF(sol#4, sol#5), or AF(sol#3). One functionality of LDNSR is to notify the SMF to insert the ULCL/BP and Local PSA. As anyhow the PCF needs to authorize whether the local routing is allowed for the UE, it is proposed that the LDNSR acts as AF to send the traffic influence routing request to the PCF as per R16 and then the PCF update the PCC rules in SMF so the SMF can trigger the ULCL/BP and Local PSA insertion.

Proposal 4: It should be concluded where the location of the LDNSR is. In our view the LDNSR interacts with the 5GC as Application Function.

Issue 2: How can the DNS query be routed towards the DNS server? As this issue is common for all three connectivity model, it will be further discussed in clause 1.3.

1.3 How to route the DNS query

All three connectivity models have to consider how the DNS query is routed to the DNS server. There are several proposals in the TR 23.748.

option 1) The DNS query is routed towards the L-DNS. This includes the Anycast mechanism. The UE may be notified about the address of the L-DNS (sol#2), or the UE use the address of the C-DNS first and then the LDNSR or UPF can convert the target address to the L-DNS address(sol#22, sol#18, sol#20). One open issue is how to route the DNS query towards the L-DNS. There are two ways, either directly forwarding the DNS query towards the L-DNS(sol#22, sol#18, sol#20), in this case no normative work is needed, or indirectly forwarding the DNS query towards the L-DNS via the 5GC(sol#9, sol#22), in this case normative work is needed.

Proposal 5: It should be concluded whether the indirect forwarding the DNS query request towards the L-DNS is needed or not. In our view indirect forwarding the DNS query via 5GC is not needed.

option 2) The DNS query is routed towards the Central DNS. The ECS options is added in the DNS query request so the Central DNS can determines a suitable EAS IP address. One open issue is what the ECS option is. Different solutions are proposed, such as the ECS option can be IP address of the Local NAT in UPF (sol#4) , or IP address/prefix of the Local DN(sol#6, sol#22). Another open issue is how to determine the ECS option and which entity adds this ECS options in DNS query request, it could be the SMF (sol#6), or UPF(sol#4) or LDNSR(sol#22).

Proposal 6: It should be concluded what the ECS options is and how to determine the ECS options.

Proposal 7: It should be concluded which entity adds this ECS option in DNS query request. This may relate with proposal 4.

Solution 22 contains the following alternative solutions:

- Option 1: C-DNS mechanism: Adds ECS option by LDNSR in the DNS query message routed to the C-DNS.

- Option 2: L-DNS mechanism. There are further two alternatives to forward the DNS query to L-DNS:

2A: Forwards the UL DNS Query to an L-DNS by LDNSR.

2B: Forwards the UL DNS Query via 5GC in case of no connectivity between the central DN and the L-DN.

- Option 3: Forwards the UL DNS Query to local PSA based on the traffic filters on ULCL/BP.

Our analysis shows (see annex below) that most of the solutions on the DNS query can be merged into solution 22. Therefore we proposed to use solution 22 as basis for further work to resolve the proposal 4/5/6/7 above.

However solution 22 has several alternatives, it is difficult to combine them into single procedure. It is benefit to describe the alternatives in each separated procedure.

Proposal 8: Solution 22 is taken as baseline for further work.

Proposal 9: Solution 22 is updated to clarify the LDNSR location (refer to S2-204834)

Proposal 10: Solution 22 should be further updated to describe the alternative solutions in different procedures

2	Proposal

It is proposed to agree the proposals above.

It is proposed to agree the following update in 23.748 v040

# * * * * First change * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.X	Evaluation of Solutions for Key Issue #1 for DNS based solutions for Multiple PDU Sessions

For Multiple Session connectivity model, the Edge Computing applications may use PDU Session with SSC mode 3. The UE may establish multiple PDU sessions with same S-NSSAI and DNN to access local DN or central DN. Solution #7 proposes the requested DNAI list provided by Application Function is sent to AMF so it can be used for SMF/I-SMF selection during the establishment PDU session towards the local DN.

7.Y	Evaluation of Solutions for Key Issue #1 for DNS based solutions for Distributed Anchor Point

For Distributed Anchor Point model, the Edge Computing applications may use PDU Session with SSC mode 2. The UE may establish new PDU sessions with same S-NSSAI and DNN to access new local DN. Solution #7 proposes the requested DNAI list provided by Application Function is sent to AMF so it can be used for SMF/I-SMF selection during the establishment PDU session towards the local DN.

# * * * * Second change * * * *

9	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

9.x	Conclusions regarding solutions for Key Issue #1 for DNS based solutions

Solution 7 address how the PDU Session towards the local DN can be established for both Distributed Anchor Point connectivity model and Multiple PDU Sessions connectivity model when the URSP rule doesn't apply. Therefore it is recommended to be proceeded for normative work.

# * * * * End of change * * * *