SA WG2 Meeting #140e	S2-2004843r02

# Aug 19th – Sept 1st, 2020 ; Elbonia        		                					(revision of S2-2004843)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2047 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G-RG indicating over 5G NAS whether it supports ATSSS over EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Are all 5G-RG that support ATSSS always assumed to support ATSSS with a PDN Connection acting as user-plane resource associated with a MA PDU Session? There are scenarios when the 5G-RG can support the MA-PDU session only in 3GPP NR access and not on EPC access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In such case, the 5G-RG is not able to support MA-PDU on EPC and consequently: 1) If the MA PDU session is established in both 3GPP and non-3GPP accesses and the MA PDU has been handed-over of 3GPP Access from NR to EPC, the 5G-RG shall keep the MA-PDU only on Wireline and consider the second leg as “not available” (relaesing the correspoonding PDN connection), 2) If the MA PDU session is established only on wireline and the EPC becomes available, the 5G-RG will not/shall add EPC to the MA-PDU considering 3GPP access still as “not available” , |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | wrong behaviour of the 5G-RG and erroneous specification in BBF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.12.3.2	PDN Connections and Multi Access PDU Sessions

If the 5G-RG supports MA-PDU session over EPC,  when the 5G-RG wants to request a new PDN Connection in EPC and wants to use this PDN Connection as user-plane resource associated with a MA PDU Session:

-	The 5G-RG requests establishment of a new PDN Connection when the 5G-RG is registered via 3GPP access in EPS using PDN Connection Establishment procedure. The 5G-RG provides via PCO to PGW-C+SMF the following information:

-	An indication that the PDN Connection is requested to be associated with a MA PDU Session

-	The 5G-RG's ATSSS capabilities as described in TS 23.501 [2], clause 5.32.2 (i.e. whether the 5G-RG is capable of supporting the ATSSS-LL functionality, or the MPTCP functionality, or both)

-	The MME may select a PGW-C+SMF as described in TS 23.401 [24] and TS 23.502 [3], clause 4.11.0a.4.

NOTE 1:	The selection of PGW-C+SMF in the correct 5GC slice requires the same mapping between EPC and 5GC slices as required for single-access PDU sessions. In order to select an ATSSS capable PGW-C+SMF it is assumed that the operator deployment ensures that all PGW-C+SMF(s) configured to support the specific APN in this network slice are also capable to support ATSSS. There is however no assumption that all PGW-U+UPFs need to support ATSSS, since PGW-C+SMF can make a selection of PGW-U+UPF taking the multi-access properties into account.

-	The PGW-C+SMF determines based its capabilities whether the request can be accepted. The PCF decides whether the multi-access connectivity is allowed or not based on operator policy and subscription data, as described in TS 23.502 [3], clause 4.22.2. The PGW-C+SMF provides the following information in the PCO to the 5G-RG:

-	An indication whether the request for using the PDN Connection for MA-PDU Session is accepted or not.

-	If the 5G-RG has indicated that it is capable of supporting the MPTCP functionality and the PGW-C+SMF accepts to activate the MPTCP functionality, then the network provides MPTCP proxy information to the 5G-RG, as described in TS 23.501 [2], clause 5.32.2.

-	UE Measurement Assistance Information (as described in clause TS 23.501 [2], clause 5.32.2).

After the PDN Connection establishment:

-	If the 5G-RG registers to 5GC and wants to add W-5GAN user-plane resources, then the 5G-RG shall send a PDU Session Establishment Request over this access containing a "MA PDU Request" indication as described in TS 23.501, clause 5.32.2.

NOTE 2:	Adding the PDU Session connectivity and user plane resources over W-5GAN in 5GS allows the PGW-C+SMF to provide ATSSS rules to the UE.

When the 5G-RG wants to request a new MA PDU Session in 5GC/W-5GAN, the description in TS 23.501 [2], clause 5.32.2, applies. After the MA PDU Session establishment in 5GS/W-5GAN, the description in TS 23.501 [2], clause 5.32.2, applies with the following additions:

-	If the 5G-RG is registered to EPC and wants to add user-plane resources on 3GPP access over EPC, then the 5G-RG shall send a PDN Connection Establishment Request over this access containing a "handover" indication and include a "MA PDU Request" indication in the PCO.

-	When the 5G-RG deregisters from the EPC access (but remains registered on the 5GC access), the MME will notify the PGW-C+SMF that the PDN Connection is released, as described in TS 23.401 [24]. The SMF can then notify the UPF that the access type has become unavailable.

A 5G-RG that has an established MA-PDU session over non-3GPP access in 5GC and 3GPP access in EPS, may be able to use EN-DC for the 3GPP access leg.

If the 5G-RG supports MA-PDU session only over NR RAT type and not over EPC RAT type, the following applies,

-	If the MA PDU session is established over both 3GPP and non-3GPP accesses,  in case of HO of 3GPP Access from NR to  EPC,  the 5G-RG shall keep the MA PDU only on Wireline access and release the PDN connection over EPC/3GPP access.

NOTE 1: The release of the PDN connection over EPC/3GPP access indicated to the network (UPF) that the 3GPP access is not available. The 5G-RG is expected to add a 3GPP access to the MA PDU Session when 3GPP access over NR becomes available again

-	If the MA PDU session is established only on wireline RAT Type and the 3GPP EPC becomes available, the 5G-RG will not add a new PDN Connection on the EPC as user-plane resource associated with the MA PDU Session.

- If the 5G-RG is connected only over 3GPP EPC access with a PDN connection and the Wireline Access becomes available, the 5G-RG may handover the PDN Connection to wireline access as a Single Access PDU session as specified clause 7.6.4.1 or initiate the establishement a new a MA PDU session over on wireline access as specified in 4.12.2.

Depending on the RAT types supported by the 5G-RG, the PDN connection may also be handed over to 3GPP access in 5GC. For a 5G-RG supporting both E-UTRAN/EPC access and NG-RAN/5GC access, the user plane resources for 3GPP access may be moved between E-UTRAN/EPC access and NG-RAN/5GC access as described in TS 23.501 [2], clause 5.17.2. The PDU Session and User Plane resources active over W-5GAN are not affected by such inter 3GPP access RAT change.

NEXT CHANGE (2)

END OF CHANGES