SA WG2 Meeting #S2-140E	S2-2004845

Aug 19th – Sept 1st, 2020 ; Elbonia       	(revision of S2-2004845)

Title:	[draft] LS on IP address to GPSI translation

Reply to	S6-200947/ S2-2004784

Release:	Rel-17

Work Item:	EDGEAPP

Source:	Nokia [will be SA2]

To:	SA6, SA3

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 draft CR2385 and 23.502 draft CR2329.

1. Overall Description:

SA2 thanks SA6 for their LS. SA2 have discussed the incoming SA6 LS and would like to answer the following

- SA6 Question: Is there a core network supported method to provide a UE’s GPSI to an EAS directly?

SA2 Answer: No

- SA6 Question: Does the core network provide an API to translate a UE's IP address (private and public) to its GPSI, and if not, would it be feasible to provide such functionality in Rel-17 in order to address the SA6 requirement for UE IP address translation?

SA2 Answer: SA2 has endorsed the attached draft CR(s) for R17 in order to

Allow an AF to use an IP Address and a port to identify the UE that is the target of an API

The IP Address and port provided by the AF could correspond to a NATed The IP Address and port, if NAT (Network Addres and Port Translation) are deployed at N6 by the 3GPP network

About “Allowing NEF to provide the AF with a corresponding GPSI “, security concerns were raised and SA2 would support this feature only if agrred by SA3

- SA6 Question: While providing the functionality requested in bullet 1, is it feasible to provide application-specific GPSIs, to ensure that a single GPSI can not be used to track an end user's activity across applications (EASs), to protect end user privacy?

SA2 Answer: Please see answer to Q2.

2. Actions:

To SA6 group.

ACTION: 	SA2 kindly requests SA6 to take the answer into account.

To SA3 group.

ACTION: 	SA2 kindly requests SA3 to comment about security issues related with “Allowing NEF to provide the AF with a corresponding GPSI “.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#141e | Oct 12th – 23rd | Elbonia | any |
| SA2#141e | Nov 16th – 20th | Elbonia | any |