SA WG2 Meeting #S2-140E	S2-2005029

19 August - 2 September, 2020, Electronic	(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2404 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and claritication related to the choice of PDU Session when exchanging BMIC or PMIC between TSN AF and NW-TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NOTE4 in the BMIC information (Table 5.28.3.1-2) currently reads as follows: NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbor discovery on behalf of DS-TT. These IEs are deliverered via the procedures for the PDU session for the DS-TT port, while the other IEs of the table are deliverered via the procedures for any of the PDU sessions of the 5GS TSN bridge. The highlighted sentence seems to imply that, when DS-TT neighbor discovery information is carried in the BMIC, the choice of “hijacked” PDU Session depends on the actual DS-TT ports for which neighbor discovery is being reported. In general, TS 23.501 is lacking clarity on how the TSN AF or NW-TT choose a PDU Session for the execution of the following Stage 3 procedures defined in TS 24.519: TS 24.519 Figure 6.2.1.2.1: TSN AF-requested Ethernet port management procedure TS 24.519 Figure 6.2.2.2.1: NW-TT-initiated Ethernet port management procedure TS 24.519 Figure 6.3.1.2.1: TSN AF-requested Bridge management procedure Figure 6.3.2.2.1: NW-TT-initiated Bridge management procedure These procedures are executed in e2e manner between TSN AF and NW-TT, transparently for the intermediate nodes (SMF, PCF). The signalling content for these procedures is carried inside a PMIC (the first two procedures) or a BMIC (the last two procedures). Specifically, the following details are unclear: - Once the TSN AF has selected a PDU Session to “hijack” for the purpose of initiating the TSN AF-requested Ethernet port or Bridge management procedure (i.e. for performing NW-TT parameter subscription, setting NW-TT parameters or reading NW-TT parameters), does the TSN AF need to send all NW-TT PMICs or BMICs by “hijacking” that same session? - Furthermore, when the NW-TT initiates the NW-TT-initiated Ethernet port or Bridge management procedure (i.e. due to NW-TT parameter change) associated with a NW-TT parameter subscription request from the TSN AF previously received on a specific “hijacked” PDU Session, does the NW-TT need to send all NW-TT PMICs and BMICs by “hijacking” that same session? |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove NOTE4 in the BMIC information (Table 5.28.3.1-2). It is proposed to clarify the following: - TSN AF can use any PDU Session participating in the 5GS TSN bridge to initiate the TSN AF-requested Ethernet port or Bridge management procedure. - NW-TT can use any PDU Session participating in the 5GS TSN bridge to initiate the NW-TT-initiated Ethernet port or Bridge management procedure. - There is no linkage between the PDU Session used for subscription with the PDU Session used for notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary restriction for the choice of PDU Session when sending a BMIC containing DS-TT neighbor information. Lack of clarity related to the choice of PDU Session when exchanging BMIC or PMIC between TSN AF and NW-TT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 5.28.3.2	Transfer of port or bridge management information

Port management information is transferred transparently via 5GS between TSN AF and DS-TT or NW-TT, respectively, inside a Port Management Information Container (PMIC). Bridge management information is transferred transparently via 5GS between TSN AF and NW-TT inside a Bridge Management Information Container (BMIC). The transfer of port or bridge management information is as follows:

-	To convey port management information from DS-TT or NW-TT to TSN AF:

-	DS-TT provides a PMIC and the DS-TT port MAC address to the UE, which includes the PMIC as an optional Information Element of an N1 SM container and triggers the UE requested PDU Session Establishment procedure or PDU Session Modification procedure to forward the PMIC to the SMF. SMF forwards the PMIC and the port number of the related DS-TT Ethernet port to TSN AF as described in TS 23.502 [3] clause 4.3.3.2;

-	NW-TT provides PMIC(s) and/or BMIC to the UPF, which triggers the N4 Session Level Reporting Procedure to forward the PMIC(s) and/or BMIC to SMF. UPF selects an N4 session corresponding to any of the N4 sessions for this NW-TT. SMF in turn forwards the PMIC(s) and the port number(s) of the related NW-TT Ethernet port(s), or the BMIC, to TSN AF as described in TS 23.502 [3] clause 4.16.5.1.

NOTE:	There has to be at least one established PDU session for DS-TT port before the UPF can report PMIC/BMIC information towards the AF.

-	To convey port management information from TSN AF to DS-TT:

-	TSN AF provides a PMIC, MAC address reported for a PDU Session (i.e. MAC address of the DS-TT port related to the PDU session) and the port number of the Ethernet port to manage to the PCF by using the AF Session level Procedure, which forwards the information to SMF based on the MAC address using the PCF initiated SM Policy Association Modification procedure as described in TS 23.502 [3] clause 4.16.5.2. SMF determines that the port number relates to a DS-TT Ethernet port and based on this forwards the PMIC to DS-TT using the network requested PDU Session Modification procedure as described in TS 23.502 [3] clause 4.3.3.2.

-	To convey port or bridge management information from TSN AF to NW-TT:

-	TSN AF selects a PCF-AF session corresponding to any of the DS-TT MAC addresses for the related PDU sessions of this 5G TSN bridge and provides a PMIC(s) and the related NW-TT port number(s) and/or BMIC to the PCF. The PCF uses the PCF initiated SM Policy Association Modification procedure to forward the information received from TSN AF to SMF as described in TS 23.502 [3] clause 4.16.5.2. SMF determines that the included information needs to be delivered to the NW-TT either by determining that the port number(s) relate(s) to a NW-TT Ethernet port(s) or based on the presence of BMIC, and forwards the container(s) and/or related port number(s) to NW-TT using the N4 Session Modification procedure described in TS 23.502 [3] clause 4.4.1.3.

* * * * End of Change * * * *