SA WG2 Meeting #S2-140E	S2-2005127

19 Aug - 2 Sep, 2020, Elbonia

Source: 	Ericsson

Title: 	Evaluation and conclusion for KI#3

Document for: 	Approval

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This paper proposes the evaluation and conclusion for KI#3.

Discussion

This is a proposal for interim evaluation and interim conclusions for KI#3 that will be a starting point and need to be improved when Editor´s Notes in the different solutions, possible new solutions and overall conclusions are reached.

The evaluation criteria are taken from the proposal from rapporteur during the conference call for eNS_ph2.

Proposal

Add the following changes to TR 23.700-20.

*** BEGIN CHANGES ***

7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

7.x	Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

System architectural impacts

- Solution#21 introduces a new NF controls the Slice-MBR value without impacting other NFs other than SMF that needs to contact NSQ per each PDU session establishment and modification in addition to the PCF, UDM, DN-AAA. The need to define a new NF to control QoS parameters is not justified, since QoS control is part of the PCF functionality.

- All solutions impact the existing NF services to a certain extent, so this is not considered as a differentiation factor.

Signalling overhead

- Solution #21 adds some additional signalling for each PDU session establishment, and new triggers for the PDU session modification.

- Solution#13, #20, #22 does not add new signalling to the PDU session establishment.

Other aspects:

- In relation to measurements for counting of packets performed at the UPF according to 23.501, solution #22 may cause that packets in non-GBR QoS Flows are counted at the UPF and later dropped at the RAN,

8	Conclusions

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

8.x	Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

Editor's note:	This clause will provide some interim conclusion based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

*** END CHANGES ***