3GPP TSG-SA/WG2 Meeting #140E	S2-2005178

# Elbonia, 19 – 31 Aug. 2020, Electronic meeting

Source:	ZTE, T-Mobile USA

Title:	KI#7, New Sol: Steering the UE towards the 5G-AN during PDU session Establishment

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution is a resubmission to propose a new solution to resolve the key issue#7

# 1	Introduction

# 2	Proposal

It is proposed to agree the following changes in TR 23.700-40.

************* Start Changes *************

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| 11 | Handling maximum number of sessions using NF status | 2 |
| x | Steering the UE towards the 5G-AN during PDU session Establishment | 7 |

************* Next Changes *************

## 6.x	Solution #x: Steering the UE towards the 5G-AN during PDU session Establishment

### 6.x.1	Introduction

This solution addresses the key issue #7, especially on the following questions:

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

-	What information should be provided to the UE to select a proper 5G-AN and how it is sent to the UE.

-	Any RAN impacts shall be identified and alignment with RAN WGs shall be pursued if any impacts are identified.

### 6.x.2	High-level Description

The assumption of this solution is there could be multiple S-NSSAIs deployed within a single Tracking Area and all S-NSSAIs in the Allowed NSSAI are served by the 5G-AN in all the cells within the UE's Registration Area; in some cells certain S-NSSAIs are better served than in other cells. In this case the 5G-AN shall report all its supported S-NSSAI per Tracking Area towards the AMF in the NG Setup procedure. The AMF doesn’t need to know the frequency band supported by the 5G-AN.

During UE registration procedure, the AMF or NSSF generates Allowed NSSAI according to current specification, i.e. the AMF takes the requested NSSAI, the subscription and the S-NSSAI supported in the Registration Area, etc into account. The Tracking Area may support multiple S-NSSAIs correspond to different frequency bands, therefore it is possible that the Allowed NSSAI may include S-NSSAI corresponding to different frequency bands.

When the UE initiate PDU Session establishment procedure via a 5G-AN, the neighbour’s cell within the Registration Area is more appropriate for the requested NSSAI (e.g. according to operator’s policy, regulation etc.).  In this case when the 5G-AN receives N2 SM information which includes the requested S-NSSAI, it can steer the UE towards another 5G-AN which is more appropriate for the requested NSSAI. The mappings between S-NSSAI and frequency band are preconfigured in the 5G-AN and it is expected that the 5G-AN would know the supported S-NSSAI(s) in its neighbours’ cells.

In case of multiple PDU sessions are activated, all PDU Sessions need to be transferred to the target 5G-AN. It is up to 5G-AN implementation or operator policy to determine whether the UE should be transferred.

### 6.x.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.x.3-1: A high-level procedure of the solution

In step 6, the source 5G-AN determines the S-NSSAI of the PDU Session is better served by another 5G-AN and the source 5G-AN is possible to handover the UE target 5G-AN, then it trigger handover procedure and sends back the N2 SM message with a reject cause value indicating the request has been temporarily rejected due to handover procedure in progress.

In step 7 The AMF forwards the N2 SM message in Nsmf_PDUSession_UpdateSMContext Request towards the SMF associated with the PDU session. The SMF suspends the PDU Session Establishment procedure.

In step8, The SMF return Nsmf_PDUSession_UpdateSMContext Response(cause) to AMF. The cause value indicates the PDU Session Establishment procedure is suspended due to handover procedure in progress.

In step 9, the source 5G-AN selects the target 5G-AN which supports the S-NSSAI or determines the target 5G-AN is more suitable for the S-NSSAI, then it performs handover towards the target 5G-AN.

In step 10, After the handover if the AMF check the PDU Session Establishment procedure is suspended due to handover procedure in progress the AMF sends Nsmf_PDUSession_UpdateSMContext request to notify the SMF about the completeness of handover.

In step 11, the SMF returns the Nsmf_PDUSession_UpdateSMContext response

In step 12 the SMF resend the N2 SM message towards the target 5G-AN via AMF

In step 13 the PDU session establishment procedure continues via target 5G-AN.

### 6.x.4	Impacts on services, entities and interfaces

Editor's note:	This clause describes impacts to existing entities and interfaces.

5G-AN:

-	Preconfigure the mapping between S-NSSAI and corresponding frequency bands.

-	Trigger handover procedure to target 5G-AN which is more preferable for the PDU Session.

SMF：

-	Suspends the SM procedure when receives reject cause value indicating the request has been temporarily rejected due to handover procedure in progress

-	New cause value to notify the AMF that the PDU Session Establishment is suspended due to handover procedure in progress.

-	Continue the SM procedure after the mobility completion.

AMF:

-	Update the SMF after handover is completed and if PDU Session Establishment is suspended due to handover procedure in progress

UE, ,PCF:

-	No impacts.

### 6.x.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

*************** End Changes ***************