3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-2005291

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Tencent

Title:	KI#12, Solution#42: Update to Clarify Analytics ID and Clean Up

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2/Rel-17

Abstract: This contribution propose update to Solution #42 to clarify the Analytics ID and also clean the editor notes from TR template.

# Discussion

In SA2#139e, for KI#12, solution#42 about NWDAF-assisted RFSP Policy Configuration was captured in TR 23.776. In this pCR, some updates regarding to Analytics ID is added and also the editor notes from TR template is cleaned.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-91.

# * * * * First change * * * *

## 6.42	Solution #42: NWDAF-assisted RFSP Policy Configuration

### 6.42.1	Description

This is a solution for Key issue #12: NWDAF-assisted RFSP policy.

In general, the Rel-16 NWDAF architecture can be used as baseline to support NWDAF-assisted RFSP policy configuration. NWDAF needs to obtain related information from other NFs in 5GC and also OAM to perform analytics for RFSP policy. Figure 6.42-1 provides a functional diagram.

According to TS 23.501 [2], RFSP index needs to be provided to NG-RAN to derive cell reselection priorities for idle mode UEs and also redirecting active mode UEs to other RATs.

### 6.42.2	Input Data and Output Analytics

Potential inputs from NFs and OAM (optional)to NWDAF are listed as follows:

-	AMF

The RFSP index in use and authorized RFSP index (authorized RFSP index can also be obtained from PCF)

Editor`s note: Why the authorized RFSP index or in use value is needed by NWDAF is FFS.

Allowed NSSAI (to derive RFSP policy considering network slicing information)

-	UDM

The subscribed RFSP index

- Editor`s note: Why the subscribed RFSP index value is needed by NWDAF is FFS.

Slice related subscription information (this can also come from AMF)

-	PCF

The authorized RFSP index value.

Editor`s note: Why the authorized RFSP index value is needed by NWDAF is FFS.

-	SMF

Session related information which can help to derive RFSP policy regarding to redirecting the active mode UE to other RATs

Editor`s note: Which Session related information is related to RFSP policy and needs to be fetched from NG-RAN is FFS.

-	OAM

To acquire NG-RAN information related to RFSP policy information

Editor`s note: Which NG-RAN information is related to RFSP policy and needs to be fetched from NG-RAN is FFS.

It is not mandatory to acquire NG-RAN related radio access information and this solution should be workable without more NG-RAN information).  However, having such information can allow NWDAF to be aware how the RFSP index is mapped to the idle mode cell reselection configurations which may help NWDAF to perform more accurate analytics.

Figure 6.42.2-1: Functional diagram for NWDAF-assisted RFSP policy configuration

With the above listed inputs, NWDAF can perform analytics and provide analytics outcome. As analytics outcome, the NWDAF can generate statistics related information related to the RFSP index in-use and the predicted performance information regarding to the candidate RFSP index to be used. The NF consumer will decide whether to change the RFSP index. The NWDAF does not generate a RFSP index value as such since recommendations are out of the scope of REñ-17.

Editor`s note: What statistics related information related to the RFSP index in-use and the predicted performance information regarding to the candidate RFSP index to be used.

A new analytics ID is needed to support this solution.

Editor`s note: What analytics the NWDAF generates, which information contains is FFS.

The analytics outcome can be provided to NF consumer which can be AMF or PCF.

### 6.42.3	Procedures

Figure 6.42.3-1 provides the procedure with PCF as the NR consumer for NWDAF.

Figure 6.42.3-1: NWDAF provides analytics outcome to PCF

Figure 6.42.3-2 provides the procedure with AMF as NF consumer for NWDAF.

Figure 6.42.3-2: NWDAF provides analytics outcome to AMF

### 6.42.4	Impacts on services, entities and interfaces

PCF, AMF, UDM, SMF and OAM need to provide RFSP index related info to NDWAF.

NWDAF provides analytics outcome to PCF or AMF regarding to RFSP policy configuration.

A new Analytics ID needs to be specified to support statistics and prediction for RFSP configuration. Editor´s note are added to clarify this before reaching conclusions.

# * * * * End of changes * * * *