3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005379r09

Aug 19 – Sep 01, 2020, Elbonia	(revision of S2-200xxxx)

Source:	China Mobile

Title:	KI#14, Sol#46: Removing FFS on how to determine App ID for new PFD

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to remove the FFS on how to determine App ID for an IP 3-tuple in Solution#46.

# Discussion

During the SA2#139E meeting, a new solution i.e. Solution #46 is approved for the Key Issue#14: NWDAF-assisted application detection. In the solution, a FFS is left as follows:

Editor's note:	It is FFS how NEF assigns PFD ID and maps it with Application ID.

The reason for the FFS is that the Label Data (i.e. Application ID) is missing in the input data, so it is unknown which Application ID has the output analytics.

Table 6.46.3-1: Output for new application detection

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) |
| URL | the significant parts of the URL to be matched, e.g. host name defined in |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |

Usually, although the IP 3-tuple for an application is changeable, Traffic Signature/Feature for the application is unchangeable. So the NWDAF could collect the historical PFD information (including Application ID, historical IP 3-tuple, historical URL, historical Domain name information) from the NEF (PFDF) for one or multiple applications, then NWDAF could also collect the massive traffic information (as defined in Table 6.46.2-1) from SMF (UPF) for the historical IP 3-tuple(s). By correlating the data from NEF and SMF, a training dataset could be generated as follows:

[IP 5-tuple 1, Timestamp 1], Application 1, traffic information 1

[IP 5-tuple 2, Timestamp 2], Application 2, traffic information 2

[IP 5-tuple 3, Timestamp 3], Application 2, traffic information 3

…

[IP 5-tuple N, Timestamp N], Application M, traffic information N

M is the number of Applications, N is the number of samples

[IP 5-tuple 1, Timestamp 1], Application 1, traffic information 1

[IP 5-tuple 2, Timestamp 2], Application 2, traffic information 2

[IP 5-tuple 3, Timestamp 3], Application 2, traffic information 3

…

[IP 5-tuple N, Timestamp N], Application M, traffic information N

M is the number of Applications, N is the number of samples

In the training phase, based on the training dataset, an Application Classifier could be trained for all the Applications or one or multiple Traffic Signature/Feature could be extracted per Application ID.

In the inference phase, with the Application Classifier or the Traffic Signature/Feature, the Application ID for a new traffic information could be identified.

Furthermore, based on statistics in the plentiful inference result, new PFD information (including Application ID, new IP 3-tuple, new URL, new Domain name information) could be derived by the NWDAF and provisioned to the NEF. Then the new PFD information could be used to quickly detect the Application ID for a new traffic information.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91 v0.4.0.

# * * * * First change * * * *

## 6.46	Solution #46: NWDAF assisted new application detection

### 6.46.1	Description

This solution is for Key Issue #14: NWDAF-assisted application detection.

To differentiate an application traffic handling, the first step is to distinguish application traffic from other traffics. In 5G network, an application can be distinguished by a set of packet headers (SDFs) or application IDs. The most common approach to detect traffic is a set of packet headers, which usually contains source and destination address. The application ID can be used for referring the UPF's specific application detection filter. For managing detection information associated with application IDs, AF may provide PFDs according to clause 4.18 of TS 23.502 [3] to update application detection filter information. A PFD contents could contain flow description (service-side 3-tuple), URL, and domain name/protocol information. When an AF deliver PFD to NEF (PFDF), it will be distributed to SMFs and UPFs to enable flow detection according to clause 5.8.2 of TS 23.501 [2].

The objective of this solution is to automate application detection method using PFDs for newly/updated applications by extracting network traffic signature using the network data analytics. To realize this solution, one of the main features is 1) collect measurement of an application to extract statistical characteristics, 2) collect payload of packets to extract payload signature (such as domain name contained in the payload). The baseline usage of the generated analytics is to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect an application defined as TS 23.502 [3].

In details, usually, although the IP 3-tuple for an application is changeable, Traffic Signature/Feature for the application is unchangeable. Hence if the AF does not provide updated PFDs to the NEF (PFDF) in time for existing Application ID who has historical PFD information in the NEF (PFDF), the NWDAF could collect the historical PFD information (including Application ID, historical IP 3-tuple, historical URL, historical Domain name information) from the NEF (PFDF) for applications. The NWDAF could also collect the traffic information (as defined in Table 6.46.2-1) from SMF (UPF).

[IP 5-tuple 1, Timestamp 1], Application 1, traffic information 1

[IP 5-tuple 2, Timestamp 2], Application 2, traffic information 2

[IP 5-tuple 3, Timestamp 3], Application 2, traffic information 3

…

[IP 5-tuple N, Timestamp N], Application M, traffic information N

M is the number of Applications, N is the number of samples

[IP 5-tuple 1, Timestamp 1], Application 1, traffic information 1

[IP 5-tuple 2, Timestamp 2], Application 2, traffic information 2

[IP 5-tuple 3, Timestamp 3], Application 2, traffic information 3

…

[IP 5-tuple N, Timestamp N], Application M, traffic information N

M is the number of Applications, N is the number of samples

Based on PFD information from NEF and traffic information from SMF (UPF), new PFD information (including Application ID, new IP 3-tuple, new URL, new Domain name information) for the existing Application ID could be derived by the NWDAF and provisioned to the NEF as analytics output. Then the new PFD information could be used by the NEF to quickly detect the Application ID for a new traffic information.

The below is the principles of the solution:

-	A consumer (e.g. NEF) request to NWDAF to detect new applications, not matched with SDF or application IDs. The triggering can be configured by local configuration or OAM.

-	NWDAF collect SMF's data for user plane usage report and/or URL, Domain name part, and IP 3-tuples of packets from unknown application for default QoS flow (not matched with any SDF or application ID)

-	NWDAF analyse the collected data to generate unique packet signature, traffic characteristic, and traffic patterns.

-	The consumer stores PFD associated with an application ID to enable the detection of application traffic in the future (out of scope).

### 6.46.2	Input Data

NWDAF collects QoS flow related data from SMF for a specific S-NSSAI, DNN, and UE. This solution collects data for matched on default QoS flow, which no SDF or Application IDs are known. The detailed data are described in Table 6.46.2-1.

Table 6.46.2-1: input data to detect new application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI |  |  |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Start/end time | SMF | Start and end time of traffic detection in a QoS flow |
| Data volume | SMF | Measured data traffic volume (per UL/DL) in a QoS flow |
| Data duration | SMF | Measured average data traffic (packets) duration (per UL/DL) in a QoS flow |
| QoS flow Bit Rate | SMF | The observed bit rate (per UL/DL) in a QoS flow |
| Packet transmission | SMF | The observed average number of packet transmission (per UL/DL) in a QoS flow |
| IP 3-tuple | SMF | to Identify a service flow of the UE for an application |
| packet’s URL | SMF | URL is extracted from the inspected user plane packets in a QoS flow. |
| Packet’s Domain Name | SMF | The domain name is extracted from the inspected user plane packets in a QoS flow |
| Size of packets | SMF | Average size of packets in a QoS flow |
| PFD Information (NOTE 1) | NEF (PFDF) | PFD Information stored in the NEF (PFDF) as defined in clause 6.1.2.3.2, TS 23.503 [4] |
| > Application ID | NEF (PFDF) | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | NEF (PFDF) | Including protocol, server side IP address and port number |
| > URL | NEF (PFDF) | the significant parts of the URL to be matched, e.g. host name |
| > Domain Name | NEF (PFDF) | a Domain name matching criteria and information about applicable protocol(s) |
| NOTE: The PFD Information could be used to derive new PFD information for the Application ID in the PFD Information. |  |  |

Editor's note:	For collecting packets from UPF to SMF, the forwarding mechanism is defined in clause 5.8.2.5.2 of TS 23.501 [2]. It is FFS, how to direct the SMF forwarded packets to the NWDAF and how many packets are needed to be collected.

### 6.46.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.46.3-1.

The output analysis applies for both new applications that don’t have PFDs, and existing applications that have application IDs but with new 3-tuple and therefore needs new PFDs.

Table 6.46.3-1: Output for new application detection

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) |
| URL | the significant parts of the URL to be matched, e.g. host name defined in |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |
| NOTE: Application ID can be empty if there is no match between the existing Application ID and new PFDs. |  |

### 6.46.4	Procedures

For the procedure to extract the unique characteristic of application traffic such as traffic pattern, URL, and/or domain name related information. The procedure depicted in 6.46.4-1 shows new application detection analytics and store as PFDs by NEF.

Figure 6.46.4-1: A procedure to extract application characteristics

1.	NEF request to NWDAF to request application detection information. This request maybe be triggered by local configuration or OAM.

2.	A target UE establishes PDU session for communication

3.	NWDAF request to subscribe session related information and request to report URL, Domain name part, and IP 3-tuples of packets from unknown application.

Editor's note:	For collecting packets from UPF to SMF, the forwarding mechanism is defined in clause 5.8.2.5.2 of TS 23.501 [2]. It is FFS, how to direct the SMF forwarded packets to the NWDAF and how many packets are needed to be collected.

4.	A NWDAF extracts the application traffic characteristics.

5.	NWDAF notifies the analytic to the NEF with outputs defined in Table 6.46.3-1

### 6.46.5	Impacts on services, entities and interfaces

NWDAF:

-	Needs to provide a new analytic for application detection information.

-	Needs to process user plane data for extracting traffic characteristics.

SMF:

-	Needs to report URL, Domain name part, and IP 3-tuples of packets from unknown application to NWDAF

-	Needs to provide requested sessions statistics including data volume, data duration, QoS flow bit rate and packet transmission.

NEF (PFDF):

-	Supports to forward historical PFD information to NWDAF.

-	Supports to consume new PFD information from NWDAF.

# * * * * End of changes * * * *