3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005409

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, CATT

Title:	New architecture for MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution introduces a potential way forward for the architecture to be used with 5MBS.

1. Introduction

Currently there are two baseline architectures proposed for 5G MBS as depicted in TR 23.757 Annex A.1 and A.2 respectively, and based on each architecture, quite a few solutions have been proposed for different key issues. To facilitate the conclusion of the TR, it would be helpful if the two architectures could be consolidated into one.

An analysis of network functionalities in architecture 1 and 2 for supporting 5G MBS is shown in Table 1.1-1.

  - On the Functionalities of two architecture baselines

Table 1.1-1 Analyses of the functionalities of the architectural alternatives

|  | Baseline architecture 1 (as per TR description) | Baseline architecture 2 (as per TR description) | Converged architecture |
| --- | --- | --- | --- |
| UE | - Support of UE policy configuration extension to MBS. - Support of SM extension for MBS flows. - Signaling for joining MBS flow (via SM signaling or user plane IGMP Join). - MBS support at AS layer. | - Support 5G MBS services. - Reception of multicast data using PTM /PTP in RRC Connected. - MBS control client which can exchange Session Join, and Session Leave messages - Switch of MBS reception from Source to Target NG-RAN when Xn and N2 Handover execution phase commences. | - Support of UE policy configuration extension to MBS. - Reception of multicast data using PTM /PTP in RRC Connected. - Support of management of MBS flows. - Signaling for joining MBS flow - Switch of MBS reception from Source to Target NG-RAN when Xn and N2 Handover execution phase commences. - MBS support at AS layer (RAN responsibility). |
| NG-RAN | - Receive MBS flows via N3 and delivery over-the-air. - Switch between multicast and unicast delivery of MBS flows. - Configure UE for MBS flow reception at AS layer. (TBD how UE AS layer configuration of 5G MBS works) | - Support Point-to-Multipoint (PTM) and Point-to-Point (PTP) delivery of MBS media. - Control switching between PTM and PTP for best service quality and resource efficiency independently. - Support for MB Sessions and MBS related signalling on N2 with AMF and on N3 with UPF. - Transmission of multicast data using PTM /PTP in RRC Connected. - Support for MB Session info in Xn Handover request. - MB Session resource setup during Xn Handover preparation phase. - MB Session resource setup during Xn Handover execution phase. - New parameters in Path Switch messages. | - Management of MBS flows via N2 and N3 - Configuration of UE for MBS flow reception at AS layer (RAN responsibility). - Delivery of MBS data over PTM or PTP (RAN responsibility). - Control switching between PTM and PTP delivery per UE (RAN responsibility). - Support for MB Sessions during Xn Handover preparation phase and execution phase and at N2 Handovers (shared S2 & RAN responsibility) |
| (MB-)SMF | - Control of MBS transport, based on received MBS policies from PCF. - Configure UPF for MBS flows and for point to point or point-to-multipoint transfer. - Configure RAN for MBS flows and QoS information. - SM configuration at the UE for MBS flows. (An SMF may be used for both unicast and MBS) | - Control MB Sessions, signaling with AF (via NEF/MBSF), QoS control using PCF, - Provision MB Session information on request from AMF. - The PDU session(s) the UE(s) maintain for individual delivery of an MBS service may be associated with the MB Session managed by the MB-SMF. - MBS control function which can maintain the MBS signaling tree and respond to or send session requests to/from the NEF/MBSF, PCF and AF including allocation of TMGIs. - Optionally support deployments with SMFs dedicated to MBS services. - For 5GC Individual MBS traffic delivery, the SMF (for PDU Session) handles the request from AMF to receive 5MBS data from MB-UPF. | - Control MBS Sessions (including MBS transport and QoS) based on received MBS policies from PCF. - Signaling with AMF and NG-RAN via AMF for MBS Session and signaling distribution tree management. - Signaling with AF (via, e.g., NEF, MBSF) for, e.g., allocation of 5GC entry point and allocation of TMGIs. - Optionally support deployments with SMFs dedicated to MBS services. - Signalling with the PCF. In addition, it may support one or more of the following based on the selected solution(s): - Handling of session join (including UE authorization) and management of MB session context. - Configure (MB-)UPF for MBS flows and for individual or shared MBS traffic delivery (including switching). - For 5GC Individual MBS traffic delivery, the SMF (for PDU Session) handles the request from AMF to receive 5MBS data from MB-UPF. |
| (MB-)UPF | - Support of packet filtering of MBS flows, and delivery of MBS flows to RAN via point to point or point-to-multipoint N3. - Receive 5G MBS flow configuration from SMF. - Detect IGMP packets and notify SMF. (A UPF may receive both unicast and MBS flows) (I-UPF(s) may be used for the delivery of MBS flows from the UPF attached to N6 to NG-RAN; the N9 interface can be used for MBS traffic delivery) | - Support MBS UP function. - MBS user plane function with southbound IP Multicast tunnelling/transport interface and northbound interface with N6 tunnelling of MBS DL media streams. - On N3 interface, IP Multicast routing protocol for receiving IGMP/MLD join/leave to the multicast transport IP address ("LL MC"). - Support MBS operations on the N4 interface for MB Session setup and release. - For 5GC Individual MBS traffic delivery, the (PSA-)UPF is requested by SMF to forward MBS data stream onto PDU Session and to join or leave the MC tree of the (MB-)UPF source host. | - Support MBS operations on the N4 interface for MB Session setup and release. - Packet filtering for MBS flows. - Replication of MBS data packets to different RAN nodes or different UPF nodes. - Delivery of MBS flows to RAN or UPF via shared MBS traffic delivery over N3/N9. - On N3 interface, IP Multicast routing protocol for receiving IGMP/MLD join/leave to the multicast transport IP address ("LL MC"). - QoS enforcement (MFBR) and counting/reporting based on existing means. In addition, it may support one or more of the following based on the selected solution(s): - Detect IGMP packets and notify SMF of the IPmc address. - Delivery of MBS flows to RAN via individual MBS traffic delivery over N9. |
| AMF | - No requirement | - Select MB-SMF and be part of the signaling distribution tree. - MBS control function which can maintain the MBS signaling tree and control MBS resources in NG-RAN. - Support selection of SMFs that have MBS capabilities using NRF. - For 5GC Individual MBS traffic delivery, the AMF interacts with SMF of PDU Session so that the PSA UPF can start/stop receiving 5MBS data from MB-UPF. | - Signaling with UE, NG-RAN and (MB-)SMF for MBS Session management and for maintaining the MBS signaling tree. - Support selection of (MB-)SMFs that have MBS capabilities. - Interaction with SMF for the PSA UPF to start/stop receiving 5MBS data from MB-UPF for 5GC Individual MBS traffic delivery. - Handling of session join and management of MB session context. |
| PCF | - Support policies for Multicast services, including QoS parameters like 5QI, MBR, GBR. - Provide policy information regarding the MBS session to SMF. - Receive MBS service information from AF, directly (operator owned) or indirectly via NEF. | - PCF is enhanced to handle QoS for MB Sessions, e.g. to keep 5G QoS Profiles for MB Sessions or authorize QoS profiles for shared delivery. - Receive MBS service requirements from AF, indirectly via (MB-)SMF and NEF. In addition, it may support one or more of the following based on the selected solution(s): - For AF specific priority of group member, the PCF recives from AF and provides towards RAN PDU session policy control information including the MBS Session ID and MBS priority using existing procedure for PDU Session QoS update. | - Support QoS handling for MBS Session, including QoS parameters like 5QI, MFBR, GFBR - Provide policy information regarding the MBS session to (MB-)SMF for authorizing the related QoS profiles. - Receive MBS service information from AF, directly or indirectly In addition, it may support one or more of the following based on the selected solution(s): - For AF specific priority of group member, the PCF recives from AF and provides towards RAN PDU session policy control information including the MBS Session ID and MBS priority using existing procedure for PDU Session QoS update. |
| MBSF | MSF Control Plane (MSF-C) - Multicast service configuration. - MBS service level management. - xMB-C/MB2-C termination. - Codec configuration (if needed). | May be part of NEF or be deployed independently. - Support TMGI allocation or other MBS signaling for maintaining the signaling distribution tree and the service level management. - Provides an interface to the Application Function or content provider. - Interface to the MBSU. - Perform authorization of the UE to join the MB session. | - Interact with AF and MB-SMF to control MBSU, e.g. if encoding at MBSU is needed. In addition, it may support based on the selected solution(s): - Support TMGI allocation if this is not done by MB-SMF. |
| MBSU | MSF User Plane (MSF-U): - xMB-U/MB2-U termination. - Encoding of data at service layer. - Multicast service layer data packets delivery via N6. | - Handle the payload part to cater for the service level functions and management. - Anchor and forward MBS data in case regionalized deployment with multiple MB-UPFs | - Handling of encoding of MBS data. - Anchor and forward MBS data in case regionalized deployment with multiple MB-UPFs NOTE: The MBSF and the MBSU may be collocated or separated physical nodes. |
| NEF | - 5G MBS service exposure. - Negotiation of 5G MBS service with AF, including QoS, 5G MBS service area | - NEF is an existing NF, which provides interface to the AF - Maintain the MBS signaling tree using MBSF. Managing session requests from the AF. - Support selection of SMFs that have MBS capabilities based on NRF registration. | - Generally being 5G MBS service exposure and specifically providing an interface for 5MBS procedures. - Maintain the MBS signaling tree using MBSF. Managing session requests from the AF. - Negotiation of 5G MBS service with AF, including QoS - Support selection of SMFs that have MBS capabilities NOTE: If separated from the MBSU, the MBSF may be implemented in the NEF. |
| AF | - Support MBS service functions, negotiation with NEF for service exposure. | - Signalling with MB-SMF (via NEF/MBSF) - The AF may provide UE specific priority to the PCF. | - Support MBS service functions, negotiation with NEF for service exposure. - The AF may provide UE specific priority to the PCF. |

Based on the analytics, a merged architecture supporting 5G MBS is proposed as follows:

UE

NG-RAN

(MB-)UPF

AMF

(MB-)SMF

NEF

MBSF

MBSU

Data Network

Application Server

PCF

N6

gMB-U

(xMB-U/

MB2-U)

Ny

N4

N3

Uu

N2

Namf

Nsmf

Npcf

Nnef

Naf

Nmbsf

SMF

Nsmf

UDR

Nudr

AF

gMB-C (xMB-C/

MB2-C)

N6

Figure A.1.1-1: 5GS Architecture supporting MBS

2. Text Proposal

It is proposed to capture the following changes to TR 23.757.

# * * * * First change * * * * (All Text New)

Annex A:
Architecture alternatives

A.X	Baseline architecture X: 5G MBS system architecture

A.X.1	General

Based on the two baseline architectures for 5G MBS as depicted in Annex A.1 and A.2, a consolidated 5GS architecture supporting MBS is proposed.

A.X.2	Reference Architecture

A.X.2.1	General

Figure A.X.2.1-1 illustrates the 5GS architecture supporting MBS.

UE

NG-RAN

MB-UPF

AMF

MB-SMF

NEF

MBSF

MBSU

Data Network

Application Server

PCF

N6

gMB-U

(xMB-U/

MB2-U)

Ny

N4

N3

Uu

N2

Namf

Nmbsmf

Npcf

Nnef

Naf

Nmbsf

SMF

Nsmf

UDR

Nudr

AF

gMB-C (xMB-C/

MB2-C)

N6

N1

Figure A.X.2.1-1: 5GS Architecture supporting MBS

In Figure A.X.2.1-1, the SMF and UPF which have the roles to support MBS Sessions are named "MB-SMF" and "MB-UPF" respectively.

NOTE 1:	If individual delivery method is used, a UPF may be placed between MB-UPF and NG-RAN.

NOTE 2:	MBSU may be required depending on deployment.

NOTE 3:	The MB-SMF is an SMF extended with MBS functionality. An MB-SMF may be dedicated to MBS servies., The MB-UPF is an UPF extended with MBS functionality. A MB-UPF may be dedicated to MBS services.

NOTE 4:	The architecture using the reference point representation may be specified during normative phase or study phase.

NOTE 5:	gMB-C and gMB-U provide similar functionalities defined for xMB-C/MB2-C and xMB-U/MB2-U interfaces, respectively.

The 5G MBS System Architecture contains the following new service-based interfaces:

Nmbsmf:	Service-based interface provided by MB-SMF, could be part of Nsmf.

Nmbsf:	Service-based interface provided by MBSF, could be part of Nnef.

A.X.2.2	Functional entities

NOTE:	The enhancements of network functions covers commonalities of architecture alternative 1 and 2, and can be refined in normative work or study phase.

A.X.2.2.1	PCF

The PCF is used to provide policy rules for MBS services, and receive MBS service information from AF, directly or indirectly. The PCF performs the following functions to support MBS:

-	Support QoS handling for MBS Session, including QoS parameters like 5QI, MFBR, GFBR

-	Provide policy information regarding the MBS session to SMF for authorizing the related QoS profiles.

-	Receive MBS service information from AF, directly or indirectly.

A.X.2.2.2	MB-SMF

The MB-SMF is used for MBS session management, management of the signalling distribution tree and control of MBS transport, including configuring the MB-UPF and RAN (via AMF) of NG UP tunnel and QoS control for MBS flows transport based on the policy rules for MBS services from PCF or local policy. The MB-SMF performs the following functions to support MBS:

-	Control MBS Sessions (including MBS transport and QoS) based on received MBS policies from PCF or local policy.

-	Signaling distribution tree management and signalling with NG-RAN, MB-UPF and AMF for MBS Session management.

-	Signaling for, e.g., allocation of 5GC user plane entry point and allocation of TMGIs if needed.

A.X.2.2.3	MB-UPF

The MB-UPF is used for delivery of MBS flows to RAN (or PSA-UPF) via shared tunnel and QoS enforcement for MBS services. The UPF performs the following functions to support MBS:

-	Packet filtering for MBS flows.

-	Distribution of MBS data packets to different RAN nodes (or PSA-UPF nodes).

-	Delivery of MBS flows to RAN via shared MBS traffic delivery over N3.

-	QoS enforcement (MFBR) and counting/reporting based on existing means.

A.X.2.2.4	AMF

The AMF performs the following functions to support MBS:

-	Selection of MB-SMFs that have MBS capabilities.

-	Signaling with NG-RAN and MB-SMF for MBS Session management.

-	Signaling with UE, NG-RAN and MB-SMF for maintaining the MBS signaling tree.

A.X.2.2.5	NG-RAN

The NG-RAN is used to receive MBS flows via shared N3 tunnel and deliver MBS flows to the UEs using PTP or PTM delivery method. The NG-RAN performs the following functions:

-	Management of MBS flows via N2 and N3.

-	Configuration of UE for MBS flow reception at AS layer.

-	Delivery of MBS data using PTM or PTP.

-	Control switching between PTM and PTP delivery per UE.

A.X.2.2.6	UE

The UE needs to support the MBS operations and receive MBS flows from NG-RAN. The UE performs the following functions:

-	Reception of multicast data using PTM /PTP.

-	Support of management of MBS flows.

-	Signaling for joining MBS flow.

-	MBS support at AS layer.

A.X.2.2.7	AF

The AF requests MBS service to the 5GC, including:

-	provide MBS service information.

-	interact with PCF for MBS policy control.

-	negotiation with NEF for MBS related network service exposure.

-	instruct MBS session operation with NEF/MBSF if needed.

A.X.2.2.8	NEF

The NEF supports 5MBS service exposure and providing an interface to AF for 5MBS procedures including service provisioning, MBS session management, maintaining the signalling distribution tree for each MBS session, selection of SMFs that have MBS capabilities, QoS management, etc.

A.X.2.2.9	MBSF

The MBSF performs the following functions:

-	Interact with AF and MB-SMF for MBS session operations.

-	Trigger MBS session management to MB-SMF for MBS transport if needed.

-  Control MBSU.

In addition, it may support the following based on the selected solution(s):

-	Support TMGI provisioning if this is not done by (MB-)SMF.

A.X.2.2.10	MBSU

The MBSU performs the following functions:

-	Handling of encoding of MBS data.

-  Media anchor for MBS data traffic if needed.

NOTE: The MBSF and the MBSU may be collocated or deployed separately.

# * * * * Second change * * * *

8	Conclusions

INTERIM CONCLUSION: Architectural option X "5G MBS system architecture" as described in Annex A.X is chosen as the baseline architecture for 5G MBS.

# * * * * End of changes * * * *