3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005409r14

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, CATT

Title:	New architecture for MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution introduces a potential way forward for the architecture to be used with 5MBS.

# 1. Introduction

The 5MBS study started with two quite different baseline architectures proposed for 5G MBS as depicted in TR 23.757 Annex A.1 and A.2 respectively, however, with the solution evolving, the needed network function/network entities have become quite similar, although the architecture in A.1 has still not been updated accordingly with the main solution for architecture in A.1 (i.e. Sol#3 in section 6.3) has evolved. To facilitate the evaluation of the TR solutions, it may be worth looking at the general principles and the required functionalities.

An analysis of network functionalities per functional entity for architecture 1 and 2 with basic solutions for supporting 5G MBS is shown in Table 1.1-1 in clause A.X.2.

Please note that architectures are typically frameworks with little or no functionality in themselves. Therefore, architectures cannot successfully be evaluated or compared without any solutions. Functionalities need to be added to an architecture e.g. by studying certain use cases and providing solutions that realize these use cases. The table in A.X.2 therefore shows respective baseline architecture together with a set of basic solutions.

# 2. Discussion

Below is a set of high-level questions on the general principles and functionalities that should be asked in order to for evaluation to be performed.

| Principles/required functionalities | Comment |
| --- | --- |
| 1. Is the proposed architecture able to cater for use cases in future, e.g. to support Receive Only UE, to support full-service mode? | Although Objective B was scoped out according to SID SP-200092, the 5MBS architecture should be able to cater for the use cases in Objective B: B) In addition, the objectives to support broadcast architecture for TV Video and Radio Services in 5GS (including linear TV, Live, smart TV, and managed and OTT content, radio services) contain the following components: - Shared broadcast services amongst multiple providers. - Allow Video/TV/Radio services to devices with no subscription (e.g., Free to air TV) or with 3rd party content provider subscription only. - Provision/service to receive-only devices such as fixed TVs or vehicular receivers. - Enabling combination of broadcast services with unicast services from same provider (similar to DVB-I, HbbTV, etc.) |
| 2. What should be used for multicast group ID? | Multicast IP address and DNN/S-NSSAI vs TMGI (as in eMBMS) |
| 3. For Multicast communication, how does UE join? | Alt1: NAS join only Alt 2: Mandatory NAS join and optional UP Alt 3: Mandatory NAS and UP join |
| 4. Is 5GC level authorization required? | When UE joins (i.e. for multicast communication), is there any need to do 5GC authorization? (in another word, can it be assumed that application layer interaction (e.g. GC1 in Public Safety) is sufficient?) If 5GC authorization is required, how does the 5GC get the info if the UE is authorized or not? Any implication? |
| 5. 5GC switching for service continuity | Can we assume that 5GC switching to individual 5GC delivery is applicable only when the RAN does not support 5MBS but the UE still supports 5MBS? In another word, for UEs not supporting 5MBS, they should not get involved in the 5MBS procedure at all. |
| 6. PTP-PTM switching in RAN | Is there any need for the 5GC to provide “assistance info” to NG-RAN? If yes, what info may be able to assist RAN switching? |
| 7. Inactivity supervision in MBS packet in RAN (depends on RAN) | If there is no MBS data for a period of time, will NG-RAN request to release the N3 tunnel if point-to-point is used? Or Leave the multicast tree? Is it necessary to have similar inactivity supervision in MB-UPF |
| 8. Inactivity supervision in MBS packet in MB-UPF | Is it necessary? If yes, why? |

# 3. Text Proposal

It is proposed to capture the following changes to TR 23.757.

# * * * * First change * * * * (All Text New)

Annex A:
Architecture alternatives

# A.X	Evaluation of 5G MBS system architectures

## A.X.1	General

Based on the two baseline architectures for 5G MBS as depicted in Annex A.1 and A.2, for evaluation purposes, a list of functionalities of respective baseline architecture divided per functional entity is proposed.

NOTE:	Please note that architectures are typically frameworks with little or no functionality in themselves. Therefore, architectures cannot successfully be evaluated or compared without any solutions. Functionalities need to be added to an architecture e.g. by studying certain use cases and providing solutions that realize these use cases. The table in A.X.2 below therefore shows respective baseline architecture together with a set of basic solutions.

## A.X.2	On the Functionalities of two architecture baselines

Table 1.1-1 Analyses of the functionalities of the architectural alternatives

|  | Baseline architecture 1 (as per TR descriptions in A.1, 6.3) | Baseline architecture 2 (as per TR descriptions in A.2, 6.2, 6.11, 6.12) |
| --- | --- | --- |
| UE | - Support of UE policy configuration extension to MBS. - Support of SM extension for MBS flows. - Signaling for joining MBS flow (via SM signaling or user plane IGMP Join). - MBS support at AS layer. | - Support 5G MBS services. - Reception of multicast data using PTM /PTP in RRC Connected. - MBS control client which can exchange Session Join, and Session Leave messages - Switch of MBS reception from Source to Target NG-RAN when Xn and N2 Handover execution phase commences. |
| NG-RAN | - Receive MBS flows via N3 and delivery over-the-air. - Switch between multicast and unicast delivery of MBS flows. - Configure UE for MBS flow reception at AS layer. (TBD how UE AS layer configuration of 5G MBS works) | - Support Point-to-Multipoint (PTM) and Point-to-Point (PTP) delivery of MBS media. - Control switching between PTM and PTP for best service quality and resource efficiency independently. - Support for MB Sessions and MBS related signalling on N2 with AMF and on N3 with UPF. - Transmission of multicast data using PTM /PTP in RRC Connected. - Support for MB Session info in Xn Handover request. - MB Session resource setup during Xn Handover preparation phase. - MB Session resource setup during Xn Handover execution phase. - New parameters in Path Switch messages. |
| (MB-)SMF | - Control of MBS transport, based on received MBS policies from PCF. - Configure UPF for MBS flows and for point to point or point-to-multipoint transfer. - Configure RAN for MBS flows and QoS information. - SM configuration at the UE for MBS flows. (An SMF may be used for both unicast and MBS) | - Control MB Sessions, signaling with AF (via NEF/MBSF), QoS control using PCF, - Provision MB Session information on request from AMF. - The PDU session(s) the UE(s) maintain for individual delivery of an MBS service may be associated with the MB Session managed by the MB-SMF. - MBS control function which can maintain the MBS signaling tree and respond to or send session requests to/from the NEF/MBSF, PCF and AF including allocation of TMGIs. - Optionally support deployments with SMFs dedicated to MBS services. - For 5GC Individual MBS traffic delivery, the SMF (for PDU Session) handles the request from AMF to receive 5MBS data from MB-UPF. |
| (MB-)UPF | - Support of packet filtering of MBS flows, and delivery of MBS flows to RAN via point to point or point-to-multipoint N3. - Receive 5G MBS flow configuration from SMF. - Detect IGMP packets and notify SMF. (A UPF may receive both unicast and MBS flows) (I-UPF(s) may be used for the delivery of MBS flows from the UPF attached to N6 to NG-RAN; the N9 interface can be used for MBS traffic delivery) | - Support MBS UP function. - MBS user plane function with southbound IP Multicast tunnelling/transport interface and northbound interface with N6 tunnelling of MBS DL media streams. - On N3 interface, IP Multicast routing protocol for receiving IGMP/MLD join/leave to the multicast transport IP address ("LL MC"). - Support MBS operations on the N4 interface for MB Session setup and release. - For 5GC Individual MBS traffic delivery, the (PSA-)UPF is requested by SMF to forward MBS data stream onto PDU Session and to join or leave the MC tree of the (MB-)UPF source host. |
| AMF | - No requirement | - Select MB-SMF and be part of the signaling distribution tree. - MBS control function which can maintain the MBS signaling tree and control MBS resources in NG-RAN. - Support selection of SMFs that have MBS capabilities using NRF. - For 5GC Individual MBS traffic delivery, the AMF interacts with SMF of PDU Session so that the PSA UPF can start/stop receiving 5MBS data from MB-UPF. |
| PCF | - Support policies for Multicast services, including QoS parameters like 5QI, MBR, GBR. - Provide policy information regarding the MBS session to SMF. - Receive MBS service information from AF, directly (operator owned) or indirectly via NEF. | - PCF is enhanced to handle QoS for MB Sessions, e.g. to keep 5G QoS Profiles for MB Sessions or authorize QoS profiles for shared delivery. - Receive MBS service requirements from AF, indirectly via (MB-)SMF and NEF. In addition, it may support one or more of the following based on the selected solution(s): - For AF specific priority of group member, the PCF recives from AF and provides towards RAN PDU session policy control information including the MBS Session ID and MBS priority using existing procedure for PDU Session QoS update. |
| MBSF | MSF Control Plane (MSF-C) - Multicast service configuration. - MBS service level management. - xMB-C/MB2-C termination. - Codec configuration (if needed). | May be part of NEF or be deployed independently. - Support TMGI allocation or other MBS signaling for maintaining the signaling distribution tree and the service level management. - Provides an interface to the Application Function or content provider. - Interface to the MBSU. - Perform authorization of the UE to join the MB session. |
| MBSU | MSF User Plane (MSF-U): - xMB-U/MB2-U termination. - Encoding of data at service layer. - Multicast service layer data packets delivery via N6. | - Handle the payload part to cater for the service level functions and management. - Anchor and forward MBS data in case regionalized deployment with multiple MB-UPFs |
| NEF | - 5G MBS service exposure. - Negotiation of 5G MBS service with AF, including QoS, 5G MBS service area | - NEF is an existing NF, which provides interface to the AF - Maintain the MBS signaling tree using MBSF. Managing session requests from the AF. - Support selection of SMFs that have MBS capabilities based on NRF registration. |
| AF | - Support MBS service functions, negotiation with NEF for service exposure. | - Signalling with MB-SMF (via NEF/MBSF) - The AF may provide UE specific priority to the PCF. |

# * * * * End of changes * * * *

N4

UPF

UE

NG-RAN

MB-UPF

AMF

MB-SMF

NEF

MBSF

MBSU

Data Network

Application Server

PCF

N6

gMB-U

(xMB-U/

MB2-U)

Ny

N4

N3

Uu

N2

Namf

Nmbsmf

Npcf

Nnef

Naf

Nmbsf

SMF

Nsmf

UDR

Nudr

AF

gMB-C (xMB-C/

MB2-C)

N6

N1