3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005413

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei (Rapporteur)

Title:	KI#1: Principles for categorization of Solutions for KI1

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This document is trying to find the common aspects of the solutions and achieve the interim conclusions.

# 1. Introduction/Discussion

# This document captures specific principles for 5G MBS.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

# * * * * First change * * * *

# 7	Evaluations

## 7.X	Evaluation for Key Issue #1: MBS session management

Evaluation should be based on the following principles for this study:

- For user join, CP (NAS) based join/leave shall be supported by UE and network.

Editor’s Note: It is FFS if the network and UE in addition may support multicast session join/leave operation via UP e.g. IGMP Join/Leve.

- For N3 transport of the shared delivery method, GTP-U tunnelling using a transport layer IP multicast method shall be supported and with support for QoS.

- For N3 transport of the shared delivery method, GTP-U tunnelling using a shared N3 (GTP-U) Point-to-Point tunnel shall be supported and with support for QoS.

- For N6 connectivity, solutions shall support tunnelling of MBS data towards the external network, and may support untunnelled MBS data (IP multicast based) towards the external network.

- Solutions of KI#1 shall consider UE Authorization for multicast.

- OAt least one QoS Flow per MBS Session shall be supported.

Editor’s Note: Whether to limit to one QoS Flow per MBS Session is FFS.

- There shall be a 1-to-1 mapping between MBS Session and N3 tunnel, though different multicast N3 tunnels may use the same transport multicast destination address (e.g., some popular media content may need to be transported to all RAN nodes or the same set of RAN nodes so the same multicast destination address can be used for the related N3 tunnels, up to the choice of vendor implementation and operator deployment considerations).

- Both 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be standardized for MBS data delivery within NR.

- The network shall only use the 5GC individual MBS Traffic delivery method for a UE served by a RAN node not supporting 5G MBS.

- An MB Session shall be uniquely identified by an ID.

- Once a MB Session is stopped, the network may release all the NG-RAN resources for a MBS Session.

- It shall be possible to restart an inactive MBS Session that has been stopped and NG-RAN resources released but still has its ID and resources in 5GC (similarly to moving a PDU Session from CM-IDLE to CM-CONNECTED).

- The network shall be able to prepare and start multicast traffic transmission for a MBS session after MBS service is requested and started from AF.

- The network shall support selection of MB-SMF by NEF or MBSF downstream at session start. The network shall support selection of MB-SMF by AMF upstream at session join.

# * * * * End of changes * * * *