3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005413

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei (Rapporteur)

Title:	KI#1: Principles for categorization of Solutions for KI1

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This document is trying to find the common aspects of the solutions and achieve the interim conclusions.

# 1. Introduction/Discussion

# This document captures specific principles for 5G MBS.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

# * * * * First change * * * *

# 7	Evaluations

## 7.X	Evaluation for Key Issue #1: MBS session management

Evaluation should be based on the following principles for this study:

- For multicast solutions, signalling from the UE to the network to join a multicast session shall be supported by UE and network. Join/leave operation via CP (NAS) signalling shall be supported.

Editor’s Note: It is FFS if the network and UE shall support multicast session join/leave operation via UP e.g. IGMP Join/Leve.

- For N3 transport of the shared delivery method, GTP-U tunnelling using a transport layer IP multicast method or shared N3 (GTP-U) Point-to-Point tunnel shall be supported with support for QoS.

- For N6 connectivity, solutions shall support tunnelling of MBS data towards the external network and may support untunnelled MBS data (IP multicast based) towards the external network.

- One QoS Flow per MBS Session shall be supported. Multiple QoS Flows per MBS Session may be supported.

- Both 5GC Shared MBS traffic delivery method (within RAN node supporting 5G MBS) and 5GC Individual MBS traffic delivery method shall be standardized for multicast data delivery. The latter method to be optional in the standard.

- ..

- The network shall be able to prepare and start multicast traffic transmission for a MBS session after MBS service is started.

- The network shall support selection of MB-SMF or SMF (depending on solution) at session start. The network shall support selection of MB-SMF or SMF (depending on solution) by AMF at session join.

- For N3 transport of the 5GC shared MBS delivery method, there shall be 1-1 mapping between MBS Session and GTP-U tunnel at a RAN node side.

# * * * * End of changes * * * *