3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005415

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Title:	[DRAFT] LS on RAN impact of FS_5MBS Study

Release:	Release 17

Work Item:	FS_5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Draft TR 23.757 v0.4.0

1. Overall Description:

SA2 has been discussing on Rel-17 FS_5MBS solutions since SA2 #135 meeting. Based on the agreements made at the SA2 #140 e-meeting, SA2 agreed to send the updated version of the Technical Report TR 23.757 to RAN to facilitate work of RAN.

SA2 would like to kindly inform RAN2 and RAN3 the following interim agreements in SA2:

-	SA2 assumes the RAN is aware of QoS requirements for an MBS Session.

-	[Depends on the outcome of SA2] SA2 agrees the Architecture X in section A.X is used as the baseline architecture.

-	[Depends on the outcome of SA2] SA2 agrees that shared N3 tunnel will be used for DL MBS data delivery, both “multicast transport based on that NG-RAN sends IGMP/MLD Join to an MB-UPF” and “setting up point-to-point unidirectional N3 tunnel from to MB-UPF to NG-RAN” could be used.

-	[Depends on the outcome of SA2] SA2 agrees one N3 tunnel is associated with only one MBS session.

SA2 would like to kindly ask RAN2 and RAN3 the following questions:

-	Some solutions assume the UEs receive MBS data in CM-CONNECTED state. The RAN node maintains UE context and MBS context, to enable the switching between PTP and PTM for MBS data transmission. AMF also triggers the RAN nodes to page the UEs in CM-IDLE state. SA2 would like to confirm with RAN2 the feasibility of this design.

-	Some solutions assume the UEs receive MBS data in CM-IDLE state. SA2 would like to request RAN2 to inform SA2 the progress for supporting the UEs receiving MBS data in CM-IDLE state, and kindly inform SA2 if any.

-	Some solutions provide service continuity based on MBS data forwarding at the Xn/N2 handover procedure from S-RAN to the T-RAN. SA2 would like to confirm with RAN3 the feasibility, and ask RAN2 and RAN3 to consider the way of handling the forward MBS data at T-RAN.

2. Actions:

To RAN2 and RAN3 group.

ACTION:

SA2 respectfully requests RAN2 and RAN3 to take the above information into account and provide feedback, if necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141E		October 12 – 23, 2020			Elbonia

TSG-SA2 Meeting #142E		November 16 – 20, 2020			Elbonia