3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005415

Elbonia, August 19 – September 2, 2020	(revision of S2-200xxxx)

# Title:	[DRAFT] LS on RAN impact of FS_5MBS Study

# Release:	Release 17

# Work Item:	FS_5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TR 23.757 v0.4.0 (and available shortly after SA2#140E TR 23.757 v0.5.0)

1. Overall Description:

SA2 has been discussing on Rel-17 FS_5MBS solutions since SA2 #135 meeting in TR 23.757.

SA2 would like to kindly inform RAN2 and RAN3 the following interim agreements in SA2:

-	SA2 will provide means to provide QoS requirements for an MBS Session to RAN nodes.

-	[Depends on the outcome of SA2#140E] SA2 agrees the Architecture X in section A.X is used as the baseline architecture.

-	[Depends on the outcome of SA2#140E] SA2 agrees that for each MBS session an own shared N3 tunnel will be used for DL MBS data delivery from MB-UPF to NG-RAN nodes. This tunnel can use either IP multicast transport (NG-RAN sends IGMP/MLD Join/Leave to a multicast router) or point-to-point unidirectional N3 tunnels from MB-UPF to NG-RAN nodes.

SA2 would like to kindly ask RAN2 and RAN3 the following questions:

- In order to save power, SA2 sees the benefit for the UE transitioning into CM-IDLE/RRC-INACTIVE after having the transmission resource configuration for a MBS session and user inactivity is observed e.g. no more multicast data. 	SA2 would appreciate RAN2 and RAN3 feedback on this topic.

- SA2 agreed that the UE shall be able to receive MBS session data while in CM-CONNECTED/RRC-CONNECTED.  In addition SA2 would like to ask whether the UE can receive MBS session data while in CM-IDLE/RRC-INACTIVE. SA2 would appreciate RAN2 and RAN3 feedback on this topic.

- Depending on answer in Q2, SA2 would like to ask whether 5G CN can trigger a notification to CM-IDLE/RRC-INACTIVE UEs before PTM/PTP delivery can commence at start of a MBS Session data delivery. SA2 has two options for the notification: a) group paging, and b) legacy session start notification. SA2 asks RAN2 and RAN3 on feedback for the RAN aspects of the aforementioned approaches.

- Some of the proposed Xn/N2 handover solutions in the SA2 study propose MBS data forwarding from S-RAN to the T-RAN to address the potential data loss in case of UE moving to a T-RAN supporting 5MBS that may or may not have the on-going MBS session. SA2 would appreciate RAN2 and RAN3 feedback on the RAN aspects of the forwarding consideration at Xn/N2 handover.

2. Actions:

To RAN2 and RAN3 group.

ACTION:

SA2 respectfully requests RAN2 and RAN3 to take the above information into account and to answer the questions raised by SA2.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141E		October 12 – 23, 2020			Elbonia

TSG-SA2 Meeting #142E		November 16 – 20, 2020			Elbonia