SA WG2 Meeting #S2-140E	S2-2005441r02

Aug 19 – Sep 01, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0183 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on mapping of expected analytics type and Exception IDs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Table 6.7.5.1-1: Relation between expected analytics type and Exception IDs, "Abnormal traffic volume" should be removed, to align with the update of Table 6.7.5.3-3 (see S2-2003039). Also in Table 6.7.5.1-1, "Unexpected radio link failures" is currently classified as "communication related" abnormal behaviours. While considering: 1) it is the AMF (instead of the SMF) that takes actions for risk resolving when "Unexpected radio link failures" is detected, see Table 6.7.5.3-3. 2) the AMF and the SMF may request/subscribe "mobility related" and "communication related" abnormal behaviours respectively to the NWDAF, based on each functionality and for taking further action. it seems more appropriate to classify "Unexpected radio link failures" as "mobility related" abnormal behaviours. |  |  |  |  |  |  |  |  |
| FO |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove "Abnormal traffic volume" from Table 6.7.5.1-1. - Change "Unexpected radio link failures" to "mobility related" abnormal behaviours in Table 6.7.5.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid Exception ID ("Abnormal traffic volume") exists. The AMF fails to get the expected exception information related to mobility for taking further actions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.1 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

6.7.5.1	General

This clause defines how to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked, with the help of NWDAF.

NOTE 1:	The misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen.

The consumer of this analytics could be a 5GC NF. The 5GC NF subscribes analytics on abnormal behaviour from a NWDAF based on the UE subscription, network configuration or application layer request.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription and returns exception reports that result from the analysis of the correlations between behavioural variables. The exception reports contain an Exception Level expressed in the form of a scalar value, possibly supplemented by additional measurements.

The consumer of this analytics shall indicate in the request:

-	Analytics ID set to "Abnormal behaviour";

-	The Target of Analytics Reporting can be one UE, any UE or an Internal Group Identifier;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information optionally including:

-	expected UE behaviour parameters;

-	expected analytics type or list of Exception IDs with associated thresholds for the Exception Level, where the expected analytics type can be mobility related, communication related or both;

-	Area of interest;

-	Application ID;

-	DNN;

-	S-NSSAI.

NOTE 2:	The expected analytics type generally indicates whether mobility or communication related abnormal behaviour analytics or both are expected by the consumer, and the list of exception IDs indicates what specific analytics are expected by the consumer. Either the expected analytics type or the list of Exception IDs needs to be indicated, but they are not presented simultaneously. When the expected analytics type is indicated, the NWDAF performs corresponding abnormal behaviour analytics which are supported by the NWDAF. The relation between the expected analytics type and Exception IDs is defined in Table 6.7.5.1-1.

-	Optionally, maximum number of objects and maximum number of SUPIs;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

Table 6.7.5.1-1: Relation between expected analytics type and Exception IDs

| Expected analytics type | Exception IDs matching the expected analytics type |
| --- | --- |
| mobility related | Unexpected UE location, Ping-ponging across neighbouring cells, Unexpected radio link failures. |
| communication related | Unexpected long-live/large rate flows, Unexpected wakeup, Suspicion of DDoS attack, Wrong destination address, Too frequent Service Access. |

If the Target of Analytics Reporting is any UE, then the Analytics Filter should at least include:

-	Area of Interest or S-NSSAI, if the expected analytics type or the list of Exception IDs is mobility related.

-	Area of Interest, application ID, DNN or S-NSSAI, if the expected analytics type or the list of Exception IDs is communication related.

If the target of Analytics Reporting is any UE, the consumer of this analytics shall request either mobility related only or communication related only abnormal behaviour analytics, but not both at the same time.

The expected UE behaviour parameters that the consumer can indicate in the request when known depend on the Exception ID that the consumer expects. They may encompass UE behaviour parameters as defined in TS 23.502 [3] clause 4.15.6.3, and other parameters. Table 6.7.5.1-1 shows the mapping between each Exception ID and UE behaviour parameters.

Table 6.7.5.1-1: Description of Expected UE Behaviour parameters per Exception ID

| Exception ID | UE behaviour parameters to provide | NF for data collection |
| --- | --- | --- |
| Unexpected UE location | Expected UE Moving Trajectory Stationary Indication | AMF |
| Unexpected long-live/large rate flows | Periodic Time Scheduled Communication Time Communication Duration Time | SMF |
| Unexpected wakeup | Periodic Time Communication Duration Time Scheduled Communication Time | SMF |
| Suspicion of DDoS attack | Periodic Time Communication Duration Time Scheduled Communication Time Scheduled Communication Type Traffic Profile | SMF |
| Too frequent Service Access | Periodic Time | SMF |
| Unexpected radio link failures | Expected UE Moving Trajectory | AMF |
| Ping-ponging across neighbouring cells | Expected UE Moving Trajectory Stationary Indication | AMF |

When the NWDAF detects those UEs that deviate from the expected UE behaviour, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.7.5.3.

******************** End of Changes ********************