3GPP TSG-SA2 Meeting #140-e	S2-2005476r02

# Online, , 19st Aug 2020 – 2nd Sep 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | draftCR | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N4 Node Level procedures for satellite backhaul connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SA2 #139e approved contribution S2-2004705 with a comment below: Whether and when QoS limitation defined in Solution #11 needs to be considred for UPF selection may be discussed in the normative work N4 Node level procedure to inform QoS limitation per UPF peer (e.g., AN, UPF) is missing, so that SMF cannot collect all QoS limitation information of the satellite backhaul connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add N4 level procedures that UPF informs all QoS limitation information from UPF peers including the limitation occurred by the satellite backhaul connection during N4 Association Setup/Update procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS limitation awared UPF selection cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.xx (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First change (all text is new) **************

4.xx.x	N4 Node Level procedures

4.xx.x.1	N4 Association Setup Procedure

This procedure is based on the procedure described in clause 4.4.3.1 with the following differences and clarifications:

-	In Figure 4.4.3.1-1, when receiving an N4 Association Setup Request, the UPF shall provide QoS limitation Information (Satellite Category, any backhaul-related information of UPF peers) in the N4 Association Setup Response.

-	In Figure 4.4.3.1-2, the UPF provides QoS limitation Information (Satellite Category, any backhaul-related information of UPF peers) in the N4 Association Setup Request.

4.xx.x.2	N4 Association Update Procedure

This procedure is based on the procedure described in clause 4.4.3.2 with the following differences and clarifications:

-	In Figure 4.4.3.2-2, the UPF provides QoS limitation Information (a list of Satellite Category, any backhaul-related information of UPF peers) in the N4 Association Update Request.

*************** End of changes ***************