SA WG2 Meeting #140E	S2-2005577

19 August - 2 September, 2020, Electronic

Source:	Ericsson

Title:	KI#7 Sol#31 – resolving Editor´s notes

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This contribution resolves the Editor´s note related to solution #31 and corrects the solution following how existing standard works.

# 1	Discussion

The following ENs exists as part of solution #31.

Editor's note:	it is FFS whether it is acceptable to change the current design principle by which the Allowed NSSAI contains all the S-NSSAI the UE can use concurrently. It is also FFS to see if the scenario by which an operator does not support services on common frequency bands, that are supposed to be run concurrently.

In many cases there is a need to prioritize between services e.g. admission control for UP is not done until there is an attempt to activate the UP i.e. just because the PDU Session is established we cannot expect that UP will always be successfully activated when requested. In the same way we cannot assume that all S-NSSAIs in an Allowed NSSAI always can be used at the same time in all situations.

However, as a basic principle the S-NSSAIs in the Allowed NSSAI should allowed at the same time. That can be achieved by defining priorities between FBs for a network slice e.g. as to achieve the QoS is possible.

If there is a need to completely isolate network slices between each other, then KI#6 can be used and the S-NSSAIs to be isolated from each other cannot be in the same Allowed NSSAI.

This solution achieves the possibility to keep all S-NSSAIs in the Allowed NSSAI and dependent on the UP to be activated the FB to be used is selected by the NG-RAN e.g. using AMF provided RFSP reflecting the curent situation as input.

Editor's note:	it is FFS what happens to PDU Sessions already established over S-NSSAI_1 in that case, and whether it is acceptable for them to be affected.

The existing PDU Sessions are moved and existing admission control is executed.

Editor's note:	It is FFS how to proceed the PDU session establishment procedure if frequency cell change is failed.

If the network slices works best with certain FBs from a performance perspective then there is no issue as the UP can be established while possible with a reduced QoS.

Editor's note:	the exact extent of impact on UE functionality in terms of activation of multiple PDU sessions to different S-NSSAIs, reacting to having only one PDU session established, etc., is FFS.

No issue as all PDU Sessions can be maintained while e.g. QoS may be impacted.

Editor's note:	it is FFS whether the solution can work for pre-Rel-17 UEs, as these UEs would expect to be able to have the ability to have PDU Sessions established on all S-NSSAIs concurrently

No issue as from a UE perspective existing standard can be re-used.

## 2. 	Proposal

Update TR 23.700-40 as follows.

* * * * Start of Change * * * *

### 6.31	Solution #xx: RAN controlled steering of the UE to a network slice in a different frequency band

### 6.31.1	Introduction

This solution addresses the below requirements from Key Issue #7: Support of 5GC assisted cell selection to access network slice.

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

### 6.31.2	High Level Description

The solution is based on the following high level assumptions and principles:

-	The UE is allocated Allowed NSSAI which are supported by all cells in the Registration area.

### 6.31.3	Procedures

Figure 6.31.3.1-1 - Steering the UE to a network slice in different frequency band

1)	The UE is in idle mode, registered via RAN-1 for S-NSSAI-1, which operates only in frequency band 1 (FB-1) and S-NSSAI-2, which operates only in frequency band 2 (FB-2).

2)	An application in the UE needs to establish service on S-NSSAI-2. S-NSSAI-2 is defined in the network to prefer to use FB-2.

3)	The UE establishes RRC connection with RAN-1.

4)	The UE triggers PDU Session Establishment Request on S-NSSAI-2 via RAN-1.

5)	AMF is aware of the FB preferences for S-NSSAI-2 by the RFSP configuration for S-NSSAI-2.

NOTE 1:	Not seen in the flow, but the AMF gets the RFSP to be applied from the PCF (or the SMF gets it from PCF and AMF gets it from SMF) such that RFSP sent to NG-RAN always reflects the current situation e.g. based on Allowed NSSAI when there is no UP activated for an S-NSSAI and an RFSP reflecting the combination of UP activated for a set of S-NSSAIs.

6) AMF requests RAN-1 to apply RFSP while activating the UP for S-NSSAI-2 and in this case it implies to steer the UE to a RAN Node serving, or best serving, S-NSSAI-2.

The NG-RAN takes the decision whether to perform option A, option B, or option C.

Option A (e.g. used if the S-NSSAI-2 is better served by another FB and admission control only to be applied at target):

7a)	RAN-1 indicates to AMF that request failed due to mobility and triggers inter-frequency cell change to RAN-2 which serves S-NSSAI-2 and is in UE's location.

NOTE 2:	It is for RAN Working Groups to define the inter-frequency cell change in connected mode e.g. Handover, Cell Change Order, RRC release with re-direction or RRC Connection reconfiguration.

8a)	AMF may re-transmits the N2 message in step 6 and the PDU Session establishment procedure continues on S-NSSAI-2 via RAN-2.

Option B (e.g. used if the FB for S-NSSAI-2 is to be served by current FB):

7b)	PDU Session establishment procedure on S-NSSAI-2 via RAN-1 as per current specifications.

8b)	RAN-1 triggers inter-frequency cell change to RAN-2 which is preferred for S-NSSAI-2, as per RFSP, and is in UE's location.

Option C (e.g. used if UE should not be moved)

7c)	NG-RAN adopts the AS and UP configurations to serve the new UP for S-NSSAI-2 e.g. DC/CA

8c)	PDU Session establishment procedure on S-NSSAI-2 via RAN-1 as per current specifications.

9)	After PDU session on S-NSSAI-2 is deactivated or released, the AMF updates the RFSP towards the NG-RAN and the NG-RAN steers the UE accordingly.

### 6.31.4	Impacts on services, entities and interfaces

-	UE is not impacted as the UE will be steered by NG-RAN as per current mechanisms.

-	AMF (and PCF) logic to keep RFSP updated as per current situation e.g. which S-NSSAIs have UP activated etc..

-	RAN logic to steer UE based on RFSP input.

-	No impact on RAN-CN interfaces since current mechanisms are used.

* * * * End of Change * * * *